SA WG2 Meeting #150e	S2-2202748r01

April 6th – 12th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	New Solution: Support QoS Profile exposure by UPF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for UPF exposing QoS Profile information.

1 Discussion

The solution proposal presented here addresses the key issue proposal in S2-2201823.

This paper proposes to support the UPF exposes QoS Profile to NEF/local NEF/trusted AF directly to facilitate the local AF requesting information with low latency.

2 Proposal

It is proposed to update TR 23.700-62 v0.1.0 as follows.

****************************First change****************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: QoS Profile exposure by UPF |  | X |

****************************Second change (all text is new) ****************************

6.X	Solution #X: QoS Profile exposure by UPF

6.X.1	Description

The QoS profile of a QoS flow is determined by UPF eventually. Thus, the UPF should have the ability to handle some QoS flow by itself, e.g., providing the QoS profile configured locally in UPF. By exposing this information directly from UPF to NEF, the AF can know the QoS profile with flexibility and low latency. The UPF may support Nupf_EventExposure_Subscribe service operation.

# Table 6.X.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_EventExposure | Subscribe | Subscribe/Notify | NEF, AF |
|  | Unsubscribe | Subscribe/Notify | NEF, AF |

This service can expose UPF related information to other NFs. There are two operations for this service:.

-  Allow consumer NFs to subscribe and unsubscribe for an event on PDU Session(s).

Editor’s note: The use case that UPF determines the QoS profile configured locally as the actual policy to expose and the difference with the information from SMF is FFS.

6.X.2	Procedures

The UPF may be instructed to report information about a QoS flow directly to the NEF, i.e. by passing the SMF and the PCF. This reporting may target a third-party AF that wants to know the QoS profile used by UPF to guarantee the QoS flow. The UPF may support Nupf_EventExposure_Subscribe service operation, as defined in TS 23.502 [3] clause 5.2.26.2.X.

Figure 6.X.2-1: QoS Profile exposure by UPF

1.	The AF issues an Nnef_EventExposure_Subscribe (Application ID, GPSI) service operation to request QoS profile from the L-NEF.

2.	The L-NEF finds the appropriate UPF by using the UPF locating method targeting PDU sessions of a certain UE with information of IP address, as described in other solutions.

3.	The L-NEF sends the Nupf_EventExposure_Subscribe request to the UPF to request QoS profile information.

4.	The UPF responds the QoS profile to L-NEF over the Nupf_EventExposure_Notify service operation.

5.	The L-NEF responds the QoS profile to the AF over the Nnef_EventExposure_Subscribe service operation.

**End of change**