SA WG2 Meeting #S2-150E	S2-2202749r04

06 - 12 April, 2022, E-Meeting

Source: 	NEC

Title: 	Solution for KI#2 Support of providing VPLMN network slice information to a roaming UE

Document for: 	Agreement

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_ph3 / Rel-18

Abstract of the contribution: This paper proposes Support of providing VPLMN network slice information to a roaming UE.

# Discussion

This contribution provides solution to the key issue#2 “Support of providing VPLMN network slice information to a roaming UE”.

# Proposal

Add the following solution to TR 23.700-08.

* * * First Change * * * *

## 6.x	Solution #x: “Sending rejected NSSAI to the UDM to assist the UDM to steer the UE to the PLMN supporting rejected NSSAI”

### 6.x.1	Introduction

The solution addresses KI#2 Support of providing VPLMN network slice information to a roaming UE.

During the registration procedure in a VPLMN, if the AMF determines that a S-NSSAI is not allowed e.g., the S-NSSAI is not supported by the VPLMN or the S-NSSAI is not available e.g. the no. of UE reached for the S-NSSAI, the AMF sends the rejected NSSAI and requested NSSAI to the UDM. When the UDM receives the rejected NSSAI and the requested NSSAI, the UDM can determine which other VPLMN can support the rejected NSSAI. The UDM will prioritize the VPLMN supporting the rejected NSSAI and initiates SoR procedure with the VPLMN supporting the prioritized VPLMN as highest priority PLMN. 
when the UE receives the SoR, the UE may selects the VPLMN which supports the rejected NSSAI as this VPLMN is highest priority PLMN.

### 6.x.2	Procedures

Figure 1: Steering of roaming to a VPLMN supporting the network slice which is not available in the current PLMN.

The detailed steps of the procedure are defined below:

0. the UE is configured with network slice for a VPLMN either while first registering to it or beforehand.

1. The UE initiates the registration procedure (e.g. Initial Registration, mobility registration) to VPLMN 1 for the S-NSSAIs present in the requested NSSAI.

Editor’s Note: Whether the UE capability to handle new SoR needs to be sent is FFS.

2. The AMF determines that some S-NSSAI(s) are allowed and some S-NSSAI(s) are rejected (e.g. the S-NSSAI is not supported in the TA or the PLMN). The AMF sends Registration accept message containing the allowed NSSAI and rejected NSSAI.

3-4). Upon reception of the Registration accept message, the UE sends Registration complete message.

5). The AMF sends Nudm_UECM_Update containing allowed NSSAI and rejected NSSAI.

NOTE: In order to transfer the allowed NSSAI and the rejected NSSAI securely from the UE to the UDM, steps 5 may be replaced with a dedicated secure signaling between the UE and the UDM.

Editor’s Notes: How does the V-AMF knows the UDM supports SoR assistance parameters (e.g. Rejected NSSAI or Allowed NSSAI) is FFS.

6) The UDM sends Nudm_UECM_Update response message.

7) UDM determines to trigger SoR procedure based on the Rejected NSSAI and allowed NSSAI.

8a) The UDM sends Nsoraf_SoR_Get request (VPLMN ID, allowed NSSAI, Rejected NSSAI) to the SoR-AF.

Editor’s note: “It is FFS how the UDM determines based on the Rejected NSSAI (and the respective reject cause) whether to send SoR info to the UE to steer to another VPLMN”.

8b) The SoR-AF calculate preferred PLMN list depending on the allowed NSSAI and rejected NSSAI and sends it in the Nsoraf_SoR_Get response message (Preferred PLMN list). The preferred PLMN list consists of PLMN IDs and the S-NSSAIs supported by the PLMN in the preferred PLMN list.

NOTE 1: The UDM choose not trigger the SoR procedure always when the UDM receives rejected or Allowed NSSAI. When the UDM triggers SoR is totally based on operator policy.

9)  The UDM send SoR as defined in 3GPP TS 23.122. The content of SoR can be

10) After the completion of the SoR procedure, the UE may selects the a VPLMN (e.g. VPLMN 2) according to the PLMN list received in step 9.

### 6.X.3	Impacts on services, entities and interfaces

AMF: Sending of allowed NSSAI and rejected NSSAI to the UDM.

UDM: determines to trigger SoR based on rejected NSSAI and allowed NSSAI.

SoR-AF: SoR AF preparing the preffered PLMN list based on the rejected NSSAI and Allowed NSSAI.