3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202757r01

Elbonia, April 6th – 12th, 2022

Source:	Samsung

Title:	A New Solution for KI#1: Support of Network Slice Service Continuity

Document for:	Approval

Agenda Item:		9.14

Work Item / Release: FS_eNS_Ph3 / Rel-18

Abstract: This paper proposes a solution for KI#1: support of network slice service continuity.

1. Introduction

This paper addresses KI#1 of the FS_eNS_Ph3 study item regarding support of network slice service continuity. Specifically, for a network slice that becomes unavailable (e.g., cannot serve the PDU session), a new network slice that can be used as an alternative for a network slice that becomes unavailable (e.g., cannot serve the PDU session) needs to be determined.

2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-41.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[3]	3GPP TR 38.832: "Study on enhancement of Radio Access Network (RAN) slicing".

[4]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[x]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

# * * * * Next change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#1 |  |  |  |
| X | X |  |  |  |

6.X	Solution #X: Slice Re-mapping Capabilities for Network Slice Service Continuity

6.X.1	Introduction

The solution addresses the below requirements for Key Issue #1: Support of network slice service continuity.

-	How does 5GS determine a remapped S-NSSAI, which is an S-NSSAI of network slice that can be determined as an alternative for a network slice that becomes unavailable?

6.X.2	Functional Description

The solution provides slice re-mapping that can determine a re-mapped slice for a network slice that requires slice re-mapping due to an overload, planned maintenance, etc.

6.X.3	Procedures

6.X.3.0	General

During a registration procedure, AMF selects PCF that supports slice re-mapping and obtain an information for slice re-mapping. A call flow for slice re-mapping policy at registration procedure is shown in Figure 6.X.3.1-1.

When a network slice that requires slice re-mapping occurs, due to the policy received from PCF during the registration, AMF interacts with PCF to obtain a remapped S-NSSAI for each PDU session that is associated with a network slice that requires slice re-mapping. Once a PCF selects and provides a re-mapped slice for the network slice that requires slice re-mapping, for each old PDU session associated with the network slice that requires slice re-mapping, AMF may interact with SMF to change the S-NSSAI of the PDU session. A call flow for changing S-NSSAI of PDU session is shown in Figure 6.X.3.2-1.

6.X.3.1	Registration

Figure 6.X.3.1-1: Slice re-mapping policy at registration procedure

- A UE triggers registration with the network.

- The AMF may interact with AUSF and UDM according to clause 4.2.2.2.2 in TS 23.502 [x].

- If the AMF decides to perform PCF discovery and selection and if the AMF supports slice re-mapping, the AMF may select PCF that supports slice re-mapping by utilizing NRF or local configuration.

- The AMF may request PCF to obtain AM policy.

- The PCF provides a policy including a PCR trigger that instructs the AMF to interact with PCF when a network slice that requires slice re-mapping occurs to AMF.

- A rest of the UE registration procedure according to clause 4.2.2.2.2 in TS 23.502 [x].

6.X.3.2	Change of S-NSSAI of a PDU Session

Figure 6.X.3.2-1: Change of S-NSSAI of a PDU Session

- The AMF determines that a network slice that requires slice re-mapping occurs.

- From AMF to PCF: AM Policy Association ID, S-NSSAI that requires slice re-mapping, Allowed NSSAI.

If the AMF has received a PCR trigger that instructs the AMF to interact with PCF when a network slice that requires slice re-mapping occurs and the AMF determined in step 1 that a network slice that requires slice re-mapping occurs, the AMF triggers AM Policy Association modification procedure to PCF.

- From PCF to AMF: AM Policy Association ID, selected S-NSSAI.

The PCF selects re-mapped S-NSSAI and provides it to the AMF based on the S-NSSAI that requires slice re-mapping and the Allowed NSSAI received from AMF in step 2.

The PCF may update URSP rules and triggers UE Configuration Update procedure based on the S-NSSAI that requires slice re-mapping and the selected re-mapped S-NSSAI.

- For each old PDU session associated with the network slice that requires slice re-mapping, AMF may trigger a change of the slice of the old PDU session to the selected re-mapped S-NSSAI received in step 3 by sending Update SM Context request message including the selected re-mapped S-NSSAI to the SMF. When the SMF receives the message that includes re-mapped S-NSSAI from the AMF, the SMF may determine which SSC mode to use based on its local policy and triggers the determined SSC mode operation using the selected re-mapped S-NSSAI as a S-NSSAI for the new PDU session.

6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

AMF:

-	Support slice re-mapping

-  send a message to SMF to trigger a change of S-NSSAI

SMF:

-	trigger SSC mode operation with a S-NSSAI received from AMF as a new PDU session

NRF:

-   Support discovery of PCF that support slice re-mapping

PCF:

-	Support slice re-mapping

# * * * * End of changes * * * *