3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202757r03

Elbonia, April 6th – 12th, 2022

Source:	Samsung

Title:	A New Solution for KI#1: Support of Network Slice Service Continuity

Document for:	Approval

Agenda Item:		9.14

Work Item / Release: FS_eNS_Ph3 / Rel-18

Abstract: This paper proposes a solution for KI#1: support of network slice service continuity.

1. Introduction

This paper addresses KI#1 of the FS_eNS_Ph3 study item regarding support of network slice service continuity. Specifically, for a network slice that becomes unavailable (e.g., cannot serve the PDU session), a new network slice that can be used as an alternative for a network slice that becomes unavailable (e.g., cannot serve the PDU session) needs to be determined.

2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-41.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[3]	3GPP TR 38.832: "Study on enhancement of Radio Access Network (RAN) slicing".

[4]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[x]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

# * * * * Next change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | KI#1 |  |  |  |
| X | X |  |  |  |

6.X	Solution #X: Slice Re-mapping Capabilities for Network Slice Service Continuity

6.X.1	Introduction

The solution addresses the Key Issue #1: Support of network slice service continuity.

6.X.2	Functional Description

The solution provides a mechanism to determine a re-mapped slice for a network slice that requires slice re-mapping due to an overload, planned maintenance, etc. and to move ongoing PDU sessions to the re-mapped slice.

Editor's note:	The solution considers the scenario when the network slice becomes unavailable (e.g., overload, planned maintenance, etc) but more detailed scenarios where the solution can be applied are FFS

6.X.3	Procedures

6.X.3.0	General

During a registration procedure, AMF selects PCF that supports slice re-mapping and obtain an information for slice re-mapping. A call flow for slice re-mapping policy at registration procedure is shown in Figure 6.X.3.1-1.

When a network slice that requires slice re-mapping occurs, due to the policy received from PCF during the registration, AMF interacts with PCF to obtain a remapped S-NSSAI for each PDU session that is associated with a network slice that requires slice re-mapping. Once a PCF selects and provides a re-mapped slice for the network slice that requires slice re-mapping, for each old PDU session associated with the network slice that requires slice re-mapping, AMF may interact with SMF to change the S-NSSAI of the PDU session. A call flow for changing S-NSSAI of PDU session is shown in Figure 6.X.3.2-1.

6.X.3.1	Registration

Figure 6.X.3.1-1: Slice re-mapping policy at registration procedure

- A UE triggers registration with the network.

- The AMF may interact with AUSF and UDM according to clause 4.2.2.2.2 in TS 23.502 [x].

- If the AMF decides to perform PCF discovery and selection and if the AMF supports slice re-mapping, the AMF may select PCF that supports slice re-mapping by utilizing NRF or local configuration.

Editor’s Note: whether there needs to be information in the NRF regarding support of slice re-mapping is FFS

- The AMF may request PCF to obtain AM policy.

- The PCF provides a policy including a PCR trigger that instructs the AMF to interact with PCF when a network slice that requires slice re-mapping occurs to AMF.

- A rest of the UE registration procedure according to clause 4.2.2.2.2 in TS 23.502 [x].

6.X.3.2	Change of S-NSSAI of a PDU Session

Figure 6.X.3.2-1: Change of S-NSSAI of a PDU Session

- The AMF determines that a network slice that requires slice re-mapping occurs.

- From AMF to PCF: AM Policy Association ID, S-NSSAI that requires slice re-mapping, Allowed NSSAI.

If the AMF has received a PCR trigger that instructs the AMF to interact with PCF when a network slice that requires slice re-mapping occurs and the AMF determined in step 1 that a network slice that requires slice re-mapping occurs, the AMF triggers AM Policy Association modification procedure to PCF.

- From PCF to AMF: AM Policy Association ID, selected S-NSSAI.

The PCF selects re-mapped S-NSSAI and provides it to the AMF based on the S-NSSAI that requires slice re-mapping and the Allowed NSSAI received from AMF in step 2.

The PCF may update URSP rules and triggers UE Configuration Update procedure based on the S-NSSAI that requires slice re-mapping and the selected re-mapped S-NSSAI.

Editor's note:	It is FFS whether and how the change of S-NSSAI of a PDU session applies to roaming scenario

- For each old PDU session associated with the network slice that requires slice re-mapping, AMF may trigger a change of the slice of the old PDU session to the selected re-mapped S-NSSAI received in step 3.

Editor's note:	Details on how PDU Session with the selected re-mapped S-NSSAI can be achieved are FFS

6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

AMF:

-	Support slice re-mapping

-  send a message to SMF to trigger a change of S-NSSAI

SMF:

-	trigger SSC mode operation with a S-NSSAI received from AMF as a new PDU session

NRF:

-   Support discovery of PCF that support slice re-mapping

PCF:

-	Support slice re-mapping

# * * * * End of changes * * * *