3GPP TSG-WG SA2 Me eting #150E e-meeting 	S2-2202759r03

Elbonia, April 6th – 12th, 2022

Source:	Samsung

Title:	A New Solution for KI#4: Support of Service Continuity in Multiple NSACF environment

Document for:	Approval

Agenda Item:		9.14

Work Item / Release: FS_eNS_Ph3 / Rel-18

Abstract: This paper proposes a solution for support of service continuity in multiple NSACF environment

1. Introduction/Discussion

This paper addresses KI#4 of the FS_eNS_Ph3 study item regarding support of NSAC involving multiple service area. Specifically, this paper proposes a method to provide session continuity when UE move across the service area of NSACF.

2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-41.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[3]	3GPP TR 38.832: "Study on enhancement of Radio Access Network (RAN) slicing".

[4]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[x]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

# * * * * Next change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  |  |  | KI#4 |
| X |  |  |  | X |

6.X	Solution #X: NSAC exemption on NSACF relocation due to UE mobility

6.X.1	Introduction

In a multiple NSACF deployment scenario, each NSACF may be configured with its’ quota (i.e., maximum number of UEs for a network slice and maximum number of PDU sessions for a network slice) and SA. When a new NSACF is selected due to UE mobility, network slices that are subject to NSAC and have been already accepted by a previously selected NSACF (i.e., old NSACF) can be rejected by the new NSACF resulting in interruptions on services. To avoid interruptions and provide service continuity, a method to provide a service continuity when a new NSACF is selected is proposed.

6.X.2	Functional Description

NSACF exemptions are conducted on NSACF relocation due to UE mobility (i.e., UE moves out of SA of previously selected NSACF).

6.X.3	Procedures

6.X.3.0	NSAC exemption on the number of UEs per network slice

The signalling flow for AMF to determine NSAC exemption on the number of UEs per network slice is based on the signalling flow in Figure 4.2.11.2-1 in TS 23.502 [4] with the following additions:

- In step 1, the Number of UEs per network slice availability check and update procedure can be triggered due to the change of the NSACF for the number of UEs per network slice due to UE mobility.

- In step 2, if the Number of UEs per network slice availability check and update procedure is triggered due to the change of the NSACF for the number of UEs per network slice due to UE mobility, the AMF sends Nnsacf_NSAC_NumOfUEsUpdate_Request message with the update flag indicating decrease to the old NSACF for the last Allowed NSSAI and the AMF may include an exemption indication in the update request message to the NSACF.

Editor’s Note: Further behavior of the new NSACF on receiving the exemption indication is FFS

- In step 4, if the message from AMF in step 2 includes the exemption indication and if the maximum number of UEs are reached for some network slices, the NSACF does not set the result indication as ‘S-NSSAIs reached the maximum number of UEs per S-NSSAI’ for the network slices in the response message.

6.X.3.1	NSAC exemption on the number of PDU sessions per network slice

The signalling flow for SMF to determine NSAC exemption on the number of PDU sessions per network slice is based on the signalling flow in Figure 4.2.11.4-1 in TS 23.502 [4] with the following additions:

- In step 1, the Number of PDU sessions per network slice availability check and update procedure is triggered due to the change of the NSACF for the number of PDU sessions per network slice due to UE mobility.

Editor’s Note: Further behavior of the new NSACF on receiving the exemption indication is FFS

- In step 2, if the Number of PDU sessions per network slice availability check and update procedure is triggered due to the change of the NSACF for the number of UEs per network slice due to UE mobility, the SMF sends Nnsacf_NSAC_NumOfPDUsUpdate_Request message with the update flag indicating decrease to the old NSACF and the SMF may include an exemption indicator in the update message to the new NSACF for service continuity.

- In step 4, if the message from SMF in step 2 includes the exemption indication and if the maximum number of PDU sessions per S-NSSAI’ are reached for some network slices, the NSACF does not set the result indication as ‘S-NSSAIs reached the maximum number of PDU sessions per S-NSSAI’ for the S-NSSAIs in the response message.

6.X.4	Impacts on services, entities and interfaces

AMF:

- NSAC exemptions for the number of UEs for network slices included in the last Allowed NSSAI when the NSACF changes due to UE mobility.

NSACF:

- interpretation of the exemption indication in the update request message

SMF:

- NSAC exemptions for the number of PDU sessions per network slice when the NSACF changes due to UE mobility.

# * * * * End of changes * * * *