SA WG2 Meeting #150e	S2-2202852

# April 6 – 12, 2022; Elbonia

Source: 	China Mobile

Title: 	Solution about policy enhancements to minimize the jitter

Document for: 	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: Propose solution for Key issue 7.

# 1 Discussion

In this solution, it is proposed two aspects to minimize the jitter. One aspect is the 5GS should have a clear idea about the jitter concept, i.e. how to calculate the jitter value. The second aspect is that once the PCF get the jitter value for air interface and CN part, what kind of policy enhancement can be implemented to reduce the jitter.

# 2 Proposal

It is proposed to update TR 23.700-60 v0.1.0 on FS_XRM as follows

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| #X: 5GS Policy enhancements to minimize the jitter |  |  |  |  |  |  | X |  |  |

# * * * * Second change * * * *(all new texts)

## 6.X	Solution #X: 5GS Policy enhancements to minimize the jitter

### 6.X.1	Key Issue mapping

This solution is for Key Issue #1, which addresses following aspects:

-	Potential policy enhancements to minimize the jitter, focusing on i.e. requirement provisioning from AF, extension of PCC rule.

### 6.X.2	Description

This solution proposes a method for the 5GS to have a knowledge of the end-to-end jitter and as well as the air interface transmission jitter and CN part transmission jitter, which can help PCF generate some policy to minimize the jitter in different part of 5GS.

In this solution, it is proposed two aspects to minimize the jitter. One aspect is the 5GS should have a clear idea about the jitter concept, i.e. how to calculate the jitter value. The second aspect is that once the PCF calculates the jitter value for RAN part and the part between NG-RAN and PSA UPF, what kind of policy enhancement can be implemented to reduce the jitter.

1) For the first aspect, with considering the QoS monitoring mechanism in Rel-16, 5G system can calculate the jitter based on packet transmission delay per QoS flow. For the 5GS jitter, the End-to-End delay from UE to PSA UPF should be used. While for the air interface jitter, the air interface delay from UE to RAN should be considered, and for the CN part jitter, the delay from RAN to N6 should be considered.

AF sends a request for jitter calculation by using Nnef_AFsessionWithQoS_Create request message to NEF, the indication of jitter requirement, jitter measurement period, sample frequency, reporting threshold or reporting frequency (TS 23.503 [12] clause 6.1.3.21) may be included in the request, and are sent to PCF via NEF.

When PCF accepts the jitter requirement, PCF generates the QoS Monitoring policy for a service data flow, based on the request from AF. In the QoS Monitoring PCC rule, the QoS monitoring period can be the same as jitter measurement period. Then PCF provides the QoS Monitoring policy to the SMF.

After the QoS monitoring procedure, the SMF can report the RAN part delay values, delay values between NG-RAN and PSA UPF, to PCF. PCF may use the IETF RFC 1889 method to calculate an estimation of jitter based on measured delay values and then sends the estimated jitter value to AF.

2) For the second aspect, AF may provide a jitter expectation to PCF, as well as the Alternative Service Requirements in a prioritized order. When the PCF authorizes the service information from the AF and generates a PCC rule, it shall also derive Alternative QoS Parameter Sets for this PCC rule based on the QoS Reference parameters or the Requested Alternative QoS Parameter Sets in the Alternative Service Requirements. So when the end-to-end jitter value cannot satisfy the AF requirement, the PCF may adjust the policy rule by using Alternative QoS rules to e.g. set a higher priority level for this flow.

When the PCF finds the air interface jitter value is larger than the end-to-end jitter value, PCF may notify such information to NG-RAN with requirement to minimize the air interface jitter. And the exact method is depending on RAN WG discussion.

### 6.X.3	Procedures for jitter calculation

The mechanism for AF requesting jitter requirements can be described in the Figure 6.X.3-1.

# Figure 6.X.3-1: AF requesting jitter calculation

0.	The PDU session has been established.

1.	The AF sends a QoS monitoring request to the NEF by Nnef_AFsessionWithQoS_Create request message. The request message includes indication of jitter, jitter measurement period, sample frequency.

2.	The NEF authorizes the AF request and sends the jitter calculation parameter in the AF request message to the PCF through the Npcf_PolicyAuthorization_Create request.

3.	PCF generates the authorized QoS Monitoring policy based on the AF request, measurement period, sample frequency, and triggers SMF to initiate PDU Session Modification procedure by sending Npcf_SMPolicyControl_UpdateNotify request.

4a.	SMF activates the end to end UL/DL packet delay measurement between UE and PSA UPF for the QoS Flow or GTP-tunnel during the PDU Session Modification procedure. SMF sends QoS Monitoring request to AMF by Namf_Communication_N1N2MessageTransfer message, and AMF sends QoS monitoring request to NG-RAN through N2 interface.

4b.	The SMF sends a QoS Monitoring request to the PSA UPF via N4

5.	AMF sends QoS monitoring request to NG-RAN through N2 interface.

6.	The NG-RAN initiates the RAN part measurement of UL/DL packet delay based on the QoS Monitoring request from SMF, and NG-RAN performs delay measurement based on the existing procedures in clause 5.33.3.2 of TS 23.501 [2] for QoS monitoring for URLLC.

7.	NG-RAN reports the RAN part UL/DL packet delay to the UPF in the UL packet data as defined in clause 5.33.3.2 of TS 23.501 [2].

8.	UPF calculates the packet delay between NG-RAN and PSA UPF, as described in clause 5.33.3.2 of TS 23.501 [2], and reports the packet delay value(s) between NG-RAN and PSA UPF, RAN part delay value(s) to SMF.

9.	The SMF reports the obtained packet delay value(s) to the PCF by sending an Npcf_SMPolicyControl_UpdateNotify response message.

10.	When PCF gets the packet delay value(s), including the packet delay value(s) between NG-RAN and PSA UPF and air interface delay value(s), it calculate delay jitter(s) based on these delay value(s), jitter measurement period, sample frequency. Then PCF replies the delay jitter value(s) to NEF.

11.	NEF sends jitter and other QoS monitoring data to AF through Nnef_AFsessionWithQoS_Notify.

### 6.X.4	Impacts on services, entities and interfaces

PCF:

-	Receiving the AF jitter requirement indication.

-	Calculating the estimated jitter value based on QoS monitoring E2E latency value(s) air interface delay value(s).

-  Notify the RAN about the air interface jitter value

# * * * *end of change * * * *