SA WG2 Meeting #150E	S2-2202864

6 - 12 April 2022, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	solution proposal for key issue 1 (inactive reception)

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: this paper proposes a solution for key issue 1 (inactive reception).

1		Discussion

Key issue 1 is defined as follows:

5.1	Key Issue #1: Multicast MBS data reception in RRC Inactive state

5.1.1	Description

In order to provide MBS service to more UEs in a cell, NG RAN could enable UEs within an MBS multicast session to receive MBS session data while in CM-CONNECTED with RRC Inactive state.

The following aspect will be studied for multicast:

-	Whether, how and what MBS assistance information to provide from 5GC to RAN for an MBS session allowing UEs in CM-CONNECTED with RRC Inactive state to receive MBS content, including the aspect which 5GC NF(s) determine the MBS assistance information and how they do so;

-	Whether and how to enhance the current procedures (including mobility related procedures) for MBS session with member UEs in RRC Inactive state.

NOTE 1:	During the study of this key issue, coordination with RAN WGs is needed before final conclusion.

NOTE 2:	RAN WG will determine how the switching for the UEs belonging to MBS session from CM-CONNECTED state to CM-CONNECTED with RRC Inactive state (and vice versa) is performed by the RAN node.

Solution 1 already provides a baseline for a key issue 1 solution. Although this P-CR is suggesting a new solution, Nokia is also happy to convert this P-CR into an update of solution 1 if this is agreeable to the original source companies.

2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change ***

6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 MBS session reception in RRC Inactive | 2 MOCN network sharing |  |  |
| 1 | X |  |  |  |
| 2 |  | X |  |  |
| X | X |  |  |  |

*** 2nd Change (all new)***

6.X	Solution #X: Procedures for Transmission mode tor inactive data reception

6.X.1	Introduction

This solution addresses Key Issue #1.

6.X.2	Functional description

This solution builds on top of solution 1.

It is desirable to transmit 5MBS multicast data only a single time in a cell (rather than one time in transmission mode for RRC-inactive reception and one time in transmission mode for RRC connected reception. However, RRC connected UEs may be able to receive MBS data transmitted using the transmission mode for RRC-inactive reception.  It is thus assumed that the 5MBS transmission mode for inactive reception will not be applied homogenously in all cells for a multicast session and that RAN nodes will select the transmission mode based on criteria outlined in solution 1.

Editor's note:	Those assumptions need to confirmed by RAN WGs. It also need consider in one cell there maybe some R17 UE, which can only receives the MBS data in the RRC connected state.

UEs need to be informed about and configured for the transmission mode applied in the cell where they are residing by the RAN nodes.

Editor's note:	Those assumptions need to confirmed and related procedures need to be defined by RAN WGs.

UEs that transition to RRC inactive mode can move freely in their RAN notification area without notifying the RAN nodes. Their location is thus not known at cell level. At the moment only the RAN node serving a UE is aware that it is participating in an MBS multicast session. Additional RAN nodes not handling UEs within the multicast session themselves but within RAN notification areas of UEs in the multicast session may need to be made aware of the multicast session and perform procedures for the multicast session such as transmitting data of the multicast session and also manage the related the shared delivery path.

Editor's note:	Related procedures need to be defined by RAN WGs. It also neeeds to be studied by RAN WGs whether it is possible to determine the presence of inavtive UEs that desire to receive a multicast session in a cell.

RRC_inactive UEs can move to cells where the transmission mode for RRC connected state is applied and then need to transition to the RRC connected state to receive MBS data.

The current MBS session activation procedures use paging for the MBS session and trigger RRC-inactive UEs to transition to RRC-connected state. For the transmission mode for RRC_inactive reception, RRC-inactive UEs should be made aware that the MBS multicast session is activated but remain in RRC-inactive state.

Editor's note:	Related procedures need to be defined by RAN WGs.

It was discussed in Rel-17 that it would be desirable for location dependent services to enable that the MBS session is active in some areas and inactive in other areas. However, UEs moving in RRC-Inactive state between such areas will not know whether the MBS session is active in their location.

Broadcasting of information about the MBS session in cells comprising information about whether the MBS session is active and about the transmission mode of the MBS session is suggested to solve those issues.

Editor's note:	Related procedures need to be agreed by RAN WGs.

UEs in RRC-Inactive state that move from cells with transmission mode for RRC inactice reception to cells with transmission mode for RRC-connected reception request to transition to RRC-connected state.

6.X.3	Procedures

6.X.3.1	MBS session creation, multicast session join and session establishment procedure

Figure 6.X.3.2-1: Enhancement to current MBS procedures for session creation and join

0-15	 same as in solution 1

16. The NR-RAN broadcasts the transmission mode for the MBS session in the cell and also indicates whether the MBS session is active

6.X.3.2	Moving a UE to RRC Inactive state

Figure 6.x.3.2-1: NG-RAN node moves a UE in 5MBS session to RRC Inactive state

- UE joined an MBS session and receives MBS data while in RRC connected state

- IF NG-RAN 1 uses the transmission mode for RRC-inactive reception in a cell where the UE is located, it can move one or several UE to RRC inactive state. NG-RAN 1 then releases the RRC connection to the UE. The UEs can move to other cells in their RAN notification areas with notifying NG-RAN 1.

- The NG-RAN decides the transmission mode to apply for the MBS multicast session for cells within the RAN paging areas that are also within the service area of the MBS session.

- The NR-RAN broadcasts the transmission mode for the MBS session in thosecell and also indicates whether the MBS session is active

- The NG-RAN may transmit MBS data.

5.	NG-RAN 1 determines that part of the the RAN notification areas of the UEs is served by RAN 2. It informs RAN 2 that handling of the MBS session for inactive UEs is required in that area,

Editor's note:	Related procedures need be determined by RAN WGs.

Editor's note: It is FFS how to release the shared tunnel per RNA MBS session and its related impact.

5.	The NG-RAN 2 decides the transmission mode to apply for the MBS multicast session in a cell within the area.

6.	The NR-RAN 2 broadcasts the transmission mode for the MBS session in the cell and also indicates whether the MBS session is active

7-8.	The NG-RAN 2 may establish shared delivery and transmit MBS data. It may obtain information about the MBS session via shared delivery.

Editor's note:	The procedures in this clause need be determined by RAN WGs. However, transfer of assistance information and shared delivery to additional RAN nodes impacts SA2

6.X.3.3	MBS service activation

Figure 6.x.3.2-1: MBS service activation

- RAN 1 receives paging request for MBS session.

- RAN 1 determines inactive UEs it serves and based on that paging area

- RAN 1 decides transmission mode for MBS session in cells

- For cells with transmission mode for inactive reception, RAN uses procedure that keeps UE in RRC-inactive state, for instance broadcast information that the MBS session is active

5-6.	The NG-RAN 2 may establish shared delivery and transmit MBS data. It may obtain information about the MBS session via shared delivery.

Editor's note: It is FFS how to release the shared tunnel per RNA MBS session and its related impact.

- For cells with transmission mode for connected reception, RAN uses paging procedure to make UEs transit to RRC-connected state

8.	NG-RAN 1 determines that part of the RAN notification area is served by RAN 2. It informs RAN 2 that paging of the MBS session for inactive UEs is required in that area,

9 to 13.	Same as 3 to 7, but executed by RAN 2

Editor's note:	The procedures in this clause need be determined by RAN WGs. However, transfer of assistance information and shared delivery impacts SA2

6.1.4	Impacts on services, entities and interfaces.

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] are reused.

In addition to impacts for solution 1:

NG-RAN:

-	Decide transmission mode for a multicast session in a cell

-	Broadcast information about transmission mode and MBS session status (active/inactive).

-	Handle MBS session in RAN paging areas of RRC-Inactive UEs within MBS session.

-	Inform peer RAN nodes that handling of MBS session is required in an area.

UE:

-	Receive information about transmission mode and MBS session status (active/inactive).

-	Receive MBS data in indicated transmission mode while MBS data is active.

-	Request to transition to RRC-connected state when moving in RRC-Inactive state from cell with transmission mode for RRC inactice reception to cell with transmission mode for RRC-connected reception.