3GPP TSG-SA WG2 Meeting #149E	S2-2202877r02

# E-Meeting, 6 – 12 Apr. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0719 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNPN: Correction to URSP provisioning when UE accesses an SNPN using CH credentials |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - CR0707 introduced some evaluation order for URSP rules in the UE when this UE accesses an SNPN using CH credentials. This order however however clashes with existing text on URSP rules handling. The clash originates from matching criteria introduced in CR0707; if no URSP rules provided by a PCF match, then the UE uses pre-configured rules, while existing text states that only URSP rules provided by the PCF are used in case the UE holds both provisioned and pre-configured rules are in the UE. - For a UE accessing an SNPN using CH credentials, CR0707 introduced a condition “If none of the URSP rules received from this SNPN match”. While correct, this condition is not sufficient as there may be no URSP rules received from this SNPN or the UE may have been required to not accept any URSP rules from this SNPN. - The UE can be required to not accept URSP rules from an SNPN accessed using CH credentials – which can be interpreted as being specific (a particular SNPN) or generic (any SNPN). The generic interpretation is most suitable and also simpler. - Match all URSP rules are not taken into account |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The existing text is amended to restrict to an a UE not accessing an SNPN using credentials. - The additional conditions are included. - "an" replaced with "any" - Match all URSP rules are taken into account |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Contradicting requirements for UEs access an SNPN using CH credentials, leading to unpredictable UE behavior. - Incomplete requirements - Unclear requirement - Incomplete requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

#### 6.6.2.2	Configuration and Provision of URSP

The UE may be provisioned (signalled) with URSP rules by PCF of the HPLMN. When the UE is roaming, the PCF in the HPLMN may update the URSP rule in the UE. For URSP rules, the UE shall support the provisioning (signalling) from the PCF in the HPLMN, as specified in TS 24.501 [22]. In addition, the UE may be also pre-configured with URSP rules (e.g. by the operator).

An SNPN-enabled UE, while registered in an SNPN, may be provisioned (signalled) with URSP rules by the PCF of the SNPN. For URSP rules, the UE shall support the provisioning (signalling) from the PCF in the SNPN as specified in TS 24.501 [22]. In addition, the UE may be also pre-configured with URSP rules for the SNPN (e.g. by the operator of the SNPN).

When an SNPN-enabled UE accesses an SNPN using credentials from a Credentials Holder (CH), the UE may also be provisioned (signalled) with URSP rules for the SNPN by the PCF of the SNPN. However, the UE may be required to not accept URSP rules signalled by any SNPN that the UE accesses using CH credentials as specified in TS 24.501 [22], as follows:

-	by (pre-)configuration by the PLMN or SNPN of which the CH is part of (when applicable); or

-	by provisioning (signalling) by the PLMN or SNPN of which the CH is part of, when the UE is registered in that PLMN or SNPN.

NOTE:	A network (PLMN or SNPN) when operating as a CH (see clause 5.30.2.9) does not provide PCF functionality i.e. the PCF of this network cannot provision (signal) URSP rules to the UE when the UE is accessing an SNPN using CH credentials from this network.

If a UE accesses an SNPN using credentials from a CH, the UE applies URSP rules following the procedure described in clause 6.6.2.3 with the following additions:

-	The UE first evaluates (in precedence order) the URSP rules, if any, provisioned (signalled) by the PCF of this SNPN, except the URSP rule with the "match all" Traffic descriptor.

-	If none of the URSP rules received from this SNPN match, then the UE evaluates (in precedence order) the URSP rules, if any, provisioned (signalled) by the PCF of the network (HPLMN or SNPN) holding the credentials when previously registered in the network holding the credentials, except the URSP rule with the "match all" Traffic descriptor.

-	If none of the URSP Rules matches, then the UE evaluates the UE Local Configuration, if any.

-	If none of the URSP Rules, except the URSP rule with the "match all" Traffic descriptor, matches, nor the UE Local Configuration matches, the UE. evaluates the URSP rule with the “match all” Traffic descriptor. The URSP rule with the “match all” Traffic descriptor provisioned (signaled) by the PCF of the SNPN has precedence over the one provided by the PCF of the network (HPLMN or SNPN) holding the credentials.

-	If no URSP rules are received from this SNPN or from the PCF of the network (HPLMN or SNPN) holding the credentials, then the UE evaluates (in precedence order) the URSP rules, if any, pre-configured in the UE.

If a UE accesses an SNPN using credentials from a CH, the UE applies URSP rules as follows:

-	The UE first evaluates (in precedence order) the URSP rules , except the URSP rule with the "match all" Traffic descriptor, if any, provisioned (signalled) by the PCF of this SNPN, following the procedure described in clause 6.6.2.3.

-	If none of the above URSP rules received from this SNPN match, or if there is no URSP rules except the URSP rule with the "match all" Traffic descriptor received from this SNPN (or if the UE is required to not accept any URSP rules from this SNPN), then the UE evaluates (in precedence order) the URSP rulesexcept the URSP rule with the "match all" Traffic descriptorif any, provisioned (signalled) by the PCF of the network (HPLMN or SNPN) holding the credentials when previously registered in the network holding the credentials, following the procedure described in clause 6.6.2.3.

-	If none of the above URSP rules received from this SNPN or from the PCF of the network (HPLMN or SNPN) holding the credentials match or if there is no URSP rules (except "match all" URSP rule) received from this SNPN (or if the UE is required to not accept any URSP rules from this SNPN) and there is no URSP rules (except "match all" URSP rule) received from the PCF of the network holding the credentials, then the UE evaluates (in precedence order) the URSP rules, if any, pre-configured in the UE.

-	If there is no matching URSP rules according to the above, the UE uses UE Local Configuration if any.

-	If none of the URSP Rules, except the URSP rule with the "match all" Traffic descriptor, matches, nor the UE Local Configuration matches, the UE. evaluates the URSP rule with the “match all” Traffic descriptor.  as follows:

--	The UE first uses the URSP rule with the “match all” Traffic descriptor., if any, provisioned (signalled) by the PCF of this SNPN, following the procedure described in clause 6.6.2.3.

-	If there is no URSP rule with the “match all” Traffic descriptor. from provisioned (signalled) by the PCF of this SNPN, then the UE uses the "match all" URSP rule, if any, provisioned (signalled) by the PCF of the network (HPLMN or SNPN) holding the credentials when previously registered in the network holding the credentials, following the procedure described in clause 6.6.2.3.

-	If no URSP rules are received from this SNPN or from the PCF of the network (HPLMN or SNPN) holding the credentials, then the UE evaluates (in precedence order) the URSP rules, if any, pre-configured in the UE.

Only the URSP rules provisioned (signalled) by the PCF are used by the UE, if both URSP rules provisioned (signalled) by the PCF and pre-configured URSP rules are present. If no URSP rule is provisioned (signalled) by the PCF, and the UE has pre-configured rules configured in both the USIM and ME, then only the pre-configured URSP rules configured in the USIM is used.

If the PCF receives application guidance for URSP determination that may apply to a given UE from UDR as specified in clause 4.15.6.7 of TS 23.502 [3], the PCF may verify the requested parameters (which are described in clause 4.15.6.10 of TS 23.502 [3]) with regards to the existing URSP rules and (re-)compose the URSP rules for the UE as described in clause 6.2.4 of TS 23.548 [33].

**** END OF CHANGE ****