SA WG2 Meeting #S2-145E	S2-2202890r01

06 – 12 April , 2022, E-meeting

Source:	Xiaomi

Title:	solution for KI#3: PIN management and PINE management

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN /Rel-18

Abstract of the contribution: This paper proposes to a solution for KI#3 on how to create a PIN and to add a PINE in PIN.

Discussion

This paper defines a new solution for KI#3 how an authorized UE to create a PIN.

5.3 Key Issue #3: Management of PIN and PIN Elements 5.3.1 Description This key issue intends to support the management of the PIN, including the management of different types of PIN Elements and the configuration of the PIN. Both the network operator and authorized 3rd party, i.e. PIN Element with Management Capability (PEMC) could create and configure the PIN and its elements. After a PIN has been created, PEMC can add a PEGC into the PIN, or remove a PEGC from the PIN, as well as add a PIN Element into the PIN and associate it to some PEGCs that have already been added into the PIN, or remove a PIN Element from the PIN. The Key Issue is to study the following aspects in the 5GS: - How to support mechanisms for network operator or authorized 3rd party (e.g., a PEMC) for PIN management, e.g., create/modify/delete/activate/deactivate a PIN, etc. - How to support for the management of PIN Elements, including to add/remove the PIN Elements, as well as the association between PEGC and other PIN Elements. - How to support establishing and enforcing the validity duration and the time validity of a PIN (e.g. the PIN is valid for 30 minutes, the PIN is valid from 15:00 UTC to 23:00 UTC) and of the PIN Elements in a PIN (e.g. the PINE will be member of PIN for 1 hour, the PIN element will be member of PIN from 16:00 UTC to 17;: UTC).

Proposal

It is proposed to add the following new solution in the TR 23.700-88.

*** Start of changes ***

6.X	Solution #X: PIN management and PINE management

6.X.1	Description

This solution is to address the KI#3 “Management of PIN and PIN Elements”, how an authorized UE to create a PIN. And how to add a device (PINE) into a PIN, when a PIN is created and PEMC is available.

In this solution both PINE Info and PIN Info are defined as following:

PINE Info:

- PINE ID,  unique ID within a PIN that could be allocated by PEMC locally, or network

- PINE type,  indicating whether it is PEMC, or PEGC, normal PINE

- PINE name, human readable information, e.g., printer in bedroom, or lamp in living room.

- Service, what the PINE can provide, e.g., printer, or lamp, or camera, etc.

- Valid time, how long for the PINE to be as member in the PIN

- Capability, indicating the communication capability, e.g., WiFi, BT, etc.

PIN Info:

- PIN ID,  unique ID within a PLMN and allocated by network,

- PIN name,  is readable information for user

- PIN Services: the list of the services that the PINE(s) can provide, e.g., printer, lamp, camera, etc.

- PIN valid time, to define how long the PIN can work, e.g., 30 minutes

- List of PINE Info, including all the information of PINEs in this PIN

Editor's note:	it is FFS how to authorize the UE to create PIN.

6.X.2	Procedures

An authorized UE to create a PIN

Figure 6.x.2-1: authorized UE to create a PIN

Here the authorized UE is allowed by the PIN owner to initiate to create a PIN. When an authorized UE triggers a request to network for creating a PIN, it may bring the candidates PIN Info, e.g., the PIN name, etc., optionally candidates PINE Info, which some of the PIN/PINE Info may be designed by user before creating the PIN, e.g., PINE name, capability, etc.

- UE sends the request to AMF via gNB for creating a PIN, which including candidate PIN name, candidate PIN Service(s), candidate PIN valid time, optional if available, a list of candidates PINE information containing (candidates PINE ID, candidate PINE name, candidate PINE type, candidate PINE service, candidate PINE capability, candidate PINE valid time).

When PINE ID is allocated by network, the candidates PINE ID is not included.

- AMF checks with UDM whether the UE is authorized to create PIN, or authorized to use PIN service, or whether UE subscribes to the PIN service. If no, the AMF rejects the UE with proper cause.

- AMF requests to UDM for creating the PIN, with information carried in step 1.

- UDM allocates the PIN ID based on UE request, and may allocates PINE ID for each candidate PINE if PINE info included in the request, and create PIN profile in the UE’s subscription data, including e.g.,

PIN Info

| PIN ID | PIN name | PIN Service | PIN valid time |
| --- | --- | --- | --- |
| xxxxx | John’s smart home | Lamp, printer, refrigerator, air conditioner | 1 year |
| List of PINE Info PINE ID; PINE type; Capability; Service; Name; Valid time. xxxx; Element/PEMC/PEGC; WiFi/BT; Printer; Office printer; permanent. |

5, UDM response to AMF with result of creating PIN, including the results, full PIN Info{PIN ID, PIN name, PIN Service, PIN valid time, optional list of PINE Info}

6, AMF sends the service accept to UE via gNB, including results, and full PIN Info.

A device joins a PIN

Figure 6.x.2-2: a device to join a PIN

In this solution, it assumed that PEMC broadcasts PIN network information, e.g., PIN ID, PIN name, etc. so that the device discovers and selects the target PEMC, and connection between device and PEMC is established via non-3GPP RATs, e.g., WiFi, BT, etc.

- A device sends the Join PIN Request to PEMC via the established connection between the device and PEMC, including PIN ID, Device ID, PINE type (optional), capability, Service, name, etc.

- PEMC determines to accept the request. How to determine to accept the request is up to implementation. PEMC allocates the PINE ID for the device. And PEMC determines the PINE type, e.g., act as PEMC, or PEGC, or PINE.

- PEMC sends the Join PIN response to the device, including result (accept, or reject), PIN Info and PINE Info as defined in 6.x.1.

PIN info: PIN ID, PIN name, PIN Service(s), PIN valid time, list of PINE info;

PINE info: PINE ID, PINE name, PINE type, capability, Service(s) and valid time.

- PEMC sends the PIN update request to AMF via gNB, including UE ID, PIN Info container [PIN ID, new PINE info {PINE ID, PINE type, PINE name, capability, service, valid time}]

- AMF sends the Nudm_UECM_Registration Request to UDM for updating the UE’s PIN Info, including message received in step 4

- UDM updates the received PIN info to the UE’s subscription data.

When a PIN is created, the PIN related information is established as well in the requested UE’s subscription data.

- UDM sends the response to AMF, including the results, and updated PIN Info.

- AMF sends the PIN update response to PEMC with result (accept, or reject), and updated PIN Info.

- For another option instead of step 3, PEMC sends the join PIN Response to device with result, and PIN/PINE info same as in step 3 after PEMC updating the PIN info to UDM.

6.X.3	Impacts on Existing Nodes and Functionality

Network impacts:

- UDM allocates PIN ID for each request

.

*** End of changes ***