3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202920

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key issue proposal on LPHAP UE identification

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: this is a new key issue proposal on LPHAP UE identification.

- Introduction

This is a new key issue proposal on LPHAP UE identification.

- Discussion

LPHAP（low power high accuracy positioning）is an important feature in Rel-18 SA1 discussion and corresponding requirement are documented in TS 22. 261 and TS 22.104.

SA1 considers low power high accuracy positioning is an integral part of a considerable number of industrial applications. The total energy needed for a specific operation time for such a low power high accuracy positioning optimized IoT-device is a combination of energy for positioning (varies depending on the used positioning method), energy for communication/synchronization and a difficult to predict factor to take additional losses through e.g. security, power management, microcontroller, and self-discharge of batteries into account.

Similar to Redcap UE, LPHAP UE may need specific handing in the system level, including the identification of the LPHAP UE, the particular subscription of the LPHAP UE, etc.

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change (all texts new)* * * *

5.X	Key Issue #X: support of low power and high accuracy positioning

5.X.1	Description

SA1 considers low power high accuracy positioning is an integral part of a considerable number of industrial applications. The total energy needed for a specific operation time for such a low power high accuracy positioning optimized IoT-device is a combination of energy for positioning (varies depending on the used positioning method), energy for communication/synchronization and a difficult to predict factor to take additional losses through e.g. security, power management, microcontroller, and self-discharge of batteries into account.

Similar to Redcap UE, LPHAP UE may need specific handing in the system level, therefore identification of the LPHAP UE is the pre-condition for network handling.

Reduction of power consumption can be useful for IIoT and CIoT UEs and is generally preferable for any UE to improve battery lifetime.

Following issues are proposed for study:

- How to identify an LPHAP UE by the 5G system;

- What information is for subscription data for LPHAP UE;

- How to handle LPHAP UE connection management/mobility management, for power saving purpose;

- Positioning procedure optimization for LPHAP UE, for power saving purpose

-	Reducing UE power consumption for an immediate location request (5GC-MT-LR, 5GC-MO-LR)

-	Reducing UE power consumption for a deferred location request (periodic or triggered 5GC-MT-LR)

- -	Reducing UE power consumption in association with high accuracy location

# * * * * End of changes * * * *