SA WG2 Meeting #150e	S2-2202939r01

# April 06 – 12, 2022; Elbonia               		            (revision of S2-220xxxx)

Source: 	Lenovo, Motorola Mobility

Title: 	Additions to Solution 12: Slice related TNGF selection for WLAN access

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution:

# Proposal

It is proposed to make the following changes to solution 12 in TR 23.700-53.

Start of Changes

## 6.12	Solution 12: slice related TNGF selection for WLAN access

### 6.12.1	Description

The solution addresses slice related TNGF selection for WLAN access. Other types of non-3GPP access are not covered by this solution.

Current (Rel-17) clause 6.3.12 of TS 23.501 [2] specifies that a non-3GPP access network may advertise "3GPP Cellular Network" information, by using the ANQP protocol, defined in the HS2.0 specification [6]. A non-3GPP access network (WLAN) may via ANQP "3GPP Cellular Network" information advertise (per Current (Rel-17) clause 6.3.12 of TS 23.501 [2]):

-	A PLMN List-3, which includes PLMNs with which "5G connectivity" is supported. A non-3GPP access network supports "5G connectivity" with a PLMN when it deploys a TNGF function that can connect with an AMF function and an UPF function in this PLMN via N2 and N3 interfaces, respectively.

The solution proposes to extend this capability in order for the UE to be able to select a SSID mapping at best to the NSSAI the UE wishes to register to over TNGF access. The solution assumes all TNGF that may be reached behind a SSID and TNAP support the same set of S-NSSAI(s).

The ANQP "3GPP Cellular Network" information when indicating "5G connectivity" may also advertise:

a)	the list of S-NSSAI(s) that the TNGF(s) reachable by the corresponding SSID on the TNAP support; or

b)	the list of Network Slice Access Stratum Groups (NSASGs) that the TNGF(s) reachable by the corresponding SSID on the TNAP support.

Editor's note: Whether the ANQP "3GPP Cellular Network" information contains a list of S-NSSAIs or a list of NSASGs will be decided at the TR conclusion phase.

Editor's note: It is FFS how the UE is configured with a mapping from S-NSSAIs to NSASGs that apply for TNGF selection (the NSASG configuration for NG-RAN may not fit to TNGF selection).

### 6.12.2	Procedures

The UE runs the TNGF selection process described in clause 6.3.12.2 of TS 23.501 [2] up to step 3.

Step 4 of clause 6.3.12.2 of TS 23.501 [2] is modified as follows to take into account the NSSAI the UE wishes to register to over TNGF access, as well as the ANQP "3GPP Cellular Network" information advertisement of the list of S-NSSAI(s) or the list of Network Slice Access Stratum Groups (NSASGs) that the TNGF(s) reachable by the corresponding SSID may support.

NOTE:	The list of S-NSSAI(s) that the TNAP advertises over ANQP for a SSID is a subset of the list of S-NSSAI(s) associated with the TAI that the TNGF has provided to the AMF in the N2 Setup Request message (TS 38.413 [7]).

Step 4:	The UE selects a non-3GPP access network to connect to, as follows (only step a is modified):

a.	The UE puts the available non-3GPP access networks in priority order. For WLAN access, the UE constructs a prioritized list of WLAN access networks by using the WLANSP rules (if provided) and the procedure specified in clause 6.6.1.3 of TS 23.503 [4], as well as by using the Requested NSSAI of the UE and the list of S-NSSAI(s) or the list of NSASGs advertised with the ANQP "3GPP Cellular Network" information.

Editor's note: The relative priority between the WLANSP rules and the slice-based selection is FFS.

b.	From the prioritized list of non-3GPP access networks, the UE selects the highest priority non-3GPP access network that supports the selected type of trusted connectivity to the selected PLMN.

d.	Over the selected non-3GPP access network, the UE starts the 5GC registration procedure specified in clause 4.12a.2.2 of TS 23.502 [3].

If the UE registers with a requested NSSAI equal to the Default Configured NSSAI (first registration in the serving PLMN), it may happen that the UE receives a Configured-NSSAI for the serving PLMN and an allowed NSSAI that contains more S-NSSAI(s) where there are more S-NSSAI(s) in the Configured-NSSAI for the serving PLMN than in the Allowed NSSAI. In that case the UE can retry TNGF selection in that PLMN using the Configured NSSAI for the Serving PLMN as Requested NSSAI.

### 6.12.3	Impacts on Existing Nodes and Functionality

Editor's note: This clause captures impacts on existing 3GPP nodes and functional elements.

-	TNAP is to advertise per SSID the list of S-NSSAI(s) or the list of Network Slice Access Stratum Groups (NSASGs) that the TNGF(s) reachable by the corresponding SSID support, via ANQP "3GPP Cellular Network" information that is also indicating "5G connectivity".

-	UE is to leverage this information to determine the priority order of available non-3GPP access networks per a modified clause 6.3.12 of TS 23.501 [2] process.

End of Changes