3GPP TSG-SA2 Meeting #150-e	S2-2202960r04

# Online, , 6th Apr 2022 - 12th Apr 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0116 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscelaneous corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Some mistakes from the large location dependent CR agreed at the last meeting should be corrected - For session creation with PCC wrong message flow for notification and missing event definition. - Inconsistencies between Namf_MBSBroadcast_ContextUpdate service operation parameters and Broadcast Update callflow. Also unclear how the AMF can determine which gNBs to contact because it cannot inspect changes in the N2 container. - MBS FSA ID should only ne input but no output of Nnef_MBSSession_Update service operation. - Some editor´s notes in PCC related service operations are already resolved and can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct mistakes related to large dependent services. - Complete Notification in the session creation with PCC flow. - Fix parmeter Inconsistencies between Namf_MBSBroadcast_ContextUpdate service operation parameters and Broadcast Update callflow. Add flag to tell AMF whether N2 container was changed. - Remove MBS FSA ID as output of Nnef_MBSSession_Update service operation. - Remove resolved editor´s notes in PCC related service operations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MSK upadtes not supported and varios smaller mistakes remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 7.1.1.3, 7.2.4.2.1, 7.2.4.2.6, 7.3.3, 9.1.3.1, 9.2.2.1, 9.2.3.1, 9.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.2.3	Location dependent MBS service

A location dependent MBS is identified by MBS Session ID, and provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different Area Session ID is used for each MBS service area. The Area Session ID is used, in combination with MBS Session ID, to uniquely identify the service area specific part of the content data of the MBS service within 5GS. The network supports the location-dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS Session ID (i.e. UEs are not required to be aware of the Area Session IDs). When UE moves to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE. For multicast MBS service, UEs outside all MBS service areas of the location dependent MBS session are not allowed to join the MBS service. When UE moves out of an MBS service area and there is no other MBS service area for the MBS session, the network ceases to deliver the content data to the UE. Depending on policy, for the multicast MBS service the network may remove UEs outside all MBS service areas of the location dependent MBS Session from the multicast MBS session context after a grace period The SMF may subscribe at the AMF to notifications about UE moving in or out of all MBS service areas of the location dependent MBS session.

For multicast communication towards an NG-RAN supporting MBS, the NG-RAN node handles mobility of UEs within the MBS session between MBS service areas served by the same NG-RAN without interaction with SMF.

For multicast communication, location dependent MBS services may be supported via 5GC Individual MBS traffic delivery towards RAN nodes not supporting MBS. If the SMF determines that the UE is in another MBS service area of the multicast session, the SMF configures the UPF to send multicast data relating to the new MBS service area towards the UE. The SMF additionally subscribes at the AMF to notifications about the "UE location" (e.g. for a small MBS service area) or "UE moving in or out of a subscribed "Area Of Interest"" event (with a single service area as area of interest) using the Namf_EventExposure service.

Information about different MBS service areas for a location dependent MBS service may be provided by one or several AFs or may be configured. Different ingress points for location dependent points for the MBS session are supported for different MBS service area dependent content of the MBS session; different MB-SMFs and/or MB-UPF may be assigned for different MBS service areas in an MBS session. When the different MB-SMFs are assigned for different MBS service areas in an MBS session, the same TMGI is allocated for this MBS session.

The Area Session ID is allocated by MB-SMF in MBS Session creation procedure. MB-SMF allocates Area Session ID for each MBS services area which is unique within the MBS session. MB-SMF needs to further ensure there is no MBS service area overlapping with other MBS service areas that share the same MBS session ID.

NOTE 1:	In this release, deployments topologies with specific SMF Service Areas are not supported, as a result, location dependent service using multicast communication is not supported when a UE moves outside its SMF service area.

NOTE 2:	For location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

NOTE 3:	An example of Location-dependent MBS is a nationwide weather forecast service with local weather reports.

NOTE 4:	Area Session ID is equivalent to Flow ID as specified in TS 23.246 [8].

2nd change

#### 7.1.1.3	MBS Session Creation with PCC

Figure 7.1.1.3-1: MBS Session Creation with PCC

Steps 1 to 7 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1. to 10:	Same as in Figure 7.1.1.2-1. Step 1 may include an application ID.

11. Same as step 11 in Figure 7.1.1.2-1. In addition, the NEF/MBSF decides based on local configuration or based on parameters received in step 8 (e.g. whether the session comprises several data flows) whether it will invoke the Npcf_MBSPolicy Authorization service for the MBS session. If so, the NEF/MBSF indicates to the MB-SMF that it will also provide a policy authorization for the MBS session to the PCF.

12. Same as step 12 in Figure 7.1.1.2-1.

13.	[Optional] If the NEF/MBSF indicated in step 11 that it will also provide a policy authorization for the broadcast session to the PCF, the MB-SMF selects a PCF and sends an Npcf_MBSPolicyControl_Create Request (MBS session ID) for the MBS session towards the PCF, and defers step 25 until receiving an Npcf_MBSPolicyControl_UpdateNotify for the MBS session. Otherwise, the MB-SMF decides based on local configuration whether to invoke the Npcf_MBSPolicyControl service.

14.	[Conditional on step 13] The PCF registers at the BSF that it handles the MBS session by using Nbsf_management_Register Request (MBS Session ID, PCF ID). It provides an identifier that the policy association is for MBS and the MBS Session ID, its own PCF ID and optionally its PCF set ID.

15.	[Optional] The PCF may retrieve preconfigured policy information for the MBS session based on the multicast address as multicast session ID (e.g. applicable QoS, the MBS Session-AMBR and/or default 5QI) from the UDR.

16.	[Conditional on step 13] The PCF responds with Npcf_MBSPolicyControl_Create Response (MBS Policy, see clause 6.10) with policies for the MBS Session ID. The MBS Policy may include the Session-AMBR for the MBS session and 5QI for the MBS QoS Flow.

NOTE 1:	If there is no service requirement, the PCF may create the MBS policy according the operator's policy.

There are two alternatives to initiate the policy Authorization service operation.

In the Alt-A, the MB-SMF is the Npcf_MBSPolicy Authorization_Create service operation consumer. The step 17 to step 19 are performed after step 16.

In the Alt-B, the NEF/MBSF is the Npcf_MBSPolicy Authorization_Create service operation consumer. The step 20 to step 25 are performed.

17.	[Optional, Alt-A] The MB-SMF sends an Npcf_MBSPolicy Authorization_Create Request to PCF with the MBS session ID and MBS session information (that may include an application ID).

Editor's note:	How multiple service requirements with multiple application IDs are handled is FFS.

18.	[Optional, Alt-A] The PCF may retrieve policy information for the application ID from the UDR.

19.	[Optional, Alt-A] The PCF sends an Npcf_MBSPolicy Authorization_Create Response to MB-SMF:

The PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the MB-SMF and determines whether this QoS is allowed (e.g. according to the policy input configuration in the UDR).

If the request is not authorized or the required QoS is not allowed, the PCF indicates so in the response to the MB-SMF.

20.	[Optional, Alt-B]	The MB-SMF sends a Nmbsmf_MBSSession_StatusNotify Request (MBS Session ID, PCC control established) to the NEF/MBSF

21-22.	[Optional, Alt-B] The NEF/MBSF uses the BSF Discovery service to discover the PCF serving the MBS session with the MBS session ID by using Nbsf_management_Discovery operation.

23.	[Optional, Alt-B] The NEF/MBSF sends an Npcf_MBSPolicy Authorization_Create Request to PCF with the MBS session ID and MBS session information (that may include an application ID):

Editor's note:	How multiple service requirements with multiple application IDs are handled is FFS.

24.	[Optional, Alt-B] The PCF may retrieve policy information for the application ID from the UDR.

25.	[Optional, Alt-B] The PCF sends an Npcf_MBSPolicy Authorization_Create Response to NEF/MBSF:

The PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (e.g. according to the policy input configuration in the UDR).

If the request is not authorized or the required QoS is not allowed, the PCF indicates so in the response to the NEF.

Editor's note:	How policy data is handled in UDR is FFS.

26.	When obtaining a request for the creation of a policy association (signal 21), for which it already performs policy control towards an MB-SMF, the PCF always provides a policy update to the MB-SMF; if no real policy update is required, the PCF repeats previous policies or sends an empty update message.

27-29	Same as steps 14-16 in Figure 7.1.1.2-1.

30.	When obtaining an MBS policy control update from the PCF (signal 16 for Alt-A or signal 29 for Alt-B) for a broadcast session, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1 to request the allocation of resources to for the transmission of the broadcast session.

31-35	Same as steps 18-22 in Figure 7.1.1.2-1.

NOTE:	Steps 31-35 can be executed in parallel to step 30.

3rd change

##### 7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The location dependent multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The location dependent multicast session is created as described in clause 7.2.4.2.2.

-	The UE may have information about the location dependent multicast service including all MBS service areas of the location dependent MBS session via Service Announcement as specified in clause 7.2.4.3.1.

-	If the UE determines that it is in MBS service area of the location dependent MBS session based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends Join Request to join the multicast group. If the UE determines that it is outside all MBS service area of the location dependent MBS session, the UE does not send the Join Request.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location). The MB-SMF should provide the TAI where the UE is located as input for the MB-SMF discovery. If the NRF does not have information about service areas for an MBS session ID in the MB-SMF profile but obtains a TAI in combination with MBS session ID as query parameter, the NRF ignores the TAI and provides the MB-SMF profile matching the MBS session ID.If the TAI is not included, but the related MB-SMF include the service area, the NRF return all the MB-SMF associated with that MBS session ID.

NOTE 1:	The AMF provides the User Location Information of the UE (cell ID and current tracking area ID) towards the SMF within Nsmf_PDUSession_CreateSMContext request and if the location changed also in Nsmf_PDUSession_UpdateSMContext request (as used for the join) as specified in clauses 4.3.2.2.1 and 4.3.3.2 of TS 23.502 [6].

-	The NRF provides information about the MB-SMF(s) serving the multicast session at the possible indicated location. The NRF provides the service areas and area session IDs for the multicast session as part of the MB-SMF profile(s), via Nnrf_NFDiscovery_Request Response (MB-SMF profile (Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing (if the TAI is not provided to the NRF) and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID, and subscribes to notifications about the MBS Session ID.

-	The SMF check whether the UE is inside MBS service areas of the location dependent MBS session by comparing the User Location Information of the UE provided by the AMF with the MBS service areas received from the NRF. If the UE is out of all MBS service area of the location dependent MBS session, the SMF reject the multicast session join request.

-	If the Join Request from the UE is accepted, the SMF subscribes at the AMF using the Namf_EventExposure service to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. The SMF supplies all MBS service areas of the location-dependent MBS session as Area Of Interest.

-	The SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID and MBS service area associated with the cell where UE camps.

-	If the NG-RAN node supports MBS, the NG-RAN uses the received MBS Session ID and Area Session ID to determine the local multicast session context and whether the user plane for the local multicast session is already established. If the target RAN determines the shared delivery is not established for the MBS session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4 and provides the Area Session ID, tracking area ID(s) concerning the RAN node in addition to MBS Session ID in the request for shared delivery establishment. The MB-SMF provides MBS service area information (Area session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN in the shared delivery establishment response. MB-SMF may either provide only the service areas concerning the target RAN node or all service areas associated with the MBS session. If the MB-SMF only provides the service areas concerning the target RAN node, which includes the cell ID(s), the cell ID is included if the tracking area IDs associated with the indicated cell ID is received by the MB-SMF.

-	If the NG-RAN node serving the UE does not support MBS and UE is in the MBS service area, the SMF apply individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via individual delivery within a PDU session of the UE. The SMF additionally subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes (e.g. for a small MBS service area), or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the latter case, the SMF supplies the service area associated with the multicast area session where the UE resides as Area Of Interest. The SMF configures separate ingress terminations at the UPF for each service area and interacts separately for each service area (providing MBS session ID and area session ID) with the MB-SMF to request or terminate the distribution of data for the MBS session and service area towards the UPF.

4th change

##### 7.2.4.2.6	UE location change handling by SMF for shared delivery

When the SMF receives a notification about a UE location change or is becoming aware of a UE location change (e.g. during handover), the SMF checks whether the new UE location is inside or outside all MBS service areas of the location dependent MBS session:

-	For the case that UE is out of all MBS service area of the location dependent MBS session:

-	If RAN node does not support MBS, the SMF delete the associated QoS flow from the RAN node and the UPF.

-	Per operator's policy (e.g. when a local configured timer expires since the UE left the whole MBS service area of the MBS session) the SMF may remove the UE from the multicast MBS session as defined in clause 7.2.2.3. The SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event.

-	For the case UE comes back to MBS service area of the location dependent MBS session after moving out of the MBS service area of the multicast MBS session before:

-	If the UE is still in the multicast MBS session and the multicast MBS session is in the "Active" state, the SMF tries to activate the multicast MBS session towards the UE.

5th change

### 7.3.3	MBS Session Update for Broadcast

The MBS Session Update for broadcast is used by the AF to update the broadcast area or service requirements of the MBS Session which may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

Figure 7.3.3-1: MBS Session Update for Broadcast

1.	The AF starts MBS session update procedure by sending Nnef_MBSSession_Update Request to the NEF/MBSF with TMGI. The AF may adjust service requirement and/or broadcast area. The service requirements adjustment may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

2.	The MB-SMF sends Namf_MBSBroadcast_ContextUpdate Request (TMGI, N2 SM information (MBS Session ID, [MBS QoS profile], [service area(s)], [Area Session ID(s)], [MBS IP Multicast Tunnel Info(s)], [MBS FSA ID(s)]  ), [updated MBS service area(s)], [N2 MBS info change indicator]) to the AMFs. For a location dependent service, the MB-SMF may provide information about several location areas. If the broadcast area is updated, the MB-SMF may use NRF to discover the AMF(s) based on the new broadcast area and select the appropriate one(s). The MB-SMF may include a maximum response time in the request.

Depending on the change of the MBS service area, the MB-SMF may send Namf_MBSBroadcast_ContextCreate to some AMFs in the new MBS service area, Namf_MBSBroadcast_ContextRelease to some other AMFs in the old MBS service area.

3.	The AMF sends MBS Session Resource Update to NG-RANs with TMGI, the updated 5G QoS Profile and the updated MBS service area.

Depending on the change of the MBS service area, the AMF may send MBS Session Resource Setup to some NG-RANs in new MBS service area (see clause 7.3.1) and MBS Session Resource Release to some other NG-RANs in old MBS service area.
If the N2 MBS info change indicator is not set, the AMF does not need to contact RAN nodes unaffected by the service area change,

4.	The NG-RAN updates MBS Session Context.

5.	The NG-RAN reports successful update of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Update Response (TMGI, N2 SM information ([N3mb DL Tunnel Info])) message(s) to the AMF. N3mb DL Tunnel Info is only available when point-to-point transport applies between MB-UPF and NG-RAN and the NG-RAN wants the transport to be changed. The NG-RAN should be ready to receive using the N3mb DL tunnel. For more details, refer to TS 38.413 [15].

6.	The AMF sends Namf_MBSBraodcast_ContextUpdate Response to the MB-SMF. If the AMF received the NG-RAN responses from all involved NG-RAN(s), the AMF should include an indication of completion of the operation in all NG-RANs.

7.	The NG-RAN updates the MBS Session. It takes place in parallel with step 5 to step 6.

8	Another NG-RAN may report successful update of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Update Response (TMGI, N2 SM information ([N3mb DL Tunnel Info])) message after the AMF transferred the Namf_MBSBroadcst_ContextUpdate Response () to the MB-SMF. N3mb DL Tunnel Info is only available when point-to-point transport applies between MB-UPF and NG-RAN and the NG-RAN wants the transport to be changed. The NG-RAN should be ready to receive using the N3mb DL tunnel. For more details, refer to TS 38.413 [15].

9.	The AMF transfers the Namf_MBSBroadcast_ContextStatusNotify request () to the MB-SMF. When the AMF receives the response from all NG-RAN nodes, the AMF should include an indication of completion of the operation in all NG-RANs. If the AMF does not receive responses from all NG-RAN nodes before the maximum response time elapses since the reception of the Namf_MBSBroadcast_ContextUpdate Request, then the AMF should transfer the Namf_MBSBroadcast_ContextStatusNotify request () which indicates partial success or failure.

6th change

### 9.1.3	Nmbsmf_MBSSession service

#### 9.1.3.1	General

Service description: The following are the key functionalities of this NF service:

-	(between AMF or SMF and MB-SMF) For multicast, NF Service Consumer can use this service to request the reception of MBS data or to terminate the reception of MBS data of a multicast session, or for a location-dependent MBS multicast session, the part of the MBS multicast session within a service area;

-	(between SMF and MB-SMF) For multicast, allow consumer NFs to query information (e.g. QoS information) about MBS Session. This service will be invoked by SMF for UE join event;

-	(between SMF (only for multicast) or MBSF/NEF and MB-SMF) Allow consumer NFs to subscribe and unsubscribe for an Event ID and MBS Session, or for a location-dependent MBS session, related to the part of the MBS session within a service area (only for MBSF/NEF);

-	(between SMF (only for multicast) or MBSF/NEF and MB-SMF) Notifying events on the MBS Session, or for a location-dependent MBS session, related to the part of the MBS session within a service area (only for MBSF/NEF), to the subscribed NFs;

-	(between MBSF/NEF/AF and MB-SMF) Creation/Modification/Activation/Deactivation/Release of multicast session, or for a location-dependent MBS multicast session, the part of the MBS multicast session within a service area; and

-	(between MBSF/NEF and MB-SMF) Creation/Modification/Start/Stop/Release of broadcast sessions, or for a location-dependent MBS broadcast session, the part of the MBS broadcast session within a service area.

The following events related to MBS session context can be subscribed by SMF as consumer NF:

-	(between SMF and MB-SMF, only for multicast) QoS change: The event notification is sent when QoS within a multicast session changes, e.g. adding/removing QoS flow(s);

-	(between SMF and MB-SMF only for multicast) multicast session status (activated, deactivated);

-	(between SMF and MB-SMF only for multicast) multicast session service area change (for a location-dependent MBS broadcast session, the notification relates to the part of the MBS broadcast session within a service area);

-	(between SMF and MB-SMF only for multicast) multicast session release.

The following events related to an MBS session, or for a location-dependent MBS session, related to the part of the MBS session within a service area, can be subscribed by MBSF, NEF, or AF as consumer NF:

-	(between MBSF/NEF/AF and MB-SMF) MBS session release due to TMGI expiry;

-	(between MBSF/NEF/AF and MB-SMF) Broadcast delivery status;

-	(between MBSF/NEF/AF and MB-SMF) PCC control established.

NOTE:	Whether event IDs are needed can be determined by stage 3.

7th change

#### 9.2.2.1	General

Service description: NF Service Consumer, e.g. MB-SMF can create and manage a MBS Policy Association in the PCF through which the NF Service Consumer receives policy information for a MBS Session.

As part of this service, the PCF may provide the NF Service Consumer, e.g. MB-SMF with policy information about the MBS Session that may contain:

-	MBS Session related policy information.

-	PCC rule information.

-	Policy Control Request Trigger information. When a Policy Control Request Trigger condition is met the NF Service Consumer, e.g. MB_SMF shall contact the PCF and provide information on the Policy Control Request Trigger condition that has been met.

At MBS Session establishment the NF Service Consumer, e.g. MB-SMF requests the creation of a corresponding MBS Policy Association with the PCF (Npcf_MBSSMPolicyControl_Create) and provides relevant parameters about the MBS Session to the PCF.

-	When the PCF has created the "MBS Policy Association", the PCF may provide policy information as defined above.

When a Policy Control Request Trigger condition is met the NF Service Consumer, e.g. MB-SMF requests the update (Npcf_MBSPolicyControl_Update) of the MBS Policy Association by providing information on the condition(s) that have been met. The PCF may provide updated policy information to the NF Service Consumer.

The PCF may at any time provide updated policy information (Npcf_MBSPolicyControl_UpdateNotify).

At MBS Session Release the NF Service Consumer, e.g. MB_SMF requests the deletion of the corresponding MBS Policy Association.

8th change

#### 9.2.3.1	General

Service description: This service is to authorise an AF / NEF / MBSF request for an MBS service and to create policies as requested by the authorized AF for the MBS Service. This service also allows the NF consumer to subscribe/unsubscribe the notification of events.

9th change

#### 9.3.2.3	Namf_MBSBroadcast_ContextUpdate service operation

Service operation name: Namf_MBSBroadcast_ContextUpdate

Description: This service operation is used to update the broadcast session context towards the AMF.

Inputs, Required: MBS Session ID.

Inputs, Optional: Broadcast service area, N2 MBS info change indicator, N2 container (MBS Session ID, Possible MBS QoS profile, Possible Broadcast service area, Possible Area Session ID, Possible MBS IP Multicast Tunnel Info, MBS FSA ID(s)).

Outputs, Required: Result Indication.

Outputs, Optional: N2 container (MBS Session ID, NG-RAN MBS Tunnel Info).

End of changes