3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202961r03

April 6th – 12th, 2022; Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	New Solution to Key Issue #1 of FS_GMEC WT#1.1

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: Proposes a new solution to Key Issue #1

# 1. Discussion

This paper introduces a new solution to address key issues on group management, provisioning of traffic characteristics and monitoring of performance characteristics.

# 2. Proposal

It is proposed to update TR 23.700-74 for FS_GMEC as follows

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| #1 | X | X |  |  |  |
| #2 |  |  | X |  |  |
| #3 |  |  |  | X |  |
| #4 |  |  |  | X |  |
| #5 |  |  |  | X |  |
| #6 |  |  |  |  | X |
| #7 |  |  |  |  | X |
| #X | X |  |  |  |  |

# * * * * Second change (All New) * * * *

## 6.X Solution #X: Support for group attribute management

### 6.X.1	Introduction

This solution addresses the KI#1

A group can be set with the service area that are applicable to each UE within the group.

### 6.X.2	Functional Description

When an AF creates a group, it provides an explicit list of group members (each identified by the UE ID or UE address)

AF creates and configures group, with group members, and subsequently group profiles are stored in the UDR. Consumer NFs can then retrieve such group details from UDM/UDR, as required.

### 6.X.3	Group Provisioning Procedure

Figure 6.X.3-1 provides the details of the procedures:

Figure 6.X.3-1: AF Creating and Provisioning Group ID profiles to 5GC

Step 1: AF assigns an (external) Group ID and includes a list of UEs to the group. AF then sends Nnef_ParameterProvision_Create/Update/Delete request

Editor's note:	It is FFS whether there is a need to group type, and what the values can be set.

Step 2: NEF after authorizing the AF request, translates the external information to 5GC internal information e.g. external group ID into an internal group ID. The NEF creates or updates this information in UDM by sending Nudm_ParameterProvision_Create/Update/Delete request (Group ID, Service area definition or Area of Interest or TAI(s))

Step 3-4: UDM stores the Information in UDR using Nudr_DM_Query/Update messages.

Step 5-6: AF receives the Response message to its request in step 1.

Step 7: Consumer Network Function e.g. AMF and SMF may subscribe and get notified on these information. In case of NF enforced functionality (e.g. Service area related restriction) the subscribing to the notification is mandatory. If the group information is to be provided to AMF and/or to SMFfollowing mechanism is envisaged:

- NFs (e.g. AMF, SMF) subscribe to UDM for the subscription data changes (creation / modification / deletion) of the group member UEs. When such groups are subject to a creation / modification / deletion, the UDM notifies the subscribed NF for the respective group member UEs. UDM may also indicate shared data identifier, as mentioned in TS 23.501 [2] clause 4.15.6.2 and explained in TS 29.503 [X] clause 5.2.2.

In case PCF has subscribed to receive notification on the shared data, UDR notifies using Nudr_DM_Notify procedure.

Editor's note:	It is FFS whether the same purpose can be achieved via spatial validity in AF request, e.g. clause 5.6.7 in TS 23.501 [2].

### 6.X.4	Impacts on existing entities and interfaces

The identified enhancements concern Information elements contained in AF:

- Enhancements to externally indicated service area definition into TAI or Area of Interest or to existing IE Potential Locations of Applications and IE Spatial Validity Condition)

NFs (AMF, SMF, PCF) provisioning to the group membership definition and NF subscription to the group event notifications and taking the required actions.

PCF performing mapping between the UDR stored QoS attributes and 5GS QoS parameters, manages the potential conflicts of the QoS definitions, and triggers PCF initiated SM Policy Association Modification/ Termination for each UE belonging to the group.

# * * * * End of changes * * * *