SA WG2 Meeting #150e	S2-2202981r01

6 – 12 April 2022, e-Meeting	(revision of S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	Key Issue: Coverage using Alternative RATs

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract of the contribution: Proposes a new KI for support coverage using alternative RATs

# 1. Introduction

The solution in Release 17 assumes that satellite coverage information is provided to a UE and CN regarding impending satellite coverage gaps. The UE can then deactivate the AS layer until satellite coverage will resume and the CN can avoid detaching the UE or attempting to page the UE during a coverage gap and can wait for a new UE access (e.g. a TAU) before sending any outstanding MT data to the UE.

However, this neglects the possibility of the UE obtaining network access using another satellite RAT, a TN RAT or by simply moving to a new location where satellite access is available. From the CN perspective, this may not be a problem, as long as the UE detects the new coverage and accesses the CN when such coverage is found. However, nothing in the Release 17 solution enables the UE to become aware of such new coverage and thus the UE may needlessly remain unactive for a significantly longer time than needed.

A KI to address these limitations seems needed.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-28.

*** Start of the change ***

### 5.2.1	General description

For a UE using a NG-RAN that provides discontinuous coverage (e.g. for satellite access with discontinuous coverage), the UE may be out of network coverage at a certain time. The UE may then attempt to scan for available cell due to the UL traffic or NAS layer signalling, e.g. Periodic Registration. In Rel-17, the UE in EPS may deactivate the access stratum when there is no network coverage, however there might still be some additional issues, e.g. whether de-registration occurs due to any inconsistency of CM states between UE and CN and, the usage of eDRX in CM-IDLE state, etc.

Therefore, for power saving mechanisms, like MICO mode and eDRX in CM-IDLE state, how to apply the power saving mechanism to ensure that the UE does not attempt PLMN access when there is no coverage still needs to be studied.

The key issue intends to study architectural enhancement for UE in discontinuous coverage.

At least the following aspects need to be investigated:

-	Based on the coverage information of the UE:

-	whether and how to enhance the power saving mechanisms, e.g. PSM, MICO mode and eDRX in CM-IDLE state, in order to make sure that the UE:

-	does not attempt PLMN access when there is no network coverage, and

-	when there is network coverage the UE attempts PLMN access as needed e.g. to transfer signalling, transfer data or receive paging etc.

NOTE:	network coverage can be provided by an RAT supported by the UE.