SA WG2 Meeting #S2-150E	S2-2202985r01

06 - 12 April, 2022, Electronic meeting

Title:	Draft Reply LS on Clarifications on Namf_MBSCommunication N2MessageTransfer service operation

Response to:	S2-2201933/ C4-221427

Release:	Rel-17

Work Item:	5MBS, NR_MBS-Core

Source:	SA2

To:	CT4

Cc:	RAN3

Contact Person:

Name:	Thomas Belling

E-mail Address:	Thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

The MB-SMF uses the Namf_MBSCommunication N2MessageTransfer service operation during the multicast MBS session activation and deactivation procedure and multicast session update procedure, as specified in clauses 7.2.5.2, 7.2.5.3, 7.2.6 and 9.3.3.2 of TS 23.247.

Upon receipt of this service request, the AMF distributes the N2 payload received from the MB-SMF towards the NG-RAN(s) serving the MBS service area of the multicast MBS session.

In all these 3 procedures, based on the description in the aforementioned clauses, CT4 assumes that:

- the AMF returns a successful response to the MB-SMF as soon as at least one NG-RAN returns a successful response to the AMF;

- the AMF returns an unsuccessful response to the MB-SMF, if the AMF cannot serve the request at all (e.g. unknown PDU Session ID) or if none of the NG-RANs returns a successful response;

and

- in either case, the AMF does never forward any successful or unsuccessful N2 payload(s) back to the MB-SMF.

CT4 kindly asks SA2 to:

- confirm whether CT4 understanding is correct;

SA2 confirms that understanding. The output of Namf_MBSCommunication service operation is without N2 container.

- clarify how the system should behave when a multicast session activation, deactivation or update request would fail for an NG-RAN; and

This is not specified in this Release.

2. Actions:

To SA2 group.

ACTION: 	CT4 kindly asks SA2 group to answer CT4 questions and consider clarifying stage 2 accordingly, if necessary.

3. Date of Next SA2 Meetings:

SA2#151e	16th - 20th May 2022	electronic meeting