3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2203146

Elbonia, April 6 – 12, 2022	(revision of S2-2202466r01)

Source:	Huawei, HiSilicon

Title:	KI#5 Solution: Multi-path transmission via Layer-2 U2N Relay

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This document proposes a solution for multi-path transmission via L2 U2N Relay.

# 1. Introduction/Discussion

This contribution introduces a new solution to solve multi-path transmission via Layer-2 U2N Relay in KI#5 of TR 23.700-33.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  |  |  | 5 |
| X |  |  |  | X |

# * * * * Second change (all new) * * * *

## 6.X	Solution #X: Multi-path transmission via Layer-2 UE-to-Network Relay

### 6.X.1	Description

This solution resolves Key Issue #5 Support of multi-path transmission via Layer-2 UE-to-Network Relay. As illustrated in figure 6.X.1-1, a (Remote) UE can use path #1 and path #2 for multi-path transmission between the UE and NG-RAN, where path #1 is direct network communication path (the UE directly accesses NG-RAN), and path #2 is indirect network communication path via a Layer-2 UE-to-Network Relay (the UE accesses NG-RAN using a Layer-2 UE-to-Network Relay). For the Layer-2 UE-to-Network Relay case, NG-RAN controls the multi-path connection establishment and the specific transmission such as splitting or duplication.

Figure 6.X.1-1: multi-path transmission using Layer-2 UE-to-Network Relay

This solution proposes a high-level procedure for multi-path transmission via Layer-2 UE-to-Network Relay and coordination with RAN2 WG is needed for solution alignment and update based on RAN2 progress and normative work.

### 6.X.2	Procedures

Figure 6.X.2-1: high-level procedure for Multi-path transmission via Layer-2 U2N Relay

1.	The Remote UE may get the authorization/policy from the PCF or be preconfigured with an indication that that the Remote UE is authorized for multi-path transmission via a Layer-2 UE-to-Network Relay. When the Remote UE is in CM-CONNECTED state, the Remote UE’s serving NG-RAN may get the authorization information from AMF to indicate whether the Remote UE is allowed to use multi-path transmission via Layer-2 UE-to-Network Relay, and AMF gets the authorization information from UDM as part of the subscription data for 5G ProSe services.

2.	The Remote UE sets up a RRC connection via either a direct or indirect path to NG-RAN. For indirect path, steps 0-6 within the procedure as defined in clause 6.5.2.2 in TS 23.304 [3] are performed.

3.	During PDU session establishment/modification procedure, NG-RAN can decide that multi-path transmission for a QoS flow can be performed during or after a QoS Flow establishment, considering the authorization/policy in step 1.

4.	NG-RAN sets up an AS connections to the Remote UE via both the direct network communication path and the indirect network communication path and then NG-RAN transfer of data over the QoS flow with the Remote UE using the dual paths.

Editor’s note: How the AS connections are established depends on RAN2 WG.

### 6.X.3	Impacts on services, entities and interfaces

Remote UE:

-	Multi-path connection establishment and the transfer of data via both paths.

NG-RAN:

-	Multi-path connection establishment and the transfer of data via both paths, taking into account authorisation from AMF.

AMF:

-	 Provide authorisation to NG-RAN.

# * * * * End of changes * * * *