3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2203226

# Elbonia, April 6 – 12, 2022	(revision of S2-2202498r01)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0519 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update inputs parameters for the Nadrf_DataManagement_StorageRequest service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The existing description for inputs of the Nadrf_DataManagement_StorageRequest service in TS 23.288 is very simple, some neceasary information are missing for data/analytics stoagre scenario, for example, Analytics ID(s) / Event ID (s), Target of Analytics Reporting or Target of Event Reporting are lacking, but actually these information are very useful for the subsequent data retrieval operation, without them the ADRF can not know what data to provide to the data consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Nadrf_DataManagement_StorageRequest service operation to add more required parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inputs for Nadrf_DataManagement_StorageRequest service are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2, 10.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2B.2	Historical Data and Analytics storage

The procedure depicted in figure 6.2B.2-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

Figure 6.2B.2-1: Historical Data and Analytics storage

1.	The consumer sends data and/or analytics to the ADRF by invoking the Nadrf_DataManagement_StorageRequest (collected data, analytics, Service Operation, Analytics Specification or Data Specification and Time Window) service operation.

2.	The ADRF stores the data and/or analytics sent by the consumer. The ADRF may, based on implementation, determines whether the same data and/or analytics is already stored or being stored based on the information sent in step 1 by the consumer NF and, if the data and/or analytics is already stored or being stored in the ADRF, the ADRF decides to not store again the data and/or analytics sent by the consumer.

3.	The ADRF sends Nadrf_DataManagement_StorageRequest Response message to the consumer indicating that data and/or analytics is stored, including when the ADRF may have determined at step 2 that data or analytics is already stored.

# * * * * Next change * * * *

10.2.2	Nadrf_DataManagement_StorageRequest service operation

Service operation name: Nadrf_DataManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message.

Inputs, Required: Data or Analytics to be stored, Service operation, Analytics Specification or Data Specification and Time Window.

"Service Operation" identifies the service used to obtain the data or analytics from a Data Source (e.g. Namf_EventExposure_Subscribe or Nnwdaf_AnalyticsSubscription_Subscribe).

"Analytics Specification or Data Specification" is the "Service Operation" specific required and optional input parameters that identify the data that was stored (e.g. Analytics ID(s) / Event ID (s), Target of Analytics Reporting or Target of Event Reporting, Analytics Filter or Event Filter, etc.). Service Operations and input parameters are defined in clause 7 for NWDAF and in TS 23.502 [3], clause 5.2 for the other NFs.

"Time Window" is the start and stop time when the data or analytics was collected from the Data Source.

Inputs, Optional: None.

Outputs Required: Result Indication.

Outputs, Optional: Storage Transaction Identifier.

# * * * * End of changes * * * *