SA WG2 Meeting #150e	S2-2203260

# April 6 – 12, 2022; Elbonia

Source: 	China Mobile

Title: 	Solution on support 5G VN service area restriction with SMF service area

Document for: 	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: Propose solution for Key issue 1.

1 Discussion

2 Proposal

It is proposed to update TR 23.700-74 v0.1.0 on FS_GMEC as follows

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| # Support 5G VN service area restriction with SMF service area | X |  |  |  |  |

# * * * * Second change * * * *(all new texts)

## 6.X	Solution #X: Support 5G VN service area restriction with SMF service area

### 6.X.1	Key Issue mapping

This solution is for Key Issue #1, which addresses following aspects:

-	How to provision (i.e. set, modify and delete) the service area or QoS applicable to each UE of the group via exposure interface, i.e.:

-	What is the information that constitutes the service area applicable to each UE of the group via the exposure interface, and how to enforce the service area applicable to each UE of the group by reusing the existing mechanisms;

### 6.X.2	Description

5G LAN type service may only support the UEs, which are belong to 5G VN group, when the they are located in specific service area.

In this solution, it is proposed to introduce 5G VN group service area in UDM subscription data, and SMF judge whether the UE is located within the 5G VN group service area. If the UE is located in it, the PDU session for 5G VN group can be established, otherwise the PDU session establishment request will be rejected by SMF.

SMF get the UE location from AMF.

### 6.X.3.1	Procedures for support of SMF service area

UE

RAN

AMF

SMF

UPF

UDM

NEF/AF

8. UE change location

1. AF provides 5G VN group service area in UDM

2. PDU session establishment request

9.AMF notify SMF UE location when UE move out of service area

3. Nsmf_PDUSession_CreateSMContext request

4. Subscription retrieval

7. SMF subscribe UE location to AMF

5. SMF determines whether UE is within 5G VN service area

10. SMF determine to release the PDU session

6. TS23.502 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout. Step 5-16

Figure 6.X.3.2-1: Procedures for support of SMF service area

1. The AF provides 5G VN group information (GPSI, DNN, S-NSSAI and 5G VN group service area) through NEF by invoking Nnef_ParameterProvision_Create (as described in TS 23.502 clause 4.15.6.2) and to UDM by invoking Nudm_ParameterProvision_Create (as described in TS 23.502 clause 5.2.3.6.3).

2. UE initiates 5G VN group DNN PDU session establishment request.

3. AMF sends PDU session establishment request to SMF with carrying UE location information.

4. SMF retrieves SM subscription data from UDM with Nudm_SubscriberDataManagement request include 5G VN DNN, S-NSSAI, and 5G VN group service area.

5. SMF determines whether the UE is within the 5G VN group service area. If not, the SMF may reject the PDU session establishment.

6. PDU session establishment procedure.

7. SMF subscribe the UE location with AMF by invoke Namf_EventExposure_Subscribe request. In This step, the SMF may send the 5G VN group service area to AMF as the location subscription requirement.

8-9. During UE mobility, when UE moves out of the 5G VN group service area, the AMF can notify the SMF.

10. SMF will verify if the UE is within the service area of 5G VN group, if UE move out of the service area, 5G VN PDU session will be released.

### 6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

# * * * *end of change * * * *