3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2203275

Elbonia, April 6 – 12, 2022	(revision of S2-2202732r03)

Source:	Huawei, HiSilicon

Title:	Solution for network-assisted DAA

Document for:	Approval

Agenda Item:	9.5

Work Item / Release:	FS_UAS_Ph2 / Rel-18

Abstract: This contribution introduces a new solution to solve the network-assisted DAA for KI#3 in TR 23.700-58.

1. Introduction/Discussion

This contribution introduces a new solution to solve the network-assisted DAA for KI#3 in TR 23.700-58.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-58.

# * * * * First change * * * *

## 6.X	Solution #X: Network-assisted DAA

### 6.X.1	Key Issue mapping

This solution tries to solve the key issue that what network-assisted (ground based) DAA solution may be applicable.

### 6.X.2	Description

USS knows the flight path of each UAV UE during the UAV Flight Authorization procedure or Application layer report. In some case, the flight path of different UAV UEs is overlapped or within the same area. USS can request to the 5GS system to perform the DAA between any two UAV UEs whose flight path may be overlapped or within the same area, i.e., network-assisted DAA solution is used.

The network-assisted DAA is useful, for scenarios where regulations might not consider a device to device solution sufficient or for scenarios where the availability of PC5 connectivity in all UAVs cannot be assumed.

This solution applies to the UAV UEs belonging to the same PLMN. For the UAV UEs belonging to the different PLMNs, USS can get the UAV location from the network and check by itself.

Editor’s NOTE: it is FFS how the solution works if multiple USS operated by different parties serve a specific country.

### 6.X.3	Procedures

Figure 6.X.3-1: high-level procedure for network-assisted DAA

- USS as AF/LCS client sents the relative positioning request to GMLC, including UAV UE1 ID (UE1 GPSI) and UAV UE2 ID (UE2 GPSI), and optional including a distance threshold. GMLC determines a scheduled location time to get the location information of UE1 and UE2 at the same time.

- GMLC obtains UAV UE1 location using GMLC based procedure as described in clause 6.1.2 of TS 23.273 [X], where the scheduled location time determined in step 1 is used.

- GMLC obtains UAV UE2 location using GMLC based procedure as described in clause 6.1.2 of TS 23.273 [X] , where the scheduled location time determined in step 1 is used.

- GMLC calculates the relative positioning result based on the UAV UE1 location and UAV UE2 location from step 2 and step 3.

- GMLC reports the relative position result to USS. And optionally before that, GMLC checks the relative position result is lower than the distance threshold.

After the USS gets the relative position result, USS may send the DAA notification to UAV or the corresponding UAV controller via application layer, so that the UAV can change the flight path timely to avoid the collisions. The specific action of USS is out of scope of this specification.

This solution can be extended to cover the DAA between any two UAV UEs in a UAV UE list, where USS provides a UAV UE list in the relative positioning request of step 1.

### 6.X.4	Impacts on Services, Entities, and Interfaces

GMLC:

-	Support of calculate the relative positioning between any two UAV UEs.

# * * * * End of changes * * * *