SA WG2 Meeting #S2-150e(e-meeting)	S2-2203342

06 - 12 April 2022, Electronic Meeting	(revision of S2-2202627)

Source:	CATT

Title:	KI#3, KI#4, New Sol: Solution to support ranging/sidelink positioning

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL

Abstract of the contribution: Introduce a new solution to key issues #3 and #4.

1. Discussion

This pCR is proposed to add a new solution to key issue#3 and #4:

- KI#3: Ranging/ Sidelink Positioning device discovery.

- KI#4: Control of Operations for Ranging/ Sidelink.

To trigger the sidelink positioning procedure between an observer UE and target UE(s), the application layer in the observer UE layer provides ranging/sidelink positioning parameters to ranging layer. The ranging layer in the observer UE broadcasts a ranging/sidelink positioning request message which includes the parameters provided by the application layer.

The observer UE or the target UE is responsible to trigger the sidelink positioning procedure which is in RAN WG scope to obtain measurements and calculates the sidelink location. If the procedure is triggered by the target UE, it further returns the sidelink location to the observer UE.

When the observer UE obtains the sidelink location, the ranging layer of the observer UE provides the sidelink location to the application layer.

2. Proposal

This paper proposes to add a new solution to key issue#3 and #4.

* First change *

6.0	Mapping Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  |  |  |  |
| X | KI#3 | KI#4 |  |  |

* Second change *

6.x	Solution #x: Solution to support Ranging/Sidelink Positioning

6.X.1	General

This solution addresses the KI#3: Ranging/ Sidelink Positioning device discovery and the KI#4: Control of Operations for Ranging/ Sidelink.

6.X.2	Functional Descriptions

To trigger the sidelink positioning procedure between an UE-1 and UE-2(s), the application layer in UE-1 provides ranging/sidelink positioning parameters to ranging layer. The ranging layer in the UE-1 broadcasts a ranging/sidelink positioning request message which includes the parameters provided by the application layer.

After the UE-2(s) receives the message, the sidelink positioning procedure is triggerred as follows to get the Ranging/Sidelink positioning result:

-	UE-1 based sidelink positioning: the UE-1 triggers the sidelink positioning procedure to obtain measurements and calculates the Ranging/Sidelink positioning result. Or

-	UE-1 assisted sidelink positioning: the UE-2(s) triggers the sidelink positioning procedure to obtain measurements, calculates the Ranging/Sidelink positioning result and returns it to the UE-1.

NOTE:	The details of the sidelink positioning procedure is in RAN WG scope.

NOTE:	Both the UE-1 and UE-2 can act as the reference UE. The capability exchange and reference UE negotiation between the UE-1 and UE-2 is in RAN WG scope.

When the UE-1 obtains the Ranging/Sidelink positioning result, the ranging layer of the UE-1 provides it to the application layer.

6.X.3	Ranging/ Sidelink Positioning Procedures

Figure 6.X.3.1-1: Ranging/Sidelink Positioning Procedure

1.	Application layer provides sidelink positioning parameters to ranging layer, e.g. required location QoS, target UE’s Application Layer ID may also be included in the parameters.

2.	UE-1 broadcasts a Ranging/Sidelink positioning request. The message includes UE-1’s Application Layer ID, Target UE’s Application ID and required QoS.

3.	UE-2 returns the Ranging/Sidelink Positioning response to UE-1 if the Application Layer ID for UE-2 matches.

4.	UE-1 initiates the Sidelink Positioning procedure. The details of the procedure are in scope of RAN WGs, e.g. capability exchange, reference UE negotiation.

Editor's note: It is FFS how to configure the role of the UEs (e.g. reference UE) and under which conditions these roles apply.

5.	UE-1 or UE-2 obtains the positioning measurements and calculates the Ranging/Sidelink positioning result. The details of the step are in scope of RAN WGs.

6.	UE-1 and UE-2 shares the Ranging/Sidelink positioning result.

Editor's note: It is FFS on how to share the Ranging/Sidelink positioning result between UE-1 and UE-2.

7.	The ranging layer in UE-1 provides the Ranging/Sidelink positioning result to the application layer.

6.X.4	Impacts on services, entities, and interfaces

PC5 interface is enhanced to support ranging/sidelink positioning.

* End of changes *