SA WG2 Meeting #150e	S2-2203460

# April 6th – 12th, 2022; Elbonia                  	       (revision of S2-2202562)

Source: 	vivo

Title: 	FS_eNPN_Ph2 Solution of WT#5 - hosting network selection

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This contribution proposes a solution for WT#5, KI#4.

1	Discussion

This contribution proposes a solution for a UE to select a hosting network based on Hosting Network Selection and Access Information received from local service provider.

The consists the following information:

-	Hosting Network Identifier, e.g. SNPN ID in case of the hosting network is an SNPN, CAG ID and PLMN ID in case of the hosting network is a PNI-NPN.

-	The time condition information when the hosting network provides access service.

-	The location condition information where the hosting network provides access service.

-	Optionally, credential used for the UE to access the hosting network. Credential may be only provided in case hosting network is an SNPN.

It is assumed that the UE establishes relationship with the local service provider, e.g. buy a sports game ticket from a stadium.

2 Proposal

It is proposed to update TR 23.700-08 as follows.

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| X |  |  |  | X |  |  |

************************** NEXT CHANGE ***********************

6.X	Solution X: Solution for hosting network selection

6.X.1	Introduction

This solution solves several requirements listed in Key Issue#4.

6.X.2	Functional Description

In this solution, the hosting network sends Hosting Network Selection and Access Information to the local service provider, who provides service to UEs accessing the hosting network.

The Hosting Network Selection and Access Information consists the following information:

-	Hosting Network Identifier, e.g. SNPN ID in case of the hosting network is an SNPN, CAG ID and PLMN ID in case of the hosting network is a PNI-NPN.

-	The time condition information when the hosting network provides access service.

-	The location condition information where the hosting network provides access service.

-	Optionally, credential used for the UE to access the hosting network. Credential may be only provided in case hosting network is an SNPN.

The UE receives the Hosting Network Selection and Access Information from the localized service provider. The UE selects a hosting network when the time and location condition information is satisfied.

6.X.3	Procedures

Figure 6.x.3-1 UE receives hosting network selection and access information from local SP

1. The hosting network server sends Hosting Network Selection and Access Information to the NEF.

The hosting network (HN) server stores the Hosting Network Selection and Access Information and send it to the local service providers who provide services to the UEs accessing the hosting network.

2. The NEF sends the Hosting Network Selection and Access Information to the local service provider App.

Editor's note:	The details of step 1 and 2 are FFS.

3. The UE establishes connection with the local service provider App.

The UE may establish a PDU Session via its home network and establishes connection via the PDU Session.

NOTE:	How the UE knows the address of the local service provider App is out of 3GPP scope.

4. The UE selects a hosting network based on the Hosting Network Identifier in the Hosting Network Selection and Access Information when the corresponding time and location condition information is satisfied.

Editor's note:	Whether the Hosting Network Selection and Access Information can trigger manual hosting network selection or not is FFS.

6.X.4	Impacts on services, entities, and interfaces

UE impact:

-	Ability to receive Hosting Network Selection and Access Information from the local service provider App.

-	Ability to perform hosting network selection based on the received Hosting Network Selection and Access Information.

Hosting Network impact:

-	Ability to send Hosting Network Selection and Access Information to the local service provider.

************************** END OF CHANGE ***********************