3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2203476

Elbonia, April 6 – 12, 2022	(revision of S2-2202529r02)

Source:	Huawei, HiSilicon

Title:	Solution2 Update for UL scheduling

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: This paper proposed UL scheduling based on a burst arrival time offset value to application for low latency communication on top of solution#2.

1. Introduction/Discussion

The key issue#6: Adapting downstream scheduling based on RAN feedback for low latency communication also needs to study UL scheduling as pointed by the NOTE:

“NOTE 2:	Although the focus is on downstream scheduling, any optimization on upstream scheduling should not be precluded if similar enhancement as for downstream scheduling applies.”

Considering that UL scheduling is actually also very important because of less time slot resources, this paper proposes, on top of solution #2, for uplink scheduling to meet low latency communication.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-25 V0.1.0.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[6]	IEEE 802.1Qcc: "IEEE Standard for Local and Metropolitan Area Networks--Bridges and Bridged Networks -- Amendment 31: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[7]	IEEE Std 802.1AS: "IEEE Standard for Local and Metropolitan Area Networks-Timing and Synchronization for Time-Sensitive Applications".

[8]	IEEE Std 1588: "Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems".

[x]	3GPP TS 38.321: "Medium Access Control (MAC) protocol specification".

# * * * * Second change (all new text) * * * *

6.X	Solution #X: Burst arrival time adaptation

6.X.1	Introduction

This solution enables the network to adjust the burst arrival time by signaling positive or negative offset values (e.g. +3 ms) for UL scheduling and DL scheduling to the AF so that the AF can adjust the burst sending time accordingly.

6.X.2	Functional Description

This solution is based on the following principles:

- For DL scheduling: refer to solution #2.

- For UL scheduling: Upon reception of UL packets from UE, UE determines a relative burst arrival time offset value in reference to the current Burst Arrival Time experienced by UE (i.e., in reference to when UE currently receives bursts) and the scheduling UL time slot at UE (e.g. in Configured Grants, as defined in TS 38.321[x]). UE sends the time offset to RAN via RRC message when the time offset value reaches the configured threshold, and NG-RAN sends the burst arrival time offset value to SMF in the same way as for DL scheduling.

-	For downlink or uplink flows AF adapts the burst sending time based on the received offset.

6.X.3	Procedures

In addition to clause 6.2.3 in solution #2, RRC message to signal support of burst arrival time offset value from UE to RAN

6.X.4	Impacts on services, entities and interfaces

Besides the impacts documented in clause 6.2.4 of solution #2:

-	UE: Support of determining and signalling burst arrival time offset to RAN via RRC message.

# * * * * End of changes * * * *