3GPP TSG-WG SA2 Meeting #151E e-meeting

Elbonia, May 16 – 20, 2022	S2-2204211

Source:	OPPO

Title:	KI#2 Update Solution#2 to resolve some Editor’s notes and introduce the network consent and multiple DCAF

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: Update the solution#2 to resolve the ENs and introduce the network consent and multiple DCAF.

# 1.	Introduction

This contribution adds some justifications on the benefits of exposing information to the UE instead of to the AF in order to resolve the ENs captured in the 6.2.1. Also, this contribution provides some examples to describe how the UE can leverage the 5GC information to assist AI/ML operations based on some analytic IDs that already supported by NWDAF.

In addition, when the UE requests some analytics results from the NWDAF. The analytics ID requires network consent before the exposure to the UE. Therefore, this contribution describes how the network consent can be retrieved by the NEF/NWDAF in order to verify the UE subscription for obtaining the specific analytics from network.

This contribution addresses the second EN in clause 6.2.2 of TR 23.700-80 by referring to the clause 6.2.8.2.4.1 as captured in 23.288[6].  Based on implementation, only a single PDU Session is allowed between the UE and the Data Collection AF (DCAF) which is affiliated with a specific S-NSSAI/DNN regardless the DCAF is in trusted domain or untrusted domain.

This contribution also envisions more than one DCAF to support the given application at different service areas. As the UE may move across different service areas served by different DCAFs while it is communicating with the NWDAF to collect analytic results, this contribution proposes to leverage the existing procedures as described in clause 4.17 of TS 23.502 [4] to support DCAF switching to ensure the accurate communication between the UE’s serving DCAF and the corresponding NWDAF.

NOTE: How UE switches to a different DCAF is application layer decision and not within the scope of SA2.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-80:

# * * * * First change * * * *

## 6.2	Solution #2: User Plane solution for 5GC information exposure to UE

### 6.2.1	Description

This solution is addressing KI#2: 5GC information exposure to UE.

As documented in TS 22.261 [2], the AI/ML operation may be split between UE application client and AF. The UE application client performs local AI/ML operation for the privacy-sensitive and delay sensitive part and sends the intermediate data to the AF. The AF executes the remaining parts and feeds the inference results back to the device. This AI/ML operation requires reliable and low latency data transmission for the AI/ML traffic. Moreover, 5G system should be able to expose the predicted changes of network condition (i.e. bitrate, latency, reliability) per UE to an authorized third party as specified in the TS 22.261[2].  Currently, the network condition related information, like the Network Slice load predictions information, NF load analytics and Network performance analytics are already supported in TS 23.288 [6]. If the 5GS can expose these analytics output to the UE application client, it is beneficial to assist the UE client to decide the AI/ML operation splitting, e.g. the UE application client may decide to perform the AI/ML operation when the Network Slice load is not high, the gNB status and resource usage that indicated in Network performance analytics output is in good condition.

The requirement that 5G system shall be able to provide event alerting to an authorized 3rd party, together with a predicted time of the event (e.g., alerting about traffic congestion or UE moving into/out of a different geographical area) was also captured in 22.261. The User Data Congestion Analytics supported by NWDAF can be used to address such requirement. When the UE received the alerting about the traffic congestion, the UE can decide to transmit priority portion of AI data before the traffic congestion happens.

Regarding the Distributed / Federated Learning between UE application Client and the AF, within each training iteration, UE downloads the model from AI server and reports the interim training results to the AI server. The Server aggregates the interim training results from all the selected FL UEs and updates the global model. The updated global model is then distributed back to all the UEs. The above FL procedure may last for a long time with a huge data transmission with 5GS, in order to reduce the impacts to the non-FL traffic it is also beneficial to expose the NW load related prediction to FL UEs and AI server. The FL UEs and AI server can use this information to select a proper time for the FL related data transmission, e.g. the UE may decide to accept the FL learning operation when the QoS Sustainability Analytics predict the QoS will not change during a time period and the gNB status and resource usage that indicated in Network performance analytics output is also good enough.

Since Rel-17, the DCAF functionality which is part of the AF was introduced to enable the NWDAF to perform data collection from the Application. Given the communication path between NWDAF and the DCAF has been introduced for data collection, it is reasonable to extend the existing DCAF/AF functionality and architecture as defined in TS 26.531 [7] to expose network data analytics related to the UE to assist the UE's local Application AI/ML operation.

The advantage of sending the analytics results to the UE via DCAF is that DCAF can act as a centralized node to manage analytics requests from multiple UEs that are served by the same AF. A DCAF can store the analytics results provided by the NWDAF.  When a new UE or a new application request from the UE requests the same analytics, the DCAF can provide the analytics results to the UE directly as long as the network consent is also granted to the application in the UE and in the AF.

For the UE communicate with DCAF, both the direct way (from the UE Application to the DCAF, either in trusted domain or untrusted domain) and indirect way (from the UE Application to the Application server and from the Application server to the DCAF) can be used. The direct way and indirect way are the same as far as the data collection procedure described in TS 26.531 [x].

The following assumptions are made by this solution:

-	The UE can Subscribe/Request to NWDAF via DCAF to request the information from 5GC (e.g. the Analytics result from the NWDAF).

-	The communication services and functional architecture between UE and DCAF is in the scope of SA4 and shall be compatible with the existing architecture.

-	Network specific indication is required for the UE to access UE-related network information.

-	There could be more than one DCAF to support the given application at different service areas.

### 6.2.2	Procedures

A user plane data collection from UE application client was approved in clause 6.2.8 in TS 23.288 [6] in Rel-17. A UE Application is configured the address of DCAF by the Application server. The UE Application client establishes a connection to the DCAF over user plane via a PDU session. The DCAF communicates with the UE Application Client and collects data from UE Application to NWDAF.

The same user plane connection between UE application and DCAF is used by the UE Application Client to request data exposure from NWDAF.

Based on the justification above, the following information can be exposed to the UE via the support of the DCAF based on network consent acquired by UE:

-	QoS Sustainability Analytics;

-	Network Slice load predictions information;

-	NF load analytics;

-	User Data Congestion Analytics; and

-	Network Performance Analytics.

Editor's note:	How to extend the interaction between UE and DCAF as defined in TS 26.531 [7] to enable UE to subscribe/request information to/from 5GC NF (e.g. NWDAF) via DCAF is in SA WG4 scope.

The SLA between the operator and the Application Service Provider further include the Supported Analytics ID list.

The Supported Analytics ID indicates the Analytics outputs that allowed by the operator to expose to the UE application Client and AF. The Supported Analytics ID list will be stored in DCAF.

The Application Service Provider will provision the Supported Analytics IDs to UE when UE performs application layer registration.

The UE requested data exposure procedure is described in Figure 6.2.2-1.

Figure 6.2.2-1: UE requested data exposure procedure

1.	UE sends the Analytics Request to DCAF-1 via an application layer signalling. The DCAF-1 address is provided by application server to UE application client and UE establishes the PDU session to DCAF-1 are described in clause 6.2.8 of TS 23.288 [6]. The requested Analytics ID(s) are included in the application layer request message to DCAF-1. UE may also provide the S-NSSAI to DCAF-1.

DCAF-1 can retrieve the UE IP address from the source IP address of the received packet.

Since the supported pairs of S-NSSAI+DNN for the PDU session established for UE to DCAF-1 are configured in the DCAF-1, DCAF-1 may also retrieve the S-NSSAI+DNN for the PDU sessions that used by the UE sent the Analytics request.

As described in 23.288[6], based on implementation decision, for the UE to access the Data Collection AF, only a single PDU Session is allowed to be established between the UE and the Data Collection AF which is corresponding to a specific S-NSSAI/DNN.

2.	DCAF-1 sends Nnrf_nwdafdiscovery _request to NRF to discover the NWDAF that supports the UE requested Analytics ID(s). DCAF-1 may also provide the S-NSSAI for NWDAF selection.  In case of DCAF-1 resides in the untrusted domain, DCAF-1 sends request to NRF via NEF.

3.	NRF sends the Nnrf_nwdafdiscovery_response with the discovered NWDAF identity.

4.	DCAF-1 retrieves UE Identity as described in clause 4.15.10 of TS 23.502 [4] based on UE IP address and optionally the S-NSSAI. For DCAF-1 in trusted domain, the UE Identity is the SUPI, for DCAF-1 in untrusted domain, the UE ID is the GPSI.

Editor's note:	Whether and how to improve the efficiency for UE Identity retrieving procedure is FFS.

5.	DCAF-1 informs the NRF of its profile including the UE ID by invoking the Nnrf_NFManagement_NFRegister or Nnrf_NFManagement_NFUpdate service.

6.	DCAF-1 subscribes to NWDAF for the analytics request with UE Identity and Analytics ID(s). Step 5a is for DCAF-1 in trusted domain and step 5b is for DCAF-1 in untrusted domain.

When performing step 5a, the AF asks the NWDAF for authorization information to check if the UE is allowed to obtain analytics ID from the network. The NWDAF determines the authorization information for the UE based on local policy and the network consent as a UE subscription data from the UDM about whether the UE has subscribed to the service that obtaining some specific analytics ID from network. Then the NWDAF sends the authorization information to the AF.

When performing step 5b, the AF asks the NEF for authorization information to check if the UE is allowed to obtain analytics ID from the network. The NEF determines the authorization information for the UE based on local policy and the network consent as a UE subscription data from the UDM about whether the UE has subscribed to the service that obtaining some specific analytics ID from network. Then the NEF sends the authorization information to the AF.

The DCAF-1 can store the analytics results provided by the NWDAF, if a new UE or a new application request from the UE wants to request the same analytics, the DCAF-1 will ask the NEF or NWDAF for authorization information to check if the UE is allowed to obtain analytics ID from the network. Then the DCAF-1 will provide the analytics results to the UE directly.

7.	If NWDAF does not have available analytics result for the requested analytics ID, the NWDAF will trigger the analytics procedures for the analytics ID(s). NWDAF performs user consent check to UDM to determine if the analytics procedure is allowed or not.

8.	Analytics procedure is performed as described in TS 23.288 [6].

9.	NWDAF sends analytics result to DCAF-1. Step 9a is for DCAF-1 in trusted domain and step 9b is for DCAF-1 in untrusted domain.

NOTE:	The security aspects of exposure of network data must be evaluated by SA WG3.

10.	DCAF-1 sends the analytics result to UE application client via application layer signalling.

The UE may move into a different service area supported by a different DCAF-2. During the mobility, the DCAF-1 can transfer the UE context to DCAF-2 via an application layer signalling in order to resume service with NWDAF.

11. After the UE context is received via an application layer signalling, DCAF-2 updates its profile by invoking the Nnrf_NFManagement_NFUpdate service. In case of DCAF-2 resides in the untrusted domain, DCAF-1 sends request to NRF via NEF.

12.	If a Subscribe/Notify service operation is invoked in step 6, the NWDAF may further notify the analytics to UE e.g. periodically).

13. NWDAF sends Nnrf_nwdafdiscovery _request to NRF to discover the DCAF-2 that is serving the UE currently.

14. NRF sends the Nnrf_nwdafdiscovery_response with the discovered identity of DCAF-2.

15.	NWDAF exposes the requested analytics to UE via DCAF-2.

### 6.2.3	Impacts on services, entities and interfaces

DCAF:

-	receives analytics request from UE via application layer signalling.

-	discovers NWDAF for the received analytics request from UE.

-	sends analytics request to NWDAF.

-	receives analytics response from NWDAF.

-	sends the received analytics result to UE via application layer signalling.

-	checks with NEF about authorization information for the UE if it is allowed to obtain analytics ID from the network.

NEF:

-	Checks about authorization information for the UE based on local policy, the UE subscription data about whether the UE has subscribed to the service that obtaining some specific analytics ID from network.

NWDAF:

- Checks about authorization information for the UE based on local policy, the network consent as a UE subscription data about whether the UE has subscribed to the service that obtaining some specific analytics ID from network.

- For the periodical notification, NWDAF discovers the DCAF that is currently serving the target UE before the notification.

UDM:

- Send the network consent to NEF/NWDAF

NRF：

- Maintain the mapping between the UE ID and its serving DCAF

- * * * * End of changes * * * *