3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204385

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4:Updates Solution16 for Selecting the same EAS-DNAI for collection of UEs

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: Updates Solution16 for Selecting the same EAS/DNAI for collection of UEs.

1. Introduction/Discussion

This contribution solves the following ENs:

Editor's note: It is FFS what the difference between the scenarios is, and how the scenarios are reflected in the procedures below.

Editor's note: It is FFS how eas_correlation and dnai_correlation are expressed in the PCC Rules.

Editor's note: How the SMF determines the UE belongs to the collection of UEs and what information and how the SMF retrieves for making the decision.

Editor's note: It is FFS what information is stored in the UDR, what parameter(s) are used as a key by the SMF to retrieve the information, and when and how the information in UDR is removed.

There are different ways for AF to express a UE collection, for example, (1) a UE ID list, including all the UEs in the UE collection, could be explicitly provided by AF; (2) a group id, all the UEs in the UE collection has the same group id; (3) any UE within a dedicated location area. This solution aims to support different ways for expressing UE collection.

The following illustrates how SMF determines the same EAS/DNAI for UE of a UE collection when the UE accesses to specific service.

Figure 1 Overview of how SMF determining selecting the same EAS/DNAI for UEs of a UE collection

Step 1. AF sends AF Request for selecting the same EAS or DNAI for collection of UEs accessing specific Application (identified by appID or FQDN). The following information elements are included in the AF request:

| Information | Notes |
| --- | --- |
| Traffic Description | Including AppID, or FQDN, to which collection of UEs access, if AppID is provided it assumes SMF could get FQDN corresponding to the AppID. |
| Target UE Identifier(s) | e.g. UE ID list/group ID/any UE, used to define the member of UE collection |
| Correlation Indication | Indicate selecting the same EAS or DNAI. e.g.eas_correlation indication/dnai_correlation indication |
| Correlation ID | Uniquely identifies the UE collection, e.g. AF Transaction ID |
| Spatial Validity Condition | Existing in case of “any UE”, the UE collection is defined as “any UE in the location area” |

Step 2. PCF(s) create PCC rule for each UE in the UE collection. The UE could be determined based on Target UE Identifier(s) provided by AF. The PCC rule includes the following:

| Information Name | Notes |
| --- | --- |
| Service data flow template | e.g. appID/FQDN |
| AF influenced Traffic Steering Enforcement Control | This part describes information required for AF influenced Traffic Steering. |
| eas_correlation indication/dnai_correlation indication |  |
| Correlation ID |  |

Step 3. After UE sending out DNS Query, SMF receives FQDN reported by EASDF for the DNS Query, if the FQDN match with the appID/FQDN from PCC rule and eas_correlation indication/dnai_correlation indication is set, then SMF determines UE belongs the UE collection identified by Correlation ID, and decide to select the same EAS or DNAI.

Based on the above analysis, the following ENs are removed:

Editor's note:	It is FFS what the difference between the scenarios is, and how the scenarios are reflected in the procedures below.

Editor's note:	It is FFS how eas_correlation and dnai_correlation are expressed in the PCC Rules.

Editor's note:	How the SMF determines the UE belongs to the collection of UEs and what information and how the SMF retrieves for making the decision.

In case of multiple SMFs, UDR is used for maintaining Information for collection of UE, which includes the following Information elements:

- Correlation ID

- appID/FQDN

- EAS IP address or DNAI

The Information for collection of UE is removed by SMF when the UE collection is removed by AF.

The Correlation ID is used as Data Key and appID/FQDN as Data Sub Key. The Information in UDR is as follows:

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
| …… | …… | …… | …… |
| Application data | Packet Flow Descriptions (PFDs) | Application Identifier | - |
|  | AF traffic influence request information | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 of TS 23.501 [2]) | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Information for collection of UE | Correlation ID | Application Identifier or FQDN |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI |  |
|  | EAS Deployment Information (See clause 7.1 of TS 23.548 [74]) | DNN and/or S-NSSAI | Application Identifier and/or Internal Group Identifier |
|  | AM policy influence request information (See clause 4.15.6.9.3) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
| …… | …… | …… | …… |
| …… |  |  |  |

Based on the above analysis, the following ENs is removed:

Editor's note:	It is FFS what information is stored in the UDR, what parameter(s) are used as a key by the SMF to retrieve the information, and when and how the information in UDR is removed.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48.

# * * * * First change * * * *

6.16	Solution 16 (KI#4): Selecting the same EAS/DNAI for collection of UEs

6.16.1	Description

This solution corresponds to KI#4:

-	whether and how to define a collection of UEs forming a dynamic ad-hoc group that should use the same EAS and/or same local part of DN and/or same DNAI and how the collection is identified;

-	whether and what improvements are required for EAS discovery and re-discovery for UEs belonging to a collection of UEs.

6.16.2	Procedures

6.16.2.1 EAS discovery procedure

The following is the procedure for selecting the same EAS for collection of UEs accessing the same application. The procedures defined in figure 4.3.6.2-1 in TS 23.502 [9] and figure 6.2.3.2.2-1 in TS 23.548 [3] are reused.

There could be only one SMF for serving the collection of UEs that connecting to the same EAS/DNAI for accessing the same application, or there could be multiple SMFs for different UEs in the collection. For the latter case, UDR is used for coordination between SMFs to make sure selecting the same EAS/DNAI for UEs, i.e. SMF sends the selected EAS IP/DNAI to UDR or SMF gets the updated EAS IP/DNAI from UDR.

Figure 6.16.2-1: Discovery procedure for selecting the same EAS/DNAI for collection of UEs

1.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [9] is used to request selecting the same EAS or same DNAI for UEs accessing the application as identified in the AF Request.

An eas_correlation indication or dnai_correlation indication is provided for indicating selecting the same EAS or the same DNAI (i.e. selecting EAS corresponding to the same DNAI) for collection of UEs accessing the same application (e.g. FQDN), Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or an UE list or group ID will be provided for defining UE collection accessing the same EAS or the same DNAI. The DNAI could be determined and provided by AF to PCF and then to SMF. When eas_correation is used, the solution assumes that the DNS returns only one IP address for a DNS query. If the DNS is expected to return more than one IP address, then dnai_correlation can be used.

In case of selecting the same DNAI, The DNAI could be determined and provided by AF to PCF and then to SMF.

A Correlation ID is included in AF Request for identifying the UE collection.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [9], PCF determines the UEs influenced by the AF Request, and based on AF request, PCF creates PCC rule with FQDN, Correlation ID,  and eas_correlation indication or dnai_correlation indication to SMF.

2.	The same as steps 1~9 in figure 6.2.3.2.2-1 in TS 23.548 [3].

3.	If FQDN in Neasdf_DNSContext_Notify Request is for the application (e.g. FQDN) indicated in AF request, and if eas_correlation indication is set, SMF determines the UE belongs to collection of UEs accessing the application and determines the UE needs to select the same EAS as UEs in the UE collection; or if dnai_correlation indication is set, SMF determines the UE belongs to collection of UEs accessing the application and determines the UE needs to select EAS corresponding to the same DNAI as UEs in the UE collection.

4.	In case of multiple SMFs, if EAS IP/DNAI has not been determined, or SMF could not make sure the selected EAS IP/DNAI is the one that stored in UDR, then SMF synchronizes with UDR and receives EAS IP or DNAI for the UE collection, as defined in clause 6.16.2.2. UDR maintains EAS IP or DNAI for the UE collection.

5.	Based on steps 10~19 in figure 6.2.3.2.2-1 in TS 23.548 [3]:

For selecting the same EAS case: if the same EAS for the UE collection has not been determined yet, steps 10~15 are used for discovering an EAS, and between step 14 and step 16, SMF could send the selected EAS IP to UDR; When the same EAS for the UE collection has been determined, in step 17, SMF sends DNS message handling rule with IP address for the EAS instructing EASDF to return the IP address for the EAS to UE in step 19, steps 10~15 are skipped;

For selecting EAS corresponding to the same DNAI case: if no DNAI for the UE collection has been determined, in step 10 SMF determines DNAI for the UE collection and selects information to build ECS option or Local DNS server based on the DNAI. When DNAI for the UE collection has been determined, in step 10 SMF determines information to build ECS option or local DNS server related to the DNAI, and sends DNS message handling rule with the information to build ECS option or local DNS server.

6.	If UDR is used for maintaining the same EAS or the same DNAI for collection of UEs, if EAS IP/DNAI has not been determined, or SMF could not make sure the selected EAS IP/DNAI is the one that stored in UDR, then SMF synchronizes EAS IP/DNAI with UDR via NEF, as defined in clause 6.16.2.2.

6.16.2.2	New Data in UDR

A new Data subset Information for collection of UE is defined in UDR, and it contains the following elements:

- Correlation ID

- appID/FQDN

- EAS IP address or DNAI

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
| …… | …… | …… | …… |
| Application data | Packet Flow Descriptions (PFDs) | Application Identifier | - |
|  | AF traffic influence request information | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 of TS 23.501 [2]) | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Information for collection of UE | Correlation ID | Application Identifier or FQDN |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI |  |
|  | EAS Deployment Information (See clause 7.1 of TS 23.548 [74]) | DNN and/or S-NSSAI | Application Identifier and/or Internal Group Identifier |
|  | AM policy influence request information (See clause 4.15.6.9.3) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
| …… | …… | …… | …… |
| …… |  |  |  |

6.16.2.3	Synchronization Procedure for EAS IP/DNAI

Figure 6.16.2-x:  Synchronization Procedure for EAS IP/DNAI

1. When there is no Information for collection of UE in SMF, the SMF queries for the information from UDR via NEF.

2. In case a new EAS/DNAI has been determined, SMF sends the EAS/DNAI to UDR via NEF by invoking Nnef_UECollectionInfo_Create_Request, and UDR will lock the data to prevent the data to be overwritten by other SMF invoking Nnef_UECollectionInfo_Create_Request but providing different EAS/DNAI for the same UE collection. If Nnef_UECollectionInfo_Create_Request fails, then it means the Information for collection of UE has existed, and SMF invokes Nnef_UECollectionInfo_Query for retrieving the Information for collection of UE.

3. The Information for collection of UE could be deleted by any SMF. SMF could determine to delete the Information for collection of UE under the indication from AF.

4. In case the Information for collection of UE is created/deleted, UDR notifies SMFs.

6.16.3	Impacts on services, entities and interfaces

AF:

-	to be updated with eas_correlation indication/ dnai_correlation indication for indicating selecting the same EAS/DNAI for collection of UEs accessing the application.

NEF:

-	Nnef_TrafficInfluence service is impacted to include "eas_correlation indication" or "dnai_correlation indication", list of FQDNs and list of UE identitities.

SMF:

-	to be updated for storing and retrieving the EAS/DNAI from UDR and using the retrieved EAS/DNAI when selecting the same EAS/DNAI for collection of UEs.

UDR:

-	to be updated for maintaining Information for collection of UE for the application.

EASDF:

-	to be updated for create and send DNS response to UE.

PCF:

-	Npcf_SMPolicyControl_UpdateNotify service is to be updated to transmit PCC rule with eas_correlation indication/ dnai_correlation indication, and FQDN(s).

# * * * * End of changes * * * *