3GPP TSG-SA WG2 Meeting #151E (e-meeting)	S2-2204337r05

# May 16 – 20, 2022, Elbonia	(revision of S2-2203591)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0102 | rev | 5 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AMF and AUSF selection for CP authentication and authorisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Interdigital Inc., LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR addresses some issues related with the 5G ProSe relaying CP security solution that were brought up during discussion in TSG SA #95. Following issues were raised during the discussion in SA2. Issue #1: How the Relay UE’s AMF will be ensured that it supports the clause 6.3.3.3.2 of TS 33.503 [3] before even knowing the Remote UEs that it will serve. This can be resolved by configuring the Relay UE to use set of slices that enables to select the AMF supporting CP security procedure. There was a proposal to define new sandardised SST value for roaming but it can be done as a part of roaming agreement if operator wants to support roaming scenario. Issue #2: How it will be ensured that the Relay UE’s AMF will select an AUSF in Remote UEs HPLMN supports the procedure in clause 6.3.3.3.2 of TS 33.503 before even knowing the Remote UEs that it will serve. If operator wants to use control plane security procedure, AUSF shall support control plane security procedure. Issue #3: How the Relay UE will know in advance e.g. when selecting VPLMN that the particular VPLMN has roaming agreements with the potential HPLMNs of the Remote UEs in order to “advertise” the right discovery information There is no guarantee that there is roaming agreement between HPLMN of Remote UE and serving PLMN of Relay UE, the CP security procedure can fail. Issue #4: What subscription information is required for Remote UEs in the UDM. According to current SA3, there is no need for specific Remote UE subscription for CP security procedure as the UDM is expected to be aware of the RSC (see TS 33.503). Issue #5: How the SUPI of the Remote UE is obtained by SMF in PDU session secondary authentication of Remote UE. SA3 is discussing how SUPI of Remote UE is derived by the network so proposes to add reference to TS 33.503. Issue #6: How SMF obtains the Remote UE’s subscription info in PDU session secondary authentication of Remote UE. The SMF can obtain Remote UE's subscription by using Remote UE's SUPI. (see Issue #5) Issue #7: How to support PDU session secondary authentication when Remote UE is roaming. Secondary authentication is performed when CP based security procedure is completed. Success of CP based security procedure means that there is roaming agreement between HPLMN of Remote UE and serving PLMN of Relay UE. So secondary authentication can be supported in roaming case. Issue #8: How to ensure that the SMF supports secondary authentication. Secondary authentication is performed based on request from the DN per DNN/S-NSSAI. So all UEs accessing the DN requires to perform secondary authentication. So Relay UE would also require to perform secondary authentication and the selected SMF should support secondary authentication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - - The Relay UE can be configured to use set of slices that enables to select AMF supporting Control Plane based security procedure. - Secondary authentication for 5G ProSe Layer-2 Remote UE is supported as specified in TS 23.502 - Secondary authentication for 5G ProSe Layer-3 Remote UE is supported as specified in TS 33.503 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete CP relay authentication solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4, 4.3.7.1 (New clause heading), 4.3.7.2 (New), 4.3.10, 4.3.X (New), 5.1.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

##### 5.1.4.3.2	Control Plane based security procedures for 5G ProSe UE-to-Network Relay

Control Plane-based security procedures for 5G ProSe UE-to-Network Relay call flow and procedure is defined in TS 33.503 [29]. It is characterised by the following principles:

-	5G ProSe UE-to-Network Relay's NAS signalling is used for the 5G ProSe Remote UE's authentication and authorisation.

-	The AMF serving the 5G ProSe UE-to-Network Relay selects AUSF as specified in TS 23.501 [4] clause 6.3.4 considering also the 5G ProSe Remote UE's SUCI. If the 5G ProSe Remote UE's HPLMN wants to use control plane security procedure, the 5G ProSe Remote UE's HPLMN AUSF shall support control plane based security procedure as defined in clause 6.3.3.3.2 of TS 33.303 [29].

-	Relay UE and Remote UE to use U2N Relay with C-plane security procedure are provided with S-NSSAI(s) which represents supporting U2N Relay with C-plane security procedure as subscribed S-NSSAI(s). Based on these subscribed S-NSSAI(s) and roaming agreement, VPLMN will provide configured S-NSSAI and mapping table for this S-NSSAI(s).

-	At registration, Relay UE will send registration request with the the S-NSSAI as requested S-NSSAI and S-PLMN will select AMF that support C-plane security procedure.

-	When Remote UE sends request to attach U2N Relay with C-plane security procedure and after receiving a request from Remote UE on this, the AMF serving the 5G ProSe UE-to-Network Relay selects AUSF as specified in TS 23.501 [4] clause 6.3.4 considering also the 5G ProSe Remote UE's SUCI. If the AUSF cannot be selected in roaming case, the procedure fails.

.

-	Secondary authentication for 5G ProSe Layer-2 Remote UE is supported as specified in TS 23.502 [5] clause 4.3.2.3.

-	Secondary authentication for 5G ProSe Layer-3 Remote UE is supported as specified in TS 33.503 [29] clause 6.3.3.3.4.2. In order to support secondary authentication for a 5G ProSe Layer-3 Remote UE,

-	When the 5G ProSe UE-to-Network Layer-3 Relay establishes a PDU Session for a 5G ProSe Layer-3 Remote UE to a DN, the 5G ProSe UE-to-Network Layer-3 Relay performs secondary authentication procedure as specified in TS 23.502 [5].

-	The SMF retrieves the 5G ProSe Layer-3 Remote UE's subscription data by using the 5G ProSe Layer-3 Remote UE's SUPI. The SMF receives the 5G ProSe Layer-3 Remote UE's SUPI as described in TS 33.503 [29] clause 6.3.3.3.4.2.

* * * * End of Changes * * * *