SA WG2 Meeting #151E (e-meeting)	S2-2204005r02

May 16 – 20, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	KI#2, New Sol: Slice based VPLMN Selection Policy

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes a new solution to Key Issue #2.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| Solution #1: Additional S-NSSAI associated with the PDU session | X |  |  |  |  |  |
| Solution #2: Slice Re-mapping Capabilities for Network Slice Service Continuity | X |  |  |  |  |  |
| Solution #3: Support of Network Slice Service continuity using SSC mode 3 | X |  |  |  |  |  |
| Solution #4: PDU Session on compatible network slice | X |  |  |  |  |  |
| Solution #5: PDU session handover to a target CN with an alternative S-NSSAI support | X |  |  |  |  |  |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs |  | X |  |  |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs |  | X |  |  |  |  |
| Solution #8: Gracefully network slice termination |  |  | X |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  |  | X |  |  |  |
| Solution #10: Associating a validity timer with a temporary slice |  |  | X |  |  |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available |  |  | X |  | X |  |
| Solution #12: Solution for Centralized Counting for Multiple Service Areas and 5GS-EPS Interworking |  |  |  | X |  |  |
| Solution #13: Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |  |  |
| Solution #14: Maximum Number Distribution in multiple NSACFs |  |  |  | X |  |  |
| Solution #X: Slice based VPLMN Selection Policy |  | X |  |  |  |  |

* * * * Start of 2nd Change (All new text) * * * *

6.X	Solution #X: Slice based VPLMN Selection Policy

6.X.1	Introduction

This solution addresses key issue #2: Support of providing VPLMN network slice information to a roaming UE. This solution proposes a solution that can control in which condition the UE can trigger VPLMN selection by providing operator controlled policy. This solution is not a stand-alone solution and expected to be used together with other solutions such as solution #6.

NOTE:	This solution can be used together with any other solutions that provides available slice information to the UE.

6.X.2	Functional Description

A UE can determine to trigger VPLMN selection if the UE knows which network slices are supported in a VPLMN and the registered VPLMN does not support slice the UE want to use. The UE determines to use a network slice for an application by using URSP rules.

However, current specification allows the network to provide multiple RSDs for a traffic descriptor, i.e. the network can use multiple slices for an application. Also network can provide "match all" traffic descriptor to the UE. In this case, the UE may use a slice in the RSD of "match all" traffic descriptor if the UE fails to establish a PDU Session using the slice with traffic descriptor for the specific application. From the UE perspective, all slices (e.g. slice in the RSD of matched traffic descriptor with higher priority, slice in the RSD of matched traffic descriptor with lower priority and slice in the RSD of "match all" traffic descriptor) are allowed for the application. So the UE can choose whether to trigger VPLMN selection when a slice in the higher priority URSP rule is in not available in the registered VPLMN. For example, some UEs may not triggers VPLMN selection at all since the slice in the "match all" traffic descriptor is expected to be supported in all VPLMNs. But operator may prefer to use a slice in the higher priority URSP rule if it is available by triggering VPLMN selection.

In addition to above case, there are some cases that a slice the UE wants to use is supported in the VPLMN but temporally rejected by the network due to some reasons. For example, the network may not allow using the slice due to NSAC failure, congestion control, not allowed in the current registration area. In these cases, each operator may have different preference, e.g. as those conditions are temporal restriction some operators may want to keep the UE in the registered PLMN while some operators may want to trigger VPLMN selection.

In order to control the UE behaviour in above cases, this solution proposes that the HPLMN may provide Slice based VPLMN Selection Policy (SVSP) to the UE. The policy indicates in which condition the UE is allowed to trigger VPLMN selection to find a network that supports slices the UE want to use. The SVSP contains following information:

-	Whether the UE can trigger VPLMN selection when the S-NSSAI the UE wants to use is not available

Editor’s note: It is FFS whether the condition to trigger VPLMN selection after receiving slice-aware SoR information is needed to be specified

6.X.3	Procedures

This procedure is used to deliver SVSP to the UE and supports both roaming and non-roaming scenarios.

Figure 6.X.3-1: Overall procedure

In the non-roaming case, the V-PCF is not involved and the role of the H-PCF is performed by the PCF. For the roaming scenarios, the V-PCF interacts with the AMF and the H-PCF interacts with the V-PCF:

1.	The UE performs Registration procedure. The UE indicates that it supports SVSP in the Registration request.

NOTE:	During the Registration procedure, the UE also indicates whether it supports slice aware SoR.

2.	The AMF creates UE policy association with PCF. The AMF provides the UE's capability, i.e. support of SVSP to the PCF.

3.	The AMF provides Registration accept to the UE.

4.	The H-PCF may provide the UE policy container including SVSP. The policy is delivered to the UE by using the UE Configuration Update procedure for transparent UE Policy delivery as described in TS 23.502.

5.	The UE wants to use a slice but the VPLMN does not allows the slice due to some reason. The UE checks the stored SVSP and determine that the SVSP allows triggering VPLMN selection. The UE triggers VPLMN selection by using SoR AF or UDM provided information e.g. as described in solution 6.

6.X.4	Impacts on services, entities and interfaces

UE:

-	The UE supports receiving SVSP from the PCF.

-	The UE determines whether to trigger PLMN selection based on SVSP.

-	The UE indicates SVSP capability during the registration.

AMF:

-	The AMF provides UE's SVSP capability to the PCF during the UE Policy Association Establishment.

PCF:

-	The PCF supports providing SVSP to the UE.

* * * * End of Changes * * * *