3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2204112

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	KDDI, Huawei, Hisilicon

Title:	KI #5, Sol #24: Update to apply identification of importance information of PDU Sets based on H.265/H.266 NAL unit header

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: This contribution proposes update of solution#24 for KI#5 to apply identification of importance information of PDU Sets based on H.265/H.266 NAL unit header.

1. Introduction

“Solution #24: Support differentiated QoS handling for different PDU Sets” was agreed to be added to TR 23.700-60.

This solution contains identification of the importance information of PDU Sets based on H.264 NAL unit header.

H.265/HEVC and H.266/VVC are extensions of the concepts for H.264 and offer better data compression at the same level of video quality, or substantially improved video quality at the same bit rate in comparison to H.264. These codecs are also used in wide range of contents, such as XR, 4K/8K video stream and Cloud gaming.

This paper proposes how to identify the importance information of PDU Sets based on H.265/H.266 NAL unit header for the Solution #24.

According to LS response from SA4.

- GOP structure is used by certain codec only, and indicates mainly the temporal dependency relationship between the PDU set. It is proposed to generalize the GOP to a group of PDU sets. The importance and dependency information of each PDU sets are valid in a certain group of PDU sets.

- GOP structure is quasi-static, the dependency relationship may change with dynamic decision in the application layer, e.g., inserting of a I-frame, change the codec configuration when the network get congested. Therefore, option #3 is further clarified, where the assistance info from AF can be updated based on the application configuration and is used together with the UP information to identify the importance/dependency information of the PDU sets.

This paper also proposes update to make the solution more general and also decouple from specific application implementations.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# *** Start of Changes ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[7]	IEEE SA P1918.1: "Tactile Internet: Application Scenarios, Definitions and Terminology, Architecture, Functions, and Technical Assumptions", https://standards.ieee.org/project/1918_1.html

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[9]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", STD 64, July 2003.

[10]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[11]	draft-ietf-avtext-framemarking: "IETF experimental draft-ietf-avtext-framemarking-13 - Frame Marking RTP Header Extension".

[12]	IETF RFC 6184: "RTP Payload Format for H.264 Video", May 2011.

[13]	IETF RFC 6190: "RTP Payload Format for Scalable Video Coding", May 2011.

[14]	IETF RFC 6437: "IPv6 Flow Label Specification", November 2011.

[15]	IETF RFC 8285: A General Mechanism for RTP Header Extensions.

[16]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[17]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[18]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[19]	ITU-T Recommendation H.264: "Advanced video coding for generic audiovisual services".

[20]	IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control", July 2003.

[21]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC)", March 2016.

[22]	ISO/IEC 14496-12 | 15444-12: "Information technology - Coding of audio-visual objects - Part 12: ISO base media file format".

[23]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[24]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[25]	IETF RFC 1889: "RTP: A Transport Protocol for Real-Time Applications", January 1996.

[26]	3GPP TS 29.998: "Open Services Architecture (OSA) Application Programming Interface (API) part 2".

[W]	ITU-T Recommendation H.265, "High efficiency video coding".

[X]	ITU-T Recommendation H.266, "Versatile video coding".

[Y]	draft-ietf-avtcore-rtp-vvc-14: "RTP Payload Format for Versatile Video Coding (VVC)".

# *** Next Change ***

6.24	Solution #24: Support differentiated QoS Handling for different PDU Sets

6.24.1	Key Issue mapping

This solution corresponds to KI#5 on Differentiated PDU Set Handling.

6.24.2	Description

For interactive media services, e.g. Cloud XR, cloud gaming, real-time video based remote control, PDUs of the media services carry different frame/video slice. For example, in GOP, the first I-frame is considered as the most important frame, other P-frames are encoded based on the I-frame. To ensure the limited radio resources be used to guarantee the successful delivery of the most important packet, 5GS can bring better user experience with same resource. 5GS should be aware of the different importance/dependencies of PDU Sets to achieve such target.

The design of typical media codec and transporting protocol, e.g. RTP, have already considered such transporting requirements and provided importance information via RTP header or NAL (Network Abstract Layer) header (See details in clause 6.24.3.2), so that network node can protect more important NAL units better than they do less important NAL units as described in clause 5.3 of RFC 6184 [12], clause 1.1.4 of RFC 7798 [21] and clause 1.1.4 of draft-ietf-avtcore-rtp-vvc-14 [y]

In this solution, the following aspects are included:

1.	AF initiates AF session with required QoS as described in clause 4.15.6.6 of TS 23.502 [3].

2.	PCF determines updated or new policy information based on AF requirements. The policy information includes detection template for PDU Set importance/dependency and different QoS parameters for different PDU Sets, and optional dependency related instructions. The PCF initiates PDU Session Modification procedure and sends the policy information to SMF.

3.	Based on the policy information, the SMF assigns different QFIs with corresponding QoS parameters for different PDU Sets. The SMF sends N4 rules including PDR and FAR to UPF and N2 SM information including QoS Profiles to RAN.

4.	When the UPF receives DL packets from the application server, the UPF identifies the importance/dependency information of PDU Set of the packet and maps the packet into corresponding QoS Flow. Details on how to identify the importance/dependency information of PDU Set is described in clause 6.24.3.2.

5.	RAN receives DL packets from the UPF. The RAN handles the packets based on N2 SM information from SMF.

The dependency related instructions include:

-	Allow for extended transmission for reference PDU sets, i.e. reference PDU set will not be dropped even its delay exceeds PDB for a limited time duration.

-	Drop the PDU sets if transmission of its reference PDU set is considered as failed.

6.24.3	Procedures

6.24.3.1 Processing of PDU Sets based on different importance/dependency information

Figure 6.24.3.1-1 shows the procedure for supporting differentiated QoS handling for different PDU Sets.

Figure 6.24.3.1-1: Differentiated QoS Handling for different PDU Sets

0.	The UE establishes a PDU Session as defined in clause 4.3.2.2.1 of TS 23.502 [3].

1.	AF sets up an AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

The AF request may include flow description for XR/media services, the dependency information of PDU Sets (See option 3 of clause 6.24.3.2.1), and traffic pattern (See option 4 of clause 6.24.3.2.1).

2.	The PCF derives detection template for PDU Set importance/dependency and different QoS parameters for different PDU Sets and makes policy decision. Optionally, the PCF may derive the dependency related instructions in the PCC rule. The PCF initiates the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3], steps 1b, 3b, 4-8b. The PCF sends policy information to SMF.

3.	The SMF assigns different QoS Flow IDs and QoS parameters for the PDU Sets of the service data flow based on their importance/dependencies. For example, the SMF assigns a QoS Flow ID and corresponding QoS parameters for important PDU Set (e.g. those I-frame or base layer which are references of other frames and layers hence better not be dropped), and another QoS Flow ID and corresponding QoS parameters for other PDU Set (e.g. other frames or additional layers that have dependence on the I-frame or base layer). Different QoS handling on PDU Sets may be performed:

-	For important PDU Set, prioritized admission control/resource scheduling can be performed. These PDU Set should be delivered to the UE no matter whether PDB is exceeded or not.

-	For other droppable PDU Set, perform correlated packets dropping, resource pre-emption.

The SMF sends N4 rules to the UPF to indicate the UPF to identify importance/dependency information of PDU Sets for received packets and map the packets to corresponding QoS Flows. Details on how the UPF identify importance/dependency information of PDU Sets are described in clause 6.24.3.2.

The SMF also sends N2 SM information including QoS profiles to RAN.

4.	The UPF identifies the importance/dependency information of PDU Sets for the DL packets and maps the packets into corresponding QoS Flow based on N4 rules received in step 2(See details in clause 6.24.3.2). UPF sends the DL packets to RAN via different QoS Flows. UPF may also provide dependency information between PDU Sets to RAN via GTP-U header.

5.	The RAN handles the DL packets based on N2 SM information received in step 2 considering the above different QoS handling of the PDU Set. In addition, if dependency information is provided and a PDU Set has been dropped, the other PDU Sets depends on this dropped PDU Sets should also be dropped.

6.24.3.2 PDU Set Importance/dependency information identification on UPF

Real-time Transport Protocol (RTP) are widely used for data with real-time characteristics, such as interactive audio and video.

To support differentiated QoS handling for PDU Sets in 5GS, the UPF should firstly identify the PDU Sets (corresponding to a frame or video slice), then identify the importance/dependency of different PDU Set.

NOTE 1:	Solution 12 for KI#4 described how to identify PDU Set in UPF. This clause will focus on how to further identify the importance/dependency information of different PDU Sets.

6.24.3.2.1	Identification of importance/dependency information of PDU Set as a video frame.

If the PDU Set represents a video frame, the identification of importance/dependency information of the PDU Set can be realized via following options.

Option#1. Identification based on RTP header extension

The RTP header extension defined in draft-ietf-avtext-framemarking [11] is as following:

Figure 6.24.3.2.1-1: Frame Marking RTP header extension for non-scalable streams

In the above RTP header extension (see clause 3.1 of draft-ietf-avtext-framemarking [11]):

-	The I field named as Independent Frame reflects whether a frame can be decoded independent of temporally prior frames or not. 1 means this frame can be decoded independent of temporally prior frames, e.g. intra-frame, IDR frame of H.264/H.265 or VP8/9 keyframe.

-	The D field named as Discardable Frame which reflects whether a frame is droppable or not. 1 means the sender knows this frame can be discarded, and still provide a decodable media stream.

UPF can based the above I field or D field to identify the important PDU Set/frame.

If scalable streams (i.e. different temporary, spatial, quality layers) is used, the RTP header extension is defined as following:

Figure 6.24.3.2.1-2: Frame Marking RTP header extension for scalable streams

The TID field named as Temporal ID identifies the temporal layer/sub-layer encoded, starting with 0 for the base layer, and increasing with higher temporal fidelity. Higher values of TID identify higher temporal layers with higher frame rates as defined in draft-ietf-avtext-framemarking [11].

The LID field named as Layer ID identifies the spatial and quality layer encoded, starting with 0 for the base layer, and increasing with higher fidelity. Higher values of LID identify higher spatial and/or quality layers with higher resolutions and/or bitrates as defined in draft-ietf-avtext-framemarking [11].

UPF can based on the TID and LID field to identify the independent PDU Set/base layer.

Option#2. Identification based on H.264 NALU header

For each RTP packet, a H.264 NAL unit header is included following RTP header. For RTP payload carrying H.264 video slices, according to RFC 6184 [12], the H.264 NAL unit header is as following:

Figure 6.24.3.2.1-3: H.264 NAL unit header format

In the H.264 NAL unit header, the NRI value indicates the relative transport priority, as determined by the encoder. According to clause 5.3 of RFC6184 [12], a value of 00 indicates that the content of the NAL unit is not used to reconstruct reference pictures. Such NAL units can be discarded without risking the integrity of the reference. Values greater than 00 indicate that the decoding of the NAL unit is required to maintain the integrity of the reference pictures. The highest transport priority is 11, followed by 10, and then by 01; finally, 00 is the lowest.

Therefore, UPF can use the NRI value to identify the importance information of NAL unit carrying the PDU Set/slice. In case the frame includes only one slice, the method can be used to detect the importance of the frame.

If scalable streams are used, the RTP NAL unit header is defined in RFC 6190 [13] can be used to identify the importance/dependency information of PDU Set in different media layers.

The scalable streams use NAL unit type 14, 15, and 20 for prefix NAL unit, subset sequence parameter set and coded slice in scalable extension respectively as described in Figure 6.24.3.2.1-3. NAL unit type 14 and 20 indicate the presence of three additional octets in the NAL unit header, as shown below.

Figure 6.24.3.2.1-4: additional octets in the NAL unit header

The PRID field named as priority_id specifies a priority identifier for the NAL unit. A lower value of PRID indicates a higher priority.

The DID field named as dependency_id indicates the inter-layer coding dependency level of a layer representation. At any PDU Set, a layer representation with a given dependency_id may be used for inter-layer prediction for coding of a layer representation with a higher dependency_id, while a layer representation with a given dependency_id shall not be used for inter-layer prediction for coding of a layer representation with a lower dependency_id. The dependency_id gives the details on the dependency relationship between different layers. For example, a layer of DID=n will be decoded based on another layer of DID=n-1.

UPF can determine importance information based on PRID field and/or determine dependency information based on the DID field.

NOTE 1:	Option 2 depends on the readability of NALU header in RTP payload. If RTP payload is encrypted, e.g. SRTP is used, option 2 will not be applicable.

Option#3. Identification based on assistance information from AF

The media application (i.e., AF) may provide/update to 5GS importance/dependency information of PDU sets in the form of group size (i.e., number of PDU set), indication of starting/ending PDU set of a group of PDU sets (e.g., sequence number, transmission time), importance/dependency information of the PDU set with respects to the position/order of PDU set in the group of PDU sets. The importance/dependency information of a PDU set is valid within the group of PDU sets. 5GS derives the importance/dependency relationship between different PDU sets using the assistance information from AF. Option#3 can be used together with Option #1, Option #2, Option #X or Option #Y, e.g., PDU set with ‘I’ field set in the RTP header indicates/resets the start of a group of PDU sets. The PDU sets with the highest importance or the base PDU set is detected using option #1, #2, #X, #Y, the importance/dependency information of other PDU sets is derived further with the assistance information from AF. Option#3 also applies to the cases where the RTP header or NAL header is encrypted.

Option#4. Identification based on traffic pattern

In general, different PDU Sets have different traffic patterns, e.g. size of one PDU Set, number of packets within one PDU Set. For example, the size of I frame is generally 2-3 times larger than P frame. Therefore, the traffic patterns can be used to identify the frame type and hence the importance information of PDU Set.

NOTE 2:	Accuracy of the detection in option 4 depends on the implementation of UPF.

Option#X. Identification based on H.265 NALU header

For each RTP packet, a H.265 NAL unit header is included following RTP header. For RTP payload carrying H.265 video slices, according to RFC 7798 [21], the H.265 NAL unit header is as following:

Figure 6.24.3.2.1-x: H.265 NAL unit header format

The 6-bit Type field in the H.265 NAL unit header can indicate the content of VCL (video coding layer) NAL unit. Table 6.24.3.2.1-x shows the VCL NAL units as defined in H.265 [w]. H.265 [w] includes signaling in the NAL unit header, through NAL unit types, of IRAP (Intra-Random Access Point) pictures beyond IDR (Instantaneous Decoding Refresh) pictures defined in H.264 [19]. Three types of IRAP pictures, namely IDR, CRA (Clean Random Access), and BLA (Broken Link Access) pictures, are supported: IDR pictures are conventionally referred to as closed group-of-pictures (closed-GOP) random access points whereas CRA and BLA pictures are conventionally referred to as open-GOP random access points. The BLA, IDR and CRA pictures do not refer to any pictures other than themselves for inter prediction in its decoding process. The BLA, IDR and CRA pictures are considered as the most important pictures, other pictures are encoded based on these pictures.

Therefore, UPF can use the NAL unit value of BLA, IDR or CRA to identify the importance information of NAL unit carrying the PDU Set/slice.

Table 6.24.3.2.1-x H.265 VCL NAL units

| NAL unit type | Name of NAL unit type | Content of VCL NAL unit |
| --- | --- | --- |
| 0 1 | TRAIL_N TRAIL_R | TP (Trailing Picture) picture |
| 2 3 | TSA_N TSA_R | TSA (Temporal Sub-layer Access) picture |
| 4 5 | STSA_N STSA_R | STSA (Step-wise Temporal Sub-layer Access) picture |
| 6 7 | RADL_N RADL_R | RADL (Random Access Decodable Leading) picture |
| 8 9 | RASL_N RASL_R | RASL (Random Access Skipped Leading) picture |
| 10 12 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved non-IRAP SLNR VCL NAL unit types |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types |
| 16 17 18 | BLA_W_LP BLA_W_RADL BLA_N_LP | BLA (Broken Link Access) picture |
| 19 20 | IDR_W_RADL IDR_N_LP | IDR (Instantaneous Decoding Refresh) picture |
| 21 | CRA_NUT | CRA (Clean Random Access) picture |
| 22 23 | RSV_IRAP_VCL22 RSV_IRAP_VCL23 | Reserved IRAP VCL NAL unit types |
| 24..31 | RSV_VCL24.. RSV_VCL31 | Reserved non-IRAP VCL NAL unit types |

If temporal scalability streams are used, TID (TemporalId) field in the NAL unit header indicates the relative importance of an RTP packet. For example, NAL units belonging to higher temporal sub-layers are not used for the decoding of lower temporal sub-layers. A lower value of TID indicates a higher importance. More-important NAL units may be better protected against transmission losses than less-important NAL units based on temporal scalability.

Therefore, UPF can determine importance information of NAL unit carrying the PDU Set/slice based on TID field.

NOTE y:	Option#X depends on the readability of NALU header in RTP payload. If RTP payload is encrypted, e.g. SRTP is used, option#X will not be applicable.

Option#Y. Identification based on H.266 NALU header

For each RTP packet, a H.266 NAL unit header is included following RTP header. For RTP payload carrying H.266 video slices, according to draft-ietf-avtcore-rtp-vvc-14 [y], the NAL unit header is as following:

Figure 6.24.3.2.1-y: H.266 NAL unit header format

The 5-bit Type field in the H.266 NAL unit header can indicate the content of VCL(video coding layer) NAL unit. Table 6.24.3.2.1-y shows the VCL NAL units as defined in H.266 [x]. The IDR and CRA pictures do not refer to any pictures other than themselves for inter prediction in its decoding process. One of the H.266 features is that a the first coded picture in a bitstream can start with a GDR(Gradual Decoding Refresh) picture, without requiring presence of traditional random access points in the bitstream, such as IDR and CRA. In GOP, the IDR, CRA and GDR pictures are considered as the most important pictures, other NAL units are encoded based on these pictures.

Therefore, UPF can use the NAL unit values of IDR, CRA or GDR to identify the importance information of NAL unit carrying the PDU Set/slice.

Table 6.24.3.2.1-y H.266 VCL NAL units

| NAL unit type | Name of NAL unit type | Content of VCL NAL unit |
| --- | --- | --- |
| 0 | TRAIL_NUT | TP (Trailing Picture) picture |
| 1 | STSA_NUT | STSA (Step-wise Temporal Sub-layer Access) picture |
| 2 | RADL_NUT | RADL (Random Access Decodable Leading) picture |
| 3 | RASL_NUT | RASL (Random Access Skipped Leading) picture |
| 4..6 | RSV_VCL_4.. RSV_VCL_6 | Reserved non-IRAP VCL NAL unit types |
| 7 8 | IDR_W_RADL IDR_N_LP | IDR (Instantaneous Decoding Refresh) picture |
| 9 | CRA_NUT | CRA (Clean Random Access) picture |
| 10 | GDR_NUT | GDR (Gradual Decoding Refresh) picture |
| 11 | RSV_IRAP_11 | Reserved IRAP VCL NAL unit type |

If temporal scalability streams are used, TID (TemporalId) field in the NAL unit header indicates the relative importance of an RTP packet. For example, NAL units belonging to higher temporal sub-layers are not used for the decoding of lower temporal sub-layers. A lower value of TID indicates a higher importance. More-important NAL units may be better protected against transmission losses than less-important NAL units based on temporal scalability.

Therefore, UPF can determine importance information of NAL unit carrying the PDU Set/slice based on TID field.

NOTE x:	Option#Y depends on the readability of NALU header in RTP payload. If RTP payload is encrypted, e.g. SRTP is used, option#Y will not be applicable.

6.24.3.2.2	Identification of importance/dependency information of PDU Set as a video slice

When the PDU Set stands for a video slice, the RTP Payload as defined in RFC 6184 [12], RFC 7798 [21] and draft-ietf-avtcore-rtp-vvc-14 [y] can be used to identify the priorities between different video slices. Details can be found in option#2 , option#X and option#Y of clause 6.24.3.2.1.

6.24.4	Impacts on services, entities and interfaces

AF:

-	Optionally provide importance/dependency assistance information of PDU Sets to 5GC.

PCF:

-	Optionally receive importance/dependency assistance information from AF;

-	Generate different QoS parameters and optionally dependency related instructions) for different PDU Sets.

SMF:

-	Assign different QoS Flows for different PDU Sets.

-	Instruct UPF to identify importance/dependency information of PDU Sets and map PDUs of different PDU Sets into corresponding QoS Flow accordingly.

-	Optionally receive dependency related instructions from PCF and distribute to RAN.

UPF:

-	Identify importance/dependency information of PDU Sets for received DL packets and map the PDUs into corresponding QoS Flow based on SMF instructions.

-	Optionally provide dependency info between different PDU Sets via GTP-U header.

RAN:

-	Optionally receive dependency information of PDU sets and dependency related instructions, identify the dependency information of GTP-U header and perform differentiated PDU Set level packet handling/treatment accordingly.

NOTE:	The details of RAN behaviour is up to RAN WG.

# *** End of Changes ***