3GPP TSG-SA2 Meeting #151-e	S2-2204502r02

# E-Meeting, 16th – 20th  May 2022

Source:	Ericsson

Title:	KI#2: New Solution: AI/ML Translator (AI/ML-T) for 5GC Information Exposure to UE

Spec:	3GPP TR 23.700-80

Agenda item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Document for:	Approval

Abstract:	This contribution proposes solutions for 5GC to prepare and provide information to assist the Application layer AI/ML operations at UE side.

1. Introduction

Key Issue #2: 5GC information exposure to UE is to study whether and how to enhance the 5GC to expose information to the UE to facilitate its Application AI/ML operation (e.g., Model Training, Splitting and inference feedback, etc.). This document covers the following bullets

- 	Whether and for which purpose there is a need for 5GC to expose information or data analytics to the UE, and what are the information or data analytics that can be provided by 5GC to the UE to assist the Application AI/ML operation at the UE side.

-	How the 5GC exposes such information or data analytics to the UE.

To solve the above problems, an AI/ML translator (AIML-T) is proposed. The AIML-T is responsible for translating (mapping) the Application layer AI/ML related requests received from UE(s) to the requests for 5GC (i.e., from AI/ML request to 5GC request), and converting the information or analytics produced at 5GC to AI/ML assistance information for UE(s) (i.e., from 5GC information/analytics to AI/ML assistance information).

The AI/ML translator could be integrated into DCAF or AF. The DCAF or AF is responsible for receiving the requests from UE(s), and for exposing the required assistance information to UE(s) to assist its local Application layer AI/ML operations.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-80 v0.2.0.

* * * First change* * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | X |  |  |  |  | X |  |
| 2 |  | X |  |  |  |  |  |
| 3 |  | X |  |  |  |  |  |
| 4 |  | X |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |
| 6 |  |  | X |  |  |  | X |
| 7 |  |  | X |  |  | X |  |
| 8 |  | X | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |
| 10 |  |  |  |  | X |  |  |
| 11 | X |  |  |  | X |  |  |
| 12 |  |  |  |  | X |  |  |
| 13 |  |  |  |  | X |  |  |
| 14 |  |  |  |  |  | X | X |
| 15 |  |  |  | X |  | X |  |
| 16 | X |  |  |  | X | X | X |
| 17 |  |  |  |  |  |  | X |
| 18 |  |  |  |  |  |  | X |
| 19 |  |  |  |  |  |  | X |
| 20 |  |  |  |  |  |  | X |
| 21 |  |  |  |  |  |  | X |
| 22 |  |  |  |  |  |  | X |
| 23 |  |  |  |  |  |  | X |
| 24 |  |  |  |  |  |  | X |
| 25 |  |  |  |  |  |  | X |
| 26 |  |  |  |  |  | X | X |
| 27 |  |  |  |  |  |  | X |
| x |  | x |  |  |  |  |  |

* * * Second change (all new text) * * * *

## 6.X	Solution #X: AI/ML Translator (AI/ML-T) for 5GC Information Exposure to UE

### 6.X.1	Description

Key Issue #2: 5GC information exposure to UE is to study whether and how to enhance the 5GC to expose information to the UE to facilitate its Application AI/ML operation (e.g., Model Training, Splitting and inference feedback, etc.). This document covers the following bullets.

- 	Whether and for which purpose there is a need for 5GC to expose information or data analytics to the UE, and what are the information or data analytics that can be provided by 5GC to the UE to assist the Application AI/ML operation at the UE side.

-	How the 5GC exposes such information or data analytics to the UE.

To solve the above problems, an AI/ML translator (AIML-T) is proposed. The AIML-T is responsible for translating (mapping) the Application layer AI/ML related requests received from UE(s) to the requests for 5GC (i.e., from AI/ML request to 5GC request), and converting the information or analytics produced at 5GC to AI/ML assistance information for UE(s) (i.e., from 5GC information/analytics to AI/ML assistance information).

The AI/ML translator could be integrated into DCAF or AF. The DCAF or AF is responsible for receiving the requirements from UE(s), and for exposure the required assistance information to the UE(s) to assist its local Application layer AI/ML operations. If the DCAF or AF is in trusted domain, it interacts with the NFs in 5GC directly. If the DCAF or AF is in untrust domain, it interacts with the NFs in 5GC via NEF. Alternatively, the AIML-T can be placed in NEF.

Figure 6.X.1-1: AIML-T integrated into DCAF or AF or NEF

Figure 6.X.1-1 shows the AIML-T integrated into DCAF or AF or NEF. The AI/ML requests from UE(s) could be obtained and processed by the integrated AI/ML-T at the DCAF or AF or NEF.

After the AI/ML request is translated (mapped) to 5GC request at the AIML-T, the DCAF or AF (or via NEF) sends request or subscribes to the other NFs (e.g., NWDAF, etc.) in 5GC for analytics and/or information. The integrated AIML-T at the DCAF or AF (or NEF) processes the received analytics and information, converts them into AI/ML assistance information required by UE(s). After the converting is completed, the DCAF or AF responses or notifies to the UE(s) with its required assistance information. Based on the AI/ML assistance information, the UE(s) make decision and performs corresponding AI/ML operations.

### 6.X.2	Procedures

Figure 6.X.2-1: 5GC information preparation and exposure to assist AI/ML operations at UE side, with AIML-T integrated at DCAF/AF or NEF

Figure 6.X.2-1 illustrates the procedure of interactions among UE(s), DCAF/AF (or via NEF), and the other NFs (e.g., NWDAF(s)) in 5GC for assistance information preparation and exposure.

- UE(s) requests or subscribes to DCAF or AF for AI/ML assistance information to assist local AI/ML decisions/operations at UE side. The following requirements/parameters may contain in the request from UE(s)

- Local type of AI/ML operations at the UE(s), include

- AI/ML operation splitting

- AI/ML model/data distribution and sharing

- Distributed/Federated Learning

- Type of assistance information in need, e.g., statistics, predictions, information of network conditions, etc.

- Time window (for one-time) or time interval (for periodically) requirement to AF to provide the assistance information

- Specific requirement for AI/ML operations, e.g., UL/DL latency, data rate, communication service availability, and data transmission reliability, etc. (could be different for the three types of AI/ML operations)

NOTE: If AIML-T is placed in DCAF, how to configure the AIML-T integrated in DCAF needs to be coordinated with SA WG4.

- [If AIML-T is integrated at DCAF or AF] After receiving the request (for request information or subscription request) from the UE(s), the DCAF/AF translates (maps) the AI/ML request to request for 5GC (e.g., requests to analytics (statistics, predictions) for one or multiple Analytics IDs) according to the requirements/parameters in the request from UE(s).

For example: If receive assist AI/ML splitting operation request, this request may be translated to request to analytics and Analytics IDs “DN Performance”, “Service Experience”.

Editor’s note:	It is FFS how AIML-T generates/determines Analytics Filter Information to request analytics data.

- AF/DCAF requests or subscribes to one or multiple NFs (e.g., NWDAF, etc.) for analytics and/or data. The NFs perform corresponding operations to produce analytics or collect data, and responses/notifies to the DCAF/AF with the analytics and/or data.

If AIML-T is integrated at NEF, the AF/DCAF forwards the AI/ML request from UE to NEF, where the request will be translated (mapped) to request for 5GC at NEF. Then NEF will requests or subscribes to one or multiple NFs (e.g., NWDAF, etc.) for analytics and/or data.

- [If AIML-T is integrated at AF] The AF translates (converts) the received analytics/information to AI/ML assistance information according to the type of assistance information required. One AI/ML assistance information may be produced based on one or multiple analytics/information.

Alternatively, if AIML-T is integrated at NEF, the received analytics/information will be translated (converted) to AI/ML assistance information at NEF and NEF sends the information back to AF/DCAF

The AI/ML assistance information to UE(s) might be

- Predictions on network conditions (e.g., latency, bitrate, communication service availability, reliability, etc.) and corresponding time window

- Predictions on network condition changes and corresponding time

- Predictions on the need of downloading ML model or certain amount of data

- Information of resource utilization (e.g., monitoring information or analytics information), etc.

- The AF responses or notifies to the UE(s) with the required AI/ML assistance information.

Editor’s note:	Whether and how the UE can use 5GC information for AI/ML operations is FFS and needs to be describe with valid justification before solution can be adopted, considering also that the same information will be used by the AI/ML application server as well.

### 6.X.3	Impacts on Existing Nodes and Functionality

AF or DCAF:

- If AIML-T is integrated at DCAF or AF. The AIML-T integrated at the DCAF/AF translates (maps) the Application layer AI/ML related requests received from UE(s) to the requests for 5GC (i.e., from AI/ML request to 5GC request), and converts the information or analytics produced at 5GC to AI/ML assistance information for UE(s) (i.e., from 5GC information/analytics to AI/ML assistance information).

- Receive AI/ML requests from UE(s).

- Expose AI/ML assistance information to UE(s).

NEF:

- If AIML-T is integrated at NEF. The AIML-T integrated at NEF translates (maps) the Application layer AI/ML related requests received from UE(s) to the requests for 5GC (i.e., from AI/ML request to 5GC request), and converts the information or analytics produced at 5GC to AI/ML assistance information for UE(s) (i.e., from 5GC information/analytics to AI/ML assistance information).

- (If AIML-T is placed in NEF) Receive AI/ML requests from DCAF or AF.

- (If AIML-T is placed in NEF) Expose AI/ML assistance information to DCAF or AF.

UE:

- UE should be able to communicate with AIML-T.

* * * End of changes * * * *