SA WG2 Meeting #151e	S2-2203680r01

# May 16th – 20th, 2022; Elbonia                  	       (revision of S2-22xxxxx)

Source: 	vivo

Title: 	FS_eNPN_Ph2 Update to Solution #14

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This contribution proposes to update Solution #14.

# 1	Discussion

This contribution proposes to update Solution #14 by:

1) limiting the scenario to when the hosing network is an SNPN.

2) providing more details to steps of how the hosting network sends the Hosting Network Selection and Access Information to the localized service provider.

3) providing more details to how the UE selects the hosting network, automatically and manually.

4) adding another mechanism for UE receiving hosting network selection and access information from local SP “using SoR procedure”, and revise the existing one to “using UP provisioning mechanism”.

# 2 Proposal

It is proposed to update TR 23.700-08 as follows.

# 6.14	Solution #14: Solution for hosting network selection

### 6.14.1	Introduction

This solution solves several requirements listed in Key Issue#4.

This solution is used in the scenario when the hosting network is an SNPN. In this solution, two alternatives are provided for a UE to receive hosting network selection and access information from Localized service provider, after the UE establishes business relationship with the Localized service provider.

1) UE receives hosting network selection and access information from Localized service provider using UP provisioning mechanism;

2) UE receives hosting network selection and access information from local SP using SoR procedure.

In both alternatives, the Localized service provider obtains Hosting Network Selection and Access Information from the Hosting network.

### 6.14.2	Functional Description

In this solution, the hosting network sends Hosting Network Selection and Access Information to the local service provider, who provides service to UEs accessing the hosting network.

The Hosting Network Selection and Access Information consists the following information:

-	Hosting Network Identifier, e.g. SNPN ID in case of the hosting network is an SNPN.

-	The time condition information when the hosting network provides access service.

-	The location condition information where the hosting network provides access service.

-	Optionally, credential used for the UE to access the hosting network. Credential may be only provided in case hosting network is an SNPN.

The UE receives the Hosting Network Selection and Access Information from the localized service provider. The UE selects a hosting network when the time and location condition information is satisfied.

### 6.14.3	Procedures

Figure 6.14.3-1: UE receives hosting network selection and access information from Localized service provider using UP provisioning mechanism

1.	The hosting network server sends Hosting Network Selection and Access Information to the NEF.

The hosting network (HN) server stores the Hosting Network Selection and Access Information and send it to the local service providers who provide services to the UEs accessing the hosting network.

The HN server may be the UDR in the hosting network. The UDR sends Nudr_DM_Notify message to the NEF of hosting network, including LSP ID and the Hosting Network Selection and Access Information.

The HN server may include the IDs of all the LSPs which provide localized services via the hosting network.

2.	The NEF sends the Hosting Network Selection and Access Information to the Local Service Provider (LSP) App.

The NEF sends Nnef_EventExposure_Notify message to the LSP App, including the Hosting Network Selection and Access Information.

3.	The UE establishes connection with the local service provider App.

The UE may establish a PDU Session via its home network and establishes connection via the PDU Session.

NOTE:	How the UE knows the address of the local service provider App is out of 3GPP scope, for example, from the ticket the user bought for a football match.

4.	The UE receives the Hosting Network Selection and Access Information from the local service provider App.

5.	For automatically network selection, when the corresponding time and location condition information is satisfied, the UE activates SNPN access mode. The UE selects a hosting network based on the Hosting Network Identifier in the Hosting Network Selection and Access Information when the corresponding time and location condition information is satisfied.

For manual network selection, when the corresponding time and location condition information is satisfied, the UE provides to the user the list of hosting networks (each is identified by a PLMN ID and NID) and related human-readable names (if available) of the available hosting networks. The user selects manually one hosting network from the list.

Figure 6.14.3-2: UE receives hosting network selection and access information from local SP using SoR procedure

1.	The hosting network server sends Hosting Network Selection and Access Information to the NEF.

The hosting network (HN) server stores the Hosting Network Selection and Access Information and send it to the local service providers who provide services to the UEs accessing the hosting network.

The HN server may be the UDR in the hosting network. The UDR sends Nudr_DM_Notify message to the NEF of hosting network, including LSP ID and the Hosting Network Selection and Access Information.

The HN server may include the IDs of all the LSPs which provide localized services via the hosting network.

2.	The NEF sends the Hosting Network Selection and Access Information to the local service provider App.

The NEF sends Nnef_EventExposure_Notify message to the LSP App, including the Hosting Network Selection and Access Information.

3.	The LSP App sends Nnef_ParameterProvision_Create message to the home network of the UE including the Hosting Network Selection and Access Information.

NOTE:	How the LSP App can obtain the home network of the UE is out of 3GPP scope, for example, from the UE’s mobile phone number.

4.	The NEF of the home network of the UE sends Nudm_ParameterProvision_Create to the UDM of the home network of the UE, including the Hosting Network Selection and Access Information.

5.	The UDM notifies the AMF of the UE of the Hosting Network Selection and Access Information via Nudm_SDM_Notification Notify message.

6.	The AMF sends a DL NAS TRANSPORT message to the UE, including the Hosting Network Selection and Access Information.

NOTE:	Step 5 and 6 can reuse the corresponding steps in SoR mechanism as described in TS 23.122 [6] Annex C.

Editor's note:	It is FFS whether SoR requires SoR AF or SoR information can originate also from other AFs as per the procedure.

7.	For automatically network selection, when the corresponding time and location condition information is satisfied, the UE activates SNPN access mode. The UE selects a hosting network based on the Hosting Network Identifier in the Hosting Network Selection and Access Information when the corresponding time and location condition information is satisfied.

For manual network selection, when the corresponding time and location condition information is satisfied, the UE provides to the user the list of hosting networks (each is identified by a PLMN ID and NID) and related human-readable names (if available) of the available hosting networks. The user selects manually one hosting network from the list.

### 6.14.4	Impacts on services, entities, and interfaces

UE impact:

-	Ability to receive Hosting Network Selection and Access Information from the local service provider App or via SoR procedure.

-	Ability to perform hosting network selection based on the received Hosting Network Selection and Access Information.

Hosting Network impact:

-	Ability to send Hosting Network Selection and Access Information to the local service provider.

Home Network impact:

-	NEF: Ability to receive Hosting Network Selection and Access Information from the local service provider.

-	UDM: Ability to receive Hosting Network Selection and Access Information from the NEF and sends Hosting Network Selection and Access Information to the UE using SoR procedure.

************************** END OF CHANGE ***********************