# 3GPP TSG-SA WG2 Meeting #151E	S2-2203714r02

Elbonia, 16 - 20 May 2022	(Revision of S2-xxxxxxx)

Title:	[Draft] LS On FS_REDCAP_Ph2 option feasibility

Response to:	-

Release:	Rel-18

# Work Item:	     FS_REDCAP_Ph2

Source:	Ericsson [to be SA2]

To:	RAN3, RAN2

cc:	CT4, CT1

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 Rel-18 study FS_REDCAP_Ph2 is ongoing and approaching the conclusion phase. The study is focused on one key issue which enabling long eDRX>10.24s cycle support for UE in RRC_INACTIVE state. See more details in latest TR 23.700-68 (link).

SA2 discussed mainly two different types of solutions:

A) 	NG-RAN provides UE unreachability information (e.g. the eDRX information) in RRC_INACTIVE state to CN when UE enters RRC_INACTIVE state with long eDRX and CN handles the MT data/signalling while the UE is deemed unreachable. CN triggers MT data/signalling and RAN paging when the UE becomes connected again or when the UE is considered reachable. See solution#6 in TR 23.700-68 for reference.

B) 	NG-RAN handles MT data/signalling while the UE is RRC_Inactive. In case UE moves out of the RNA area during the unreachable time period and performs resume outside the RNA, the UE context and data retrieval between RAN nodes and data forwarding are supported via CN when there is no Xn interface. See solution#2 in TR 23.700-68 for reference.

Considering different deployment and capabilities in different network entities, SA2 considers it may be beneficial to conclude a solution with parts from both types of solutions.

SA2 would like to ask RAN2 and RAN3 on their views on the following aspects:

- gNB providing UE eDRX in RRC_INACTIVE state information to CN for MT data/signalling handling when UE is not reachable in RRC_INACTIVE state.

- Including the UE context retrieval with data forwarding handling via CN in Rel-18.

2. Actions:

To RAN3, RAN2

ACTION: 	SA2 kindly asks RAN2 and RAN3 take the above information into consideration and provide their views on the two aspects.

3. Date of Next RAN3 Meetings:

SA2#152e                   August 22-26, 2022                                    Electronic