SA WG2 Meeting #S2-151e(e-meeting)	S2-2203716r01

16 - 20 May 2022, Electronic Meeting	(revision of S2-220xxxx)

Source:	Ericsson, Apple, Huawei

Title:	KI#4, Sol#6: Update to clarify the procedures

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR

Abstract of the contribution: Introduce updates to solution#6.

1. Introduction

This paper proposes updates for solution #6 to have the proper flow sequence when mobile base station relay moves from HPLMN to VPLMN using different procedures.

2. Proposal

This paper proposes the following updates to TR 23.700-68 clause 6.6.

* Start of change *

6.6	Solution #6: Enabling roaming of mobile base station relays

6.6.1	General

This solution addresses Key Issue#4 as defined in clause 5.4.

6.6.2	Functional description

The high-level points of the proposed solution are:

1.	The mobile base station relay (i.e. IAB-node) may be pre-configured with one or more of the following:

a)	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node;

NOTE 1:	The geographic area format used for V2X policy in TS 24.588 [8] can be re-used. A geographic area is encoded as a list of coordinates, each corresponding to a summit of a polygon.

b)	a list of PDU session parameters, e.g. specific DNN and S-NSSAI, and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session or using backhaul IP layer by implementation toward a regional OAM server for IAB node configuration;

2a.	When the mobile base station relay moves out of coverage of the HPLMN and into VPLMN using IAB-node integration procedure:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB nodeas described in step 1a), the relay attempts to select the PLMNs in the list, in priority order;

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [12];

2b.	The alternative is that the mobile base station relay moves out of coverage of the HPLMN and into VPLMN using inter-IAB-donor gNB mobility procedure:

-	the inter-IAB-donor gNB mobility procedure is the outcome of KI#3;

NOTE 2:	The inter-IAB-donor gNB mobility procedure depends on RAN Rel-18 work.

NOTE 3:	The PDU sessions of UE camped on the mobile base station relay may be transferred from the HPLMN to the VPLMN. This assumes that inter-PLMN handover between the HPLMN and the VPLMN is supported.

3.	Upon selecting a VPLMN as described in step 2, the mobile base station relay initiates a registration procedure and indicates its intention to operate as an IAB node in RRC connection to the NG-RAN. The intention to operate as an IAB node is included in the N2 parameters to the AMF by the NG-RAN

4.	The AMF in the VPLMN either:

a)	accepts the Registration Request of the mobile base station relay; or

b)	rejects the Registration Request of the mobile base station relay with a 5GMM cause value indicating that the mobile base station relay is not allowed to operate as a relay in the VPLMN;

NOTE 4:	In case of 2b, the acceptance of the mobile base station relay is partially controlled during the inter-IAB-donor gNB procedure.

5.	If the Registration Request of the mobile base station relay is accepted

, in case of 2a, the mobile base station relay

performs an IAB node integration procedure with the VPLMN as specified in TS 38.401 [6] clause 8.12.1. Using the pre-configured information as described in step 1b) and solution provided for KI#1;

NOTE 4:	When performing IAB-node integration procedure, the PDU sessions of any UE camped on the mobile base station relay will be interrupted.

6.6.3	Signalling flows

6.6.3.1	Signalling flow for roaming of mobile base station relay using IAB node integration procedure

Figure 6.6.3.1-1: Signalling flow for roaming of mobile base station relay operating as an IAB node using IAB node integration procedure

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node;

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session or using backhaul IP layer by implementation toward a regional OAM server for relay configuration.

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN as follows:

-	if the mobile base station relay is pre-configured with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node, the IAB-UE attempts to select the PLMNs in the list, in priority order;

-	otherwise, the mobile base station relay performs PLMN selection as specified in TS 23.122 [12].

2.	The mobile base station relay (i.e. IAB-UE) provides an IAB-indication to the NG-RAN when the RRC connection is established as defined in clause 5.35.2 of the TS 23.501[2].  The mobile base station relay sends a Registration Request message to the NG-RAN.

3. RAN may use this IAB-indication for AMF selection that supports mobile base station relay operation and forwards  IAB-indication in N2 message with Registration Request.

4.	The VPLMN AMF performs UE authentication and registration and further checks with H-UDM . The UDM provides the IAB-Operation allowed indication to AMF as part of the Access and Mobility Subscription data as defined in clause 4.2.2.2.2 of the TS 23.502[5].

5.	If the AMF determines that the mobile base station relay is authorized to operate as an IAB node, the AMF sends IAB authorized indication to NG-RAN and a Registration Accept message to the mobile base station relay.

6.	The mobile base station relay performs an IAB node integration procedure with the VPLMN as specified in clause 8.12.1 of TS 38.401 [6]. Using the pre-configured information as described in step 0.

6.6.3.2	Signalling flow for roaming of mobile base station relay  using inter-IAB-donor gNB mobility procedure

Figure 6.6.3.2-1: Signalling flow for roaming of mobile base station relay operating as an IAB node using IAB inter-donor full migration procedure

0.	The mobile base station relay may be pre-configured with:

-	a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node;

-	a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session or using backhaul IP layer by implementation toward a regional OAM server for relay configuration.

1.	The mobile base station relay moves out of coverage of its HPLMN and selects a VPLMN .

2.	The mobile base station relay performs an inter-IAB-donor gNB mobility procedure as described in step 2b in clause 6.6.2.

NOTE 1:	The inter-IAB-donor gNB mobility procedure for mobile base station relay, including the reconfiguration during mobility, depends on RAN Rel-18 work.

The mobile base station relay (i.e. IAB-UE) provides an IAB-indication to the NG-RAN when the RRC connection is established as defined in clause 5.35.2 of the TS 23.501[2].

3.	The mobile base station relay sends a Registration Request message to the NG-RAN.

-  NG-RAN may use this IAB-indication for AMF selection that supports mobile base station relay operation and forwards IAB-indication in N2 message with Registration Request.

4.	The VPLMN AMF performs UE registration and further checks with H-UDM whether the mobile base station relay is authorized to operate as an IAB node. The UDM provides the IAB-Operation allowed indication to AMF as part of the Access and Mobility Subscription data as defined in clause 4.2.2.2.2 of the TS 23.502[5]

5.	If the AMF determines that the mobile base station relay is authorized to operate as an IAB node , the AMF sends a Registration Accept message to the mobile base station relay.

6.6.4	Impacts on services, entities and interfaces

The following impacts have been identified:

-	UE:

-	No impact.

-	Mobile base station relay:

-	Support pre-configuration with a prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node.

Editor's note:	It needs to be evaluated if existing UE PLMN selection operation defined in TS 23.122 [12] is sufficient.

-	Support pre-configuration with a list of PDU session parameters and OAM server FQDN, each set associated with a PLMN ID and/or geographic area, to be used to establish a PDU session toward a regional OAM server for relay configuration.

-	Support for PLMN selection based on the prioritized list of PLMNs and/or geographic areas in which the mobile base station relay is allowed to operate as an IAB node.

-	Support inter-IAB-donor gNB mobility procedure.

Editor's note:	The support of inter-IAB-donor gNB mobility procedure needs to be coordinated with RAN WGs.

Editor's Note: the support of mobile base station relay needs to be coordinated with RAN WGs.

-	AMF:

-	Support for determining whether a mobile base station relay attempting to register is allowed to operate as an IAB node and accepting/rejecting the Registration Request accordingly.

* End of changes *