3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2203783

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2, Sol#8: Update to remove the FFS

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: This contribution to update the Solution #8 on DFI to remove some FFS.

# 1. Introduction/Discussion

Editor's note:	Basic service types may also be recognized via application detection at the UPF. Further explanation of use cases where that functionality is not sufficient is desirable.

>> Can be removed. Traditional application detection at the UPF is depend on DPI for the unencrypted packets or the PFD mechanism for the encrypted packets. In 5G and future network, most of the packets will be encrypted based on e.g. HTTPS, so DPI is invalidated. Furthermore, the PFD mechanism rely on the PFD information provisioning by the 3rd AF, which is out of control of the 5G network. So, the solution is needed.

Editor's note:	whether there is a need for a Service Type or the Application ID can cover is FFS.

>> Can be removed. During the SA2#150E meeting, it was commented that “The existing PFD management provides PFDs for an Application ID, then the Application identifier: An identifier referring to a specific application detection filter. According to 23.503 definitions, the Application detection filter: A logic used to detect packets generated by an application based on extended inspection of these packets, e.g. header and/or payload information, as well as dynamics of packet flows. This definition does not limit the detection to a particular application but it can be a service type or even broader.” This solution proposes to define a new terminology i.e. Service Type including Browsing, Video Streaming, Audio Streaming, File Sharing, Multimedia, Interactive Messages, Video call over IP, Voice call over IP and so on. Alternatively, the definition of the Application ID can be enhanced to support these service types.

Editor's note:	The newly detected Service Type needs to be known to the different NFs, such as SMF, UPF, PCF and how this is done is FFS. How to use the Service Type for QoS control is FFS.

>> Can be removed. For any UE, based on operator policy, PCF may trigger the Service Type detection in a specific area of interest and requests the NEF(PFDF) to push the new PFD information (including the Service Type related information) to the UPFs located in the AoI. Then the UPF may report the service type detection result to PCF for QoS control. For a single UE, PCF may get the service type detection requirement per UE/per AF session from AF, then SMF may pull the PFD information from the NEF(PFDF) to the UPF.

Editor's note:	It is ffs how the UPF can determine packet delay

>> Can be removed. The current QoS Monitoring Event (including end to end delay i.e. Uu+N3/N9 delay for specific QoS flow or for specific PDU session) by UPF as defined in clause 5.2.26.2 of TS 23.502 can be re-used.

Editor's note:	Extensive reporting of all traffic flows may conflicts with requirement to avoid extra UPF load in the key issue

>> Can be removed. Add a NOTE to clarify that how to decrease the UPF load is dependent on the conclusion of FS_UPEAS.

Editor's note:	Whether others information should be added in the Table 6.8.1-2/Table 6.8.1-3is FFS.

>> Can be removed. Add the traffic characteristics into the Table 6.8.1-2/Table 6.8.1-3 to help the Service Type Detection i.e. the UPF may determine a Service Type for a real-time traffic information if the real-time traffic information and the traffic characteristics for the Service Type is matched.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-81.

# * * * * First change * * * *

## 6.8	Solution #8: NWDAF assisted service type detection

### 6.8.1	Description

This solution is for Key Issue#2: NWDAF-assisted application detection.

Traditionally, the application traffic detection in user plane is depend on DPI (deep packet inspection) function installed in UPF. However, the DPI in only suitable for plaintext traffic (i.e. original readable text in the packet) but not for the encrypted traffic.

To help the encrypted application traffic detection, as defined in clause 6.1.2.3 of TS 23.503 [4], ASP may provide the PFD (packet flow description) information including Application ID, 3-tuple(s) etc. to the UPF via NEF (PFDF)/SMF. Then the UPF can determine the Application ID for the encrypted traffic based on the PFD information and the 5-tuple of the encrypted traffic. However, it is the ASP who determines if the PFD information is provided or not, which may be out of control for the Operator.

If no any PFD information from the ASP, it is possible for NWDAF to derive Service Type analytics (including e.g. Service Type and its corresponding IP 3-tuple and/or Traffic characteristics) to help the traffic detection in UPF.

Editor's note:	Basic service types may also be recognized via application detection at the UPF. Further explanation of use cases where that functionality is not sufficient is desirable.

This solution proposes to define a new terminology i.e. Service Type including Browsing, Video Streaming, Audio Streaming, File Sharing, Multimedia, Interactive Messages, Video call over IP, Voice call over IP and so on. Alternatively, the definition of the current Application ID in clause 3.1 of TS 23.503 [4] can be enhanced to support these service types.

Editor's note:	whether there is a need for a Service Type or the Application ID can cover is FFS.

As a potential consumer NF, the PFDF may subscribe or request the Service Type analytics and locally store the analytics as new PFD information, and then the PFDF pushes the new PFD information to SMF and UPF to identify the Service Type of the traffic flows based on the new PFD information and the 3-tuple of traffic in the UPF. For example:

-	For any UE, based on operator policy, PCF may trigger the Service Type detection in a specific area of interest and requests the NEF(PFDF) to push the new PFD information (including the Service Type related information) to the UPFs located in the AoI. Then the UPF may report the service type detection result for this AoI to PCF for QoS control.

-	For a single UE, PCF may get the service type detection requirement per UE/per AF session from AF, then SMF may pull the PFD information from the NEF(PFDF) to the UPF. Then the UPF may report the service type detection result for this UE to PCF for QoS control.

Editor's note:	How the new PDR is activated and the reporting from UDR to PCF is done is FFS.

Editor´s Note: How the Service Type detection and Service Type QoS control coexist with the existing SDF detection and QoS control define in 23.503 is FFS.

The input data for the Service Type analytics is shown in Table 6.8.1-1.

Table 6.8.1-1: Service flow level Data from UPF and/or AF related to a particular Service Type

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | UPF, AF | A time stamp associated with the collected information. |
| 3-tuple | UPF, AF | protocol, server side IP address and port number for a service flow |
| Service Type | AF, | Service Type of the service data flow |
| Duration | UPF, AF | Duration of the service flow level data |
| UL/DL Bit Rate | UPF, AF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| UL/DL Packet Delay | UPF, AF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Number of UL/DL packet transmission | UPF, AF | The observed number of UL/DL packet transmission. |
| Size of UL/DL packet retransmission | UPF, AF | The observed size of UL/DL packet transmission. |

Editor´s Note:  Whether the Service Type or the AF-Application ID is reported by the AF and why a new identifier is needed is FFS:

NOTE:	How to collect the QoS flow level Network Data from UPF is depend on the conclusion of FS_UPEAS.

To collect the UL/DL Packet Delay, the current QoS Monitoring Event (including end to end delay i.e. Uu+N3/N9 delay for specific QoS flow or for specific PDU session) by UPF as defined in clause 5.2.26.2 of TS 23.502 [3] can be re-used.

NOTE X:	Packet delay detection is only supported for URLLC.

Editor´s Note: Extensive reporting of all traffic flows may conflicts with requirement to avoid extra UPF load in the key issue. How to avoid the degradation of UPF Load because of the extensive reporting of all traffic flows should be aligned with the conclusion of KI#2: Support UPF expose information to other NFs in FS_UPEAS.

With the input data, NWDAF uses the ML algorithm (e.g. K-Means) to cluster the data set to derive one or more 3-tuple(s) for a Service Type. The statistics and prediction types of Service Type analytics are shown in Table 6.8.1-2 and Table 6.8.1-3, respectively.

Table 6.8.1-2: Service Type statistics

| Information | Description |
| --- | --- |
| List of Service Types (1..max) |  |
| > Service Type | Service Type (including i.e. Browsing, Video Streaming, Audio Streaming, File Sharing, Multimedia, Interactive Messages, Video call over IP, Voice call over IP) of the traffic in UPF. |
| >> IP 3-tuple | One or more IP 3-tuples for the Service Type |
| >> Traffic characteristics (NOTE) | Traffic characteristics (including UL/DL Bit Rate, UL/DL Packet Delay, Number of UL/DL packet transmission, Size of UL/DL packet retransmission, etc.) of the Service Type |
| NOTE:Traffic characteristics helps the Service Type detection i.e. the UPF may determine a Service Type for a real-time traffic information if the real-time traffic information and the traffic characteristics for the Service Type is matched. |  |

Table 6.8.1-3: Service Type predictions

| Information | Description |
| --- | --- |
| List of Service Types (1..max) |  |
| > Service Type | Service Type of the traffic in UPF. |
| >> IP 3-tuple | One or more IP 3-tuples for the Service Type |
| >> Traffic characteristics | Traffic characteristics (including UL/DL Bit Rate, UL/DL Packet Delay, Number of UL/DL packet transmission, Size of UL/DL packet retransmission, etc.) of the Service Type |
| > Confidence | Confidence of this prediction |
| NOTE: Traffic characteristics helps the Service Type detection i.e. the UPF may determine a Service Type for a real-time traffic information if the real-time traffic information and the traffic characteristics for the Service Type is matched. |  |

### 6.8.2	Procedures

Figure 6.8.2-1: Procedure for Service Type analytics provided to an NF

1.	The NF sends Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID="Service Type", Target of Analytics Reporting, Analytics Filter Information, Analytics Reporting Information = ("Reporting Thresholds" and Analytics target Period(s))) to the NWDAF.

2.	The NWDAF collects service flow level data from UPF as listed in Table 6.8.1-1.

NOTE:	How to collect the service flow level data from UPF is depend on the conclusion of FS_UPEAS.

3.	The NWDAF collects service flow level data from AF as listed in Table 6.8.1-1.

4.	The NWDAF derives the requested analytics.

5.	The NWDAF sends Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify (Service Type analytics).

6-7.	A change of user plane traffic information which is reported by UPF or AF, and is notified to NWDAF.

8.	The NWDAF derives new analytics taking into account the most recent data collected.

9.	The NWDAF provides a notification for the new analytics using Nnwdaf_AnalyticsSubscription_Notify service operation.

### 6.8.3	Impacts on Existing Nodes and Functionality

NWDAF:

-	Support for a new Analytics ID i.e. Service Type analytics.

-	Support subscribing for service flow level data from UPF and/or from AF and providing Service Type analytics to NF.

UPF/AF:

-	Report service data flow level data to NWDAF.

# * * * * End of changes * * * *