SA WG2 Meeting #S2-151E	S2-2203828r04

16 – 20 May 2022, Electronic, Elbonia	(revision of S2-2202832)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3459 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on 5G access stratum distribution in mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | AF may request 5G access stratum as a time synchronization distribution method. 5G-AN provides the 5GS time to the UE via 3GPP radio access; The request to control the 5G access stratum time distribution is described in clause 4.15.9.4 of TS 23.502. During mobility, the target need to know the 5G access stratum time distribution for the UE. In the 150E meeting, SA2 send the LS (S2-2203621) to RAN3 to confirm the RAN imapct on this. This CR modify the spec according to RAN3 response |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AMF inform the target RAN the 5G access stratum time distribution for the UE in the mobility procedure. Adding the “5G access stratum time distribution” to UE AMF context. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to support 5G access stratum time distribution inthe mobility is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.4, 5.2.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.9.4	Procedures for management of 5G access stratum time distribution

The AF can use the procedure to activate, update or delete the 5G access stratum time distribution for one UE, group of UEs or any UE using the DNN and S-NSSAI.

The AF may query the status of the 5G access stratum time distribution using Nnef_ASTIGet service operation. The Nnef_ASTICreate and Nnef_ASTIUpdate request may contain the parameters as described in Table 4.15.9.4-1.

Table 4.15.9.4-1: Description of 5G access stratum time distribution parameters

| Parameter | Description |
| --- | --- |
| 5G access stratum time distribution indication (enable, disable) | Indicates that the access stratum time distribution via Uu reference point should be activated or deactivated for the associated UE identities. |
| Time synchronization error budget | Indicates the time synchronization error budget for the time synchronization service (as described in clause 5.27.1.9 of TS 23.501 [2]). [optional] |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service is active. [optional] |

Figure 4.15.9.4-1: Management of 5G access stratum time information

1.	AM Policy Association establishment as described in clause 4.16.1.

2.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	To create a new request, the AF provides access stratum time distribution parameters to the NEF using the Nnef_ASTICreate service operation (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier. The NEF maps the External Group Identifier to an Internal Group Identifier and any GPSI to a SUPI.

-	To update or remove an existing request, the AF invokes an Nnef_ASTIUpdate or Nnef_ASTIDelete service operation providing the corresponding time synchronization configuration id.

-	To query the status of the access stratum time distribution, the AF invokes Nnef_ASTIGet service operation providing the target (List of UE identities (SUPI or GPSI)).

The AF that is part of operator's trust domain may invoke the services directly with the TSCTSF.

NOTE 1:	Steps 1 and 2 can occur in any order.

3.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_ASTICreate/Update/Delete/Get service operation with the TSCTSF discovered and selected as described in clause 6.3.24 of TS 23.501 [2].

(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The TSCTSF determines whether the targeted UE is part of a PTP instance in 5GS, if so the TSCTSF rejects the request (steps 4-7 .are skipped).

(When the procedure is triggered by PTP instance activation, modification, or deactivation in the TSCTSF):

-	If time synchronization error budget is provided by the AF, the TSCTSF may use the PTP port state of each DS-TT to determine an Uu time synchronization error budget for corresponding SUPIs that are part of the PTP instance.

-	If time synchronization error budget is provided by the AF, the TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2].

4.	The TSCTSF searches the PCF for the UE using Nbsf_Management_Subscribe with SUPI as an input parameter, indicating that it is searching for the PCF that handles the AM Policy Association of the UE.

5.	The BSF provides to the TSCTSF the identity of the PCF for the UE for the requested SUPI via an Nbsf_Management_Notify operation. If matching entries already existed in the BSF when step 6 is performed, this shall be immediately reported to the TSCTSF.

6.	The TSCTSF sends to the PCF for the UE its request for the AM policy of the UE (identified by SUPI) using Npcf_AMPolicyAuthorization request, containing the 5G access stratum time distribution indication (enable, disable) and optionally the calculated Uu time synchronization error budget.

7.	If the PCF receives multiple time synchronization error budgets for a given UE, then the PCF picks the most stringent budget. The PCF takes a policy decision and then the PCF may initiate an AM Policy Association Modification procedure for the UE as described in clause 4.16.2.2 to provide AMF the 5G access stratum time distribution parameters. As part of this, the AMF shall, if supported, store the 5G access stratum time distribution indication (enable, disable) and the Uu time synchronization error budget in the UE context in AMF and send the 5G access stratum time distribution indication (enable, disable) and the Uu time synchronization error budget, when they are available, to NG-RAN during Service Request, mobility registration, AM policy modification, N2 Handover and Xn handover using NGAP procedures as specified in TS 38.413 [10]. The NG-RAN node shall, if supported, store the information in the UE Context. Based on this information, the NG-RAN node provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

NOTE 2:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [30].

8.	The PCF of the UE replies to the TSCTSF with the result of Npcf_AMPolicyAuthorization operation.

9.	The TSCTSF responds the AF with the Ntsctsf_ASTICreate/Update/Delete/Get service operation response.

10.	The NEF informs the AF about the result of the Nnef_ASTICreate/Update/Delete/Get service operation performed in step 2.

END OF CHANGES

##### 5.2.2.2.2	Namf_Communication_UEContextTransfer service operation

Service operation name: Namf_Communication_UEContextTransfer

Description: Provides the UE context to the consumer NF.

Input, Required: 5G-GUTI or SUPI, Access Type, Reason.

Input, Optional: Integrity protected message from the UE that triggers the context transfer.

Output, Required: The UE context of the identified UE or only the SUPI and an indication that the Registration Request has been validated. The UE context is detailed in table 5.2.2.2.2-1.

Output, Optional: Mobile Equipment Identifier (if available), Allowed NSSAI, Mapping Of Allowed NSSAI.

See clause 4.2.2.2.2 for example of usage of this service operation. If the consumer NF sent an integrity protected message from the UE, the AMF uses it to verify whether this request is permitted to retrieve the UE context of the UE. If it is permitted, the AMF provides UE context to the consumer NF in the Namf_Communication_UEContextTransfer response. The following table illustrates the UE Context:

Table 5.2.2.2.2-1: UE Context in AMF

| Field | Description |
| --- | --- |
| SUPI | SUPI (Subscription Permanent Identifier) is the subscriber's permanent identity in 5GS. |
| Routing Indicator | UE's Routing Indicator that allows together with SUCI/SUPI Home Network Identifier to route network signalling to AUSF and UDM instances capable to serve the subscriber. |
| Home Network Public Key identifier | UE's Network Public Key identifier that may be used together with SUCI/SUPI Home Network Identifier and Routing Indicator to route network signalling to AUSF and UDM instances capable to serve the subscriber. |
| AUSF Group ID | The AUSF Group ID for the given UE. |
| UDM Group ID | The UDM Group ID for the UE. |
| PCF Group ID | The PCF Group ID for the UE. |
| SUPI-unauthenticated-indicator | This indicates whether the SUPI is unauthenticated. |
| GPSI | The GPSI(s) of the UE. The presence is dictated by its storage in the UDM. |
| 5G-GUTI | 5G Globally Unique Temporary Identifier. |
| PEI | Mobile Equipment Identity. |
| Internal Group ID-list | List of the subscribed internal group(s) that the UE belongs to. |
| UE Specific DRX Parameters | UE specific DRX parameters for E-UTRA and NR. |
| UE Specific DRX Parameters for NB-IoT | UE Specific DRX Parameters for NB-IoT. |
| UE MM Network Capability | Indicates the UE MM network capabilities. |
| 5GMM Capability | Includes other UE capabilities related to 5GCN or interworking with EPS. |
| Events Subscription | List of the event subscriptions by other CP NFs. Indicating the events being subscribed as well as any information on how to send the corresponding notifications. |
| LTE-M Indication | Indicates if the UE is a Category M UE. This is based on indication provided by the NG-RAN or by the MME at EPS to 5GS handover. |
| NR RedCap Indication | Indicates if the UE is a NR RedCap UE. This is based on indication provided by the NG-RAN as specified in TS 23.501 [2]. |
| MO Exception Data Counter | MO Exception Data Counter used for Small Data Rate Control purposes, see clause 5.31.14.3 of TS 23.501 [2]. |
| AMF-Associated Expected UE Behaviour parameters | Indicates per UE the Expected UE Behaviour Parameters and their corresponding validity times as specified in clause 4.15.6.3. |
| Disaster Roaming | Indicates the UE is registered for Disaster Roaming service. |
| SNPN Onboarding indication | Indicates that the UE is registered for onboarding in an SNPN. |
| For the AM Policy Association: |  |
| AM Policy Information | Information on AM policy provided by PCF. It includes the Policy Control Request Triggers and the Policy Control Request Information. It includes the authorized RFSP, the authorized Service Area Restrictions, access stratum time distribution indication and the Uu interface time synchronization error budget. It includes an indication per DNN, S-NSSAI for notification of the establishment and termination of the SM Policy Association and the PCF binding information. |
| PCF ID | The identifier of the PCF for AM Policy. In roaming, the identifier of V-PCF (NOTE 2). |
| For the UE Policy Association: |  |
| Trigger Information | The Policy Control Request Triggers on UE policy provided by PCF. |
| PCF ID(s) | The identifier of the PCF for UE Policy. In roaming, the identifiers of both V-PCF and H-PCF (NOTE 1) (NOTE 2). |
| For the UE NWDAF association: |  |
| NWDAF ID(s) | Indicating the NWDAF ID(s) (instance ID(s) or Set ID(s)) used for the UE specific Analytics. |
| Subscription Correlation ID(s) | Active UE-related analytics subscription(s) for each given NWDAF ID. |
| Analytics ID(s) | Analytics ID(s) per NWDAF ID. |
| Analytics specific data | Additional information on the Analytics ID(s) the AMF is subscribed related to the UE specific Analytics, i.e. per Analytics ID it contains the following parameters: Analytics Filter Information, Target of Analytics reporting, Analytics Reporting Info. |
| Other information |  |
| Subscribed RFSP Index | An index to specific RRM configuration in the NG-RAN that is received from the UDM. |
| RFSP Index in Use | An index to specific RRM configuration in the NG-RAN that is currently in use. |
| 5G access stratum time distribution indication | The 5G access stratum time distribution indication to be provided to RAN based on the 5G access stratum time distribution indication received from the PCF. |
| Uu time synchronization error budget | The Uu time synchronization error budget to be provided to RAN based on the Uu time synchronization error budget received from the PCF. |
| UE-AMBR in serving network | The UE-AMBR that has been sent to RAN (e.g. based on subscribed UE-AMBR from UDM or UE-AMBR received from PCF) |
| List of UE-Slice-MBR(s) | The list of UE-Slice-MBR if applicable. There is a single uplink and a single downlink value per S-NSSAI. |
| MICO Mode Indication | Indicates the MICO Mode for the UE. |
| Extended idle mode DRX Parameters | Negotiated extended idle mode DRX parameters. |
| Active Time Value for MICO mode | UE specific Active Time value allocated by AMF for MICO mode handling. |
| Strictly Periodic Registration Timer Indication | An indication that UE shall perform the Periodic Registration Update in a strictly periodic time, see clause 5.31.7.5 of TS 23.501 [2]. |
| Voice Support Match Indicator | An indication whether the UE radio capabilities are compatible with the network configuration. The AMF uses it as an input for setting the IMS voice over PS Session Supported Indication over 3GPP access. |
| Homogenous Support of IMS Voice over PS Sessions | Indicates per UE if "IMS Voice over PS Sessions" is homogeneously supported in all TAs in the serving AMF or homogeneously not supported, or, support is non-homogeneous/unknown, see clause 5.16.3.3 of TS 23.501 [2]. |
| UE Radio Capability for Paging Information | Information used by the NG-RAN to enhance the paging towards the UE (see clause 5.4.4.1 of TS 23.501 [2]). |
| Information On Recommended Cells And RAN nodes For Paging | Information sent by the NG-RAN, and used by the AMF when paging the UE to help determining the NG-RAN nodes to be paged as well as to provide the information on recommended cells to each of these NG-RAN nodes, in order to optimize the probability of successful paging while minimizing the signalling load on the radio path. |
| UE Radio Capability Information | Information sent by the NG-RAN node and stored in the AMF. The AMF sends this information to the NG-RAN node within the UE context during transition to CM-CONNECTED state, except for NB-IoT when NB-IoT specific UE Radio Access Capability are sent instead. |
| UE Radio Capability ID | Pointer that uniquely identifies a set of UE Radio Capabilities in UCMF as defined in TS 23.501 [2]. |
| NB-IoT specific UE Radio Access Capability Information | NB-IoT specific UE radio access capabilities. |
| WUS Assistance Information | Assistance information for determining the WUS group (see TS 23.501 [2]). |
| Paging Subgrouping Support Indication | UE indication of its capability to support NR paging subgrouping. |
| AMF PEIPS Assistance Information | AMF assigned NR paging subgroup information for use in NR paging subgrouping (see TS 23.501 [2]). |
| SMSF Identifier | The Identifier of the SMSF serving the UE in RM-REGISTERED state. |
| SMSF Address | The Address of the SMSF serving the UE in RM-REGISTERED state. (see clause 4.13.3.1). |
| SMS Subscription | Indicates subscription to any SMS delivery service over NAS irrespective of access type. |
| SEAF data | Master security information received from AUSF. |
| Last used EPS PLMN ID | The identifier of the last used EPS PLMN. |
| Paging Assistance Data for CE capable UE | Paging Assistance Data for Enhanced Coverage level and cell ID provided by the last NG-RAN the UE was connected to. |
| Enhanced Coverage Restricted Information | Specifies per PLMN whether CE mode B is restricted for the UE, or both CE mode A and CE mode B are restricted for the UE, or both CE mode A and CE mode B are not restricted for the UE. |
| NB-IoT Enhanced Coverage Restricted Information | Specifies per PLMN whether the Enhanced Coverage is restricted or not for the UE. |
| Service Gap Time | Used to set the Service Gap timer for Service Gap Control (see clause 5.31.16 of TS 23.501 [2]). |
| Running Service Gap expiry time | The time of expiry of a currently running Service Gap Timer (see clause 5.31.16 of TS 23.501 [2]). |
| NB-IoT UE Priority | Numerical value used by the NG-RAN to prioritise between UEs accessing via NB-IoT. |
| List of Small Data Rate Control Statuses | List of Small Data Rate Control Statuses by DNN and S-NSSAI for the released PDU Sessions, see clause 5.31.14.3 of TS 23.501 [2]. |
| List of APN Rate Control Statuses | Indicates for each APN, the APN Rate Control Status (see clause 4.7.7.3 of TS 23.401 [13]) received from an MME when mobility from EPC to 5GC occurs. This information is provided to the MME during 5GC to EPC mobility. |
| UE positioning capability | Information sent by the LMF and stored in the AMF. The AMF sends this information along with the location request to the LMF. |
| For each access type level context within the UE access and mobility context: |  |
| Access Type | Indicates the access type for this context. |
| RM State | Registration management state. |
| UUAA-MM Status | Indicates the status of UUAA-MM if the AMF is configured to perform the UAV authentication/authorization at 5GS registration as described in clause 5.2.2 of TS 23.256 [80]. Possible states are "PENDING", "SUCCESS", "FAILED". For status "PENDING" and "FAILED" the AMF rejects any PDU session establishment request from the UE for DNN and S-NSSAI that are used for UAS services. |
| Registration Area | Current Registration Area (a set of tracking areas in TAI List). |
| TAI of last Registration | TAI of the TA in which the last Registration Request was initiated. |
| User Location Information | Information on user location. |
| Mobility Restrictions | Mobility Restrictions restrict mobility handling or service access of a UE. It consists of RAT restriction, Forbidden area, Service area restrictions and Core Network type restriction. It may also contain an Allowed CAG list and, optionally an indication whether the UE is only allowed to access 5GS via CAG cells. |
| Security Information for CP | As defined in TS 33.501 [15]. |
| Security Information for UP | As defined in TS 33.501 [15]. |
| Allowed NSSAI | Allowed NSSAI consisting of one or more S-NSSAIs for serving PLMN in the present Registration Area. |
| Mapping Of Allowed NSSAI | Mapping Of Allowed NSSAI is the mapping of each S-NSSAI of the Allowed NSSAI to the S-NSSAIs of the Subscribed S-NSSAIs. |
| S-NSSAIs subject to Network Slice-Specific Authentication and Authorization | Subscribed S-NSSAIs which are subject to NSSAA procedure. Also including the status, i.e. result, of the NSSAA if already executed or whether the S-NSSAI is pending the completion of an NSSAA procedure. |
| Inclusion of NSSAI in RRC Connection Establishment Allowed by HPLMN | [Only for 3GPP access] it defines whether the UDM has indicated that the UE is allowed to include NSSAI in the RRC connection Establishment in clear text. |
| Access Stratum Connection Establishment NSSAI Inclusion Mode | Defines what NSSAI, if any, to include in the Access Stratum connection establishment as specified in clause 5.15.9 of TS 23.501 [2]. |
| CM state for UE connected via N3IWF/TNGF | Identifies the UE CM state (CM-IDLE, CM-CONNECTED) for UE connected via N3IWF/TNGF |
| N2 address information for N3IWF/TNGF | Identifies the N3IWF/TNGF to which UE is connected. Exists only if CM state for UE connected via N3IWF/TNGF is CM-CONNECTED. |
| AMF UE NGAP ID | Identifies the UE association over the NG interface within the AMF as defined in TS 38.413 [10]. This parameter exists only if CM state for the respective Access Type is CM-CONNECTED. |
| RAN UE NGAP ID | Identifies the UE association over the NG interface within the NG-RAN node as defined in TS 38.413 [10]. This parameter exists only if CM state for the respective Access Type is CM-CONNECTED. |
| Network Slice Instance(s) | The Network Slice Instances selected by 5GC for this UE. |
| URRP-AMF information | UE Reachability Request Parameter contains a list of URRP-AMF flags and associated authorised NF IDs. Each URRP-AMF flag indicates whether direct UE reachability notification has been authorised by the HPLMN towards the associated NF ID or not. |
| SoR Update Indicator for Initial Registration | An indication whether the UDM requests the AMF to retrieve SoR information when the UE performs NAS Registration Type "Initial Registration". |
| SoR Update Indicator for Emergency Registration | An indication whether the UDM requests the AMF to retrieve SoR information when the UE performs NAS Registration Type "Emergency Registration". |
| Charging Characteristics | The Charging Characteristics as defined in Annex A of TS 32.256 [71]. |
| For each PDU Session level context: |  |
| S-NSSAI(s) | The S-NSSAI(s) associated to the PDU Session. |
| DNN | The associated DNN for the PDU Session. |
| Network Slice Instance id | The network Slice Instance information for the PDU Session |
| PDU Session ID | The identifier of the PDU Session. |
| SMF Information | The associated SMF identifier and SMF address for the PDU Session. When an I-SMF is used, this additionally include the information correspond to an I-SMF. |
| Access Type | The current access type for this PDU Session (for a MA PDU Session this may correspond to information indicating 2 Access Type). |
| EBI-ARP list | The allocated EBI and associated ARP pairs for this PDU session. |
| 5GSM Core Network Capability | The UEs 5GSM Core Network Capability as defined in clause 5.4.4b of TS 23.501 [2]. |
| SMF derived CN assisted RAN parameters tuning | These are PDU Session specific parameters received from the SMF and used by the AMF to derive the Core Network assisted RAN parameters tuning. |
| NOTE 1: The AMF transfers the PCF ID to the SMF during PDU Session Establishment. The SMF may select the PCF identified by the PCF ID as described in clause 6.3.7.1 of TS 23.501 [2]. In HR roaming case, the AMF transfers the identifier of H-PCF as described in clause 4.3.2.2.2. In LBO roaming case, the AMF transfers the identifier of V-PCF as described in clause 4.3.2.2.1. NOTE 2: The PCF ID in AM Policy Association information and the PCF ID in UE Policy Association Information should be the same in non-roaming case. The V-PCF ID in AM Policy Association information and the V-PCF ID in UE Policy Association Information should be the same in roaming case. |  |

END OF CHANGES