3GPP SA WG2 Meeting #151E e-meeting	S2-2203832r01

16 – 20 May, 2022, Elbonia

Source:	Ericsson

Title:	KI #1, Sol #14: Update to remove ENs

Document for:	Discussion/Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This contribution proposes to address editor’s notes in Solution #14 and add specific procedures.

# 1	Discussion

The aim of this paper is to update Solution 14 in TR 23.700-25 by adding procedures that implement the solution and by addressing/discussing editor’s notes (ENs).

### Procedures

The solution proposes changes to existing procedures in TS 23.502, clauses 4.15.9.3.1 and 4.15.9.4. An extract of the procedures is presented, and the specific changes are highlighted.

### 1.2	Addressing EN 1

Editor's note:	Whether to include the phase/frequency level of stability and accuracy, and the service availability (including holdover time) from the AF request is FFS.

The level of stability should be a general requirement that applies to any type of timing information (time, frequency, phase). This is not a requirement specific for this solution only.

Service availability, which is related to holdover time, is a general requirement when a reliable service is to be provided (such as timing resiliency), and not a particular requirement for this solution.

When timing information is provided in terms of frequency or phase (such as in solution 14), specific information is needed, e.g., frequency accuracy (as defined in ITU-T G.810) compared to the UTC frequency and phase accuracy compared to the UTC phase. ITU-T G.8260 defines the difference between phase synchronization (clause 3.1.16) and time synchronization (clause 3.1.19). Phase accuracy is similar to time accuracy (or error budget) as it has to do with alignment with the UTC second. Phase accuracy can be measured by means of time error (ITU-T G.8260, clause 3.1.20). Frequency accuracy is related to both the input frequency accuracy to the gNB, and to the potential degradation from the radio link. Note that likewise time error budget, the part of frequency or phase accuracy left for the Uu interface should be calculated (by TSCTSF) and delivered to NG-RAN.

Moreover, since these parameters are used as part of the AF request as proposed in this solution, it makes sense to also include them in the time synchronization subscription data (as per KI #3) if this solution is approved. Hence, a note is added indicating so.

### 1.3	Addressing EN 2

Editor's note:	Use of the two types timing method (assisted and complement) details by NG-RAN are FFS.

The specific requirements of using either assisted or complement timing methods need to be provided to NG-RAN: only in case of complement timing method, phase synchronization must be provided, in case of assisted timing, that is not necessarily the case. Therefore, NG-RAN would require the knowledge of which type of timing method is to be provided.

To illustrate the above reasoning about complement timing, additional description of an implementation example applied to financial service application is provided in the following figure.

Figure 1. Example Timing service

The time distributed to a financial server is using NTP. In this example it is assumed that the accuracy that can be guaranteed by the NTP service is e’ < +/- 5 ms (i.e., deviation from the ideal UTC time is less than 5 ms).

The recovered NTP time reference is distributed to the DS-TT that also is connected to a UE through which it can receive 3GPP radio interface derived timing.

The frame timing of the 3GPP radio interface is controlled and aligned towards UTC by a gNB having access to a UTC traceable reference (e.g., GNSS) and its accuracy within the UE (e.g., DS-TT) can be assumed to be in the order of a few microseconds (better than 10 s) including RF air interface delay errors and internal UE errors.

The start of the radio frame is used by the DS-TT to re-align and to improve original NTP UTC timing to be better than 10 s accuracy (e < 10 s). In practice moving the NTP reference to the closest start of the radio frame.

Due to the fact that the NTP time is accurate within +/- 5 ms, and that the radio frame has a periodicity of 10 ms, the NTP “second” must correspond to the closest edge of the start of a radio frame.

### 1.4	Addressing EN 3

Editor's note:	If is FFS how these parameters should influence to the content of (g)PTP messages or (g)PTP-related IEEE data sets in PMIC/UMIC.

To address this editor’s note, it is proposed to add the following note:

NOTE: There can be implications regarding the provided frequency/phase accuracy influencing the operation of (g)PTP (including correcting the content of (g)PTP messages) and certain parameters (such as clockClass) in the IEEE1588 data sets to be delivered to PMIC/BMIC. (g)PTP messages may not be used (e.g., in case of a frequency synchronization service, or because they may not be needed when the level of accuracy is out of range), however (g)PTP messages may be received and there might be the need to process them and update certain fields in the Announce messages. The specific messages/parameters can be finalized during normative phase.

# 2	Proposed changes

It is proposed to agree the following changes to TR 23.700-25, regarding Solution #14:

***** Beginning of CHANGES *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[6]	IEEE 802.1Qcc: "IEEE Standard for Local and Metropolitan Area Networks--Bridges and Bridged Networks -- Amendment 31: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[7]	IEEE Std 802.1AS: "IEEE Standard for Local and Metropolitan Area Networks-Timing and Synchronization for Time-Sensitive Applications".

[8]	IEEE Std 1588: "Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems".

[9]	3GPP TR 22.878: "Feasibility Study on 5G Timing Resiliency System".

[10]	IEEE P802.1Qdj d0.2: "Configuration Enhancements for Time-Sensitive Networking".

[11]	3GPP TS 38.321: "Medium Access Control (MAC) protocol specification".

[X]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[Y]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

***** Next CHANGE *****

## 6.14	Solution #14: Assisted and Complement Timing Support

### 6.14.1	General

This solution is addressing the requirement for timing resiliency towards a client network (external network that requires a resilient timing service by the 5GS), as backup when the local primary time source fails, or as a complement to calibrate an inaccurate local primary timing source in the client network.

In these cases, delivering a stable and accurate phase or frequency is sufficient (no need to deliver time information over the 5G network).

This solution proposes to support Assisted Timing to provide backup timing service which if applied to 5GS implies a low impact since the only timing information to be provided is a stable phase or frequency only. As an alternative, the stable phase synchronization could be offered in order to complement (by calibrating) the external inaccurate primary local source, i.e., Complement Timing. In the case of Complement Timing where timing information is provided in terms of phase synchronization, NG-RAN is required to deliver phase synchronization (frequency synchronization is not sufficient).

If the client network supports receiving some form of assisted/complement timing and requires Timing Resiliency from 5GS via the already supported Time Synchronization service activation, the 5GS will be able to provide assisted/complement timing, where the timing information may be phase or frequency. The AF request therefore may indicate during the Time Synchronization service activation, that Timing Resiliency is required, whether assisted or complement timing is required, the type to timing information (phase or frequency). Additional parameters need to be provided: frequency/phase accuracy compared to UTC frequency/phase, respectively. Like with per-Uu interface time error budget, the frequency/phase accuracy has to be calculated per UE (per Uu interface) by TSCTSF and provided to NG-RAN. The timing service can be provided over different types of external interfaces, e.g., (g)PTP as defined in TS 23.501 [2] or by an implementation specific interface.

If Timing Resiliency is required, the client network should receive feedback (status report) whenever the service requirements (e.g., time error budget, UTC traceability, frequency/phase accuracy, etc.) are out of the agreed range. Moreover, status reports content just indicate that requirements were not achieved, instead of including the actual values of the parameters. How to provide the status report is out of the scope in this solution.

### 6.14.2	Functional Description

This solution is based on the following principles:

-	AF may include in its request for time synchronization service the following:

-	Timing Resiliency requirement, including the following parameters:

-	Assisted or complement timing requirement;

-	Type of timing information required (phase or frequency);

-	Frequency/phase accuracy requirement compared to UTC frequency/phase, respectively.

Editor's note:	It is FFS whether to include this list of specific parameters as part of the subscription data related to KI #3.

NOTE:	Other parameters such as service availability (including holdover time) and stability level of the timing information (time, phase or frequency) are relevant and can apply to this solution.

-	Status report is sent to UE application and to AF when one or more agreed requirements are out of range, indicating that service requirements are not achieved.

### 6.14.3	Procedures

Existing time synchronization activation procedures (see clause 4.15.9 of TS 23.502 [3]) are reused, where the AF request content is modified to include Timing Resiliency requirement, whether assisted/complement timing is required, the type of timing information (phase or frequency), frequency/phase accuracy.

Extract of clauses from TS 23.502 [3] (using approved S2-2203232 from SA2 meeting #150E for clause 4.15.9.4) are presented showing the changes proposed to support this solution. Note that, except for clause 4.15.9.3.1 where time synchronization parameters are added, procedures in clause 4.15.9.3 remain unchanged. Clause 4.15.9.4 in TS 23.502 [3] is also modified to add parameters to the time synchronization table, to state that TSCTSF calculates the Uu frequency/phase accuracy, and to transfer time resiliency parameters (including assisted or complement timing requirement, timing information type, frequency/phase accuracy), likewise Uu time synchronization error budget, to NG-RAN.

6.14.3.1	Procedures for (g)PTP time distribution

To procedures are presented as changes (underlined text) to the TS 23.502 [3], clause 4.15.9.3.1 as follows:

This procedure can be used by the AF to activate, modify or deactivate the (g)PTP instances in 5GS.

The AF may activate the time synchronization service using the Nnef_TimeSynchronization_ConfigCreate service operation. The service operation creates a time synchronization configuration based on the service parameters as indicated in the create request. The AF may update the time synchronization configuration using the Nnef_TimeSynchronization_ConfigUpdate service operation. The AF may deactivate the time synchronization service using the Nnef_TimeSynchronization_ConfigDelete service operation, which deletes the corresponding time synchronization service configuration.

The Nnef_TimeSynchronization_ConfigCreate and Nnef_TimeSynchronization_ConfigUpdate request may contain the parameters as described in Table 6.14.3.1-1.

Table 6.14.3.1-1: Description of Time Synchronization service parameters

| Time Synchronization Parameter | Description |
| --- | --- |
| PTP instance type | Identifies the requested PTP instance type as described in clause 5.27.1.4 of TS 23.501 [2]. |
| Transport protocol | Identifies the requested transport protocol for PTP instance as described in clause 5.27.1.4 of TS 23.501 [2]. This is applicable for IEEE Std 1588 [8] Boundary Clock and Transparent Clock operation. |
| PTP Profile | Identifies the PTP profile for the PTP instance as requested by AF. |
| Grandmaster enabled | Indicates whether the AF requests the PTP instance in 5GS to be able to act as a grandmaster for PTP or gPTP (depending on the requested PTP instance type). This is applicable for IEEE Std 1588 [8] Boundary Clock or IEEE Std 802.1AS [7] operation. [optional] |
| Grandmaster priority | Indicates a priority used as defaultDS.priority1 when generating Announce message when 5GS acts as (g)PTP GM. Applicable only if the Grandmaster enabled = TRUE. If omitted, the default value as described in the PTP Profile is used. [optional] |
| Time Domain | (g)PTP domain of the PTP instance as defined in IEEE Std 1588 [8]. |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service for a PTP instance is active. [optional] |
| Time synchronization error budget | Indicates the time synchronization budget for the time synchronization service (as described in clause 5.27.1.9 of TS 23.501 [2]). [optional] |
| Timing resiliency | Indicates whether the synchronization service will be used to provide timing resiliency. |
| >Assisted/Complement timing method | Indicates whether Assisted timing or Complement timing is used. |
| >Type of timing information | Indicates which type of timing information will be provided (time, frequency, phase). |
| >Required timing information accuracy | It provides the required phase or frequency (according to type of timing information specified) accuracy compared to the UTC phase or frequency (according to type of timing information specified). |
| For each PTP port in the PTP instance |  |
| Either UE identity (for a DS-TT port), or "N6 interface" indication | Identifies the UE/DS-TT which the parameters below apply. "N6 interface" indicates that the parameters below apply to the N6 interface. If the "PTP port" needs to be identified, this field refers to the UE identity (GPSI or SUPI). If the N6 termination needs to be identified, then this field indicates "N6 interface" flag, instead of SUPI or GPSI. |
| PTP enabled | TRUE/FALSE. This is used to set the portDS.portEnable. If omitted, the default value as described in the PTP Profile is used. [optional] |
| Log Sync Interval | Specifies the mean time interval between successive Sync messages. This is applicable for IEEE Std 1588 [8] Boundary Clock or IEEE Std 802.1AS [7] operation. If omitted, the default value as described in the PTP Profile is used. [optional] |
| Use management settable Log Sync Interval | TRUE/FALSE. This is applicable if the PTP Profile is I IEEE Std 802.1AS [7]. When set to FALSE, the Log Sync Interval is used to set the initialLogSyncInterval as described in IEEE Std 802.1AS [7]. When set to TRUE, the Log Sync Interval is used to set the mgtSettableLogSyncInterval as described in IEEE Std 802.1AS [7]. If omitted, the default value as described in the IEEE Std 802.1AS [7] is used. [optional] |
| Log Announce Interval | Specifies the mean time interval between successive Announce messages. This is applicable for IEEE Std 1588 [8] Boundary Clock or IEEE Std 802.1AS [7] operation. If omitted, the default value as described in the PTP Profile is used. [optional] |
| Use management settable Log Announce Interval | TRUE/FALSE. This is applicable if the PTP Profile is IEEE Std 802.1AS [7]. When set to FALSE, the Log Announce Interval is used to set the initialLogAnnounceInterval as described in IEEE 802.1AS. When set to TRUE, the Log Announce Interval is used to set the mgtSettableLogAnnounceInterval as described in IEEE Std 802.1AS [7]. If omitted, the default value as described in the IEEE Std 802.1AS [7] is used. [optional] |

The AF may use Nnef_TimeSynchronization_CapsSubscribe service operation as described in clause 4.15.9.2 in TS 23.502 [3] to learn the UE capabilities for time synchronization service. The Nnef_TimeSynchronization_CapsNotify service operation indicates the list of UE identities, User-plane Node ID, and the Subscription Correlation ID. The AF can use the Subscription Correlation ID and the user-plane node ID received in the Nnef_TimeSynchronization_CapsNotify service operation as a target of the Nnef_TimeSynchronization_ConfigCreate request. The NEF uses the Subscription Correlation ID and user-plane node ID to determine the list of UEs and list of AF-sessions to which the Nnef_TimeSynchronization_ConfigCreate service operation is targeted to.

#### 6.14.3.2	Procedures for 5G access stratum time distribution

To procedures are presented as changes (underlined text) to the TS 23.502 [3], clause 4.15.9.4 (taken from approved S2-2203232) as follows:

The AF can use the procedure to activate, update or delete the 5G access stratum time distribution for one UE or a group of UEs.

The AF may query the status of the 5G access stratum time distribution using Nnef_ASTIGet service operation. The Nnef_ASTICreate and Nnef_ASTIUpdate request may contain the parameters as described in Table 6.14.3.2-1.

Table 6.14.3.2-1: Description of 5G access stratum time distribution parameters

| Parameter | Description |
| --- | --- |
| 5G access stratum time distribution indication (enable, disable) | Indicates that the access stratum time distribution via Uu reference point should be activated or deactivated for the associated UE identities. |
| Time synchronization error budget | Indicates the time synchronization error budget for the time synchronization service (as described in clause 5.27.1.9 of TS 23.501 [2]). [optional] |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service is active. [optional] |
| Timing resiliency | Indicates whether the synchronization service will be used to provide timing resiliency. |
| >Assisted/Complement timing method | Indicates whether Assisted timing or Complement timing is used. |
| >Type of timing information | Indicates which type of timing information will be provided (frequency, phase). |
| >Required timing information accuracy | It provides the required phase or frequency accuracy (according to type of timing information specified) compared to the UTC phase or frequency (according to type of timing information specified). |

Figure 6.14.3.2-1: Management of 5G access stratum time information

1.	AM Policy Association establishment as described in clause 4.16.1 in TS 23.502 [3].

2.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	To create a new request, the AF provides access stratum time distribution parameters to the NEF using the Nnef_ASTI_Create service operation (together with the AF identifier and potentially further inputs as specified in table 6.14.3.2-1), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier. The NEF maps the External Group Identifier to an Internal Group Identifier and any GPSI to a SUPI.

-	To update or remove an existing request, the AF invokes an Nnef_ASTI_Update or Nnef_ASTI_Delete service operation providing the corresponding time synchronization configuration id.

-	To query the status of the access stratum time distribution, the AF invokes Nnef_ASTI_Get service operation providing the target (List of UE identities (SUPI or GPSI)).

The AF that is part of operator's trust domain may invoke the services directly with the TSCTSF.

NOTE 1:	Steps 1 and 2 can occur in any order.

3.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_ASTI_Create/Update/Delete/Get service operation with the TSCTSF discovered and selected as described in clause 6.3.24 of TS 23.501 [2].

-	The TSCTSF determines whether the targeted UE is part of a PTP instance in 5GS, if so the TSCTSF rejects the request (steps 4–10 are skipped).

(When the procedure is triggered by PTP instance activation, modification, or deactivation in the TSCTSF):

-	If time synchronization error budget is provided by the AF, the TSCTSF may use the PTP port state of each DS-TT to determine an Uu time synchronization error budget for corresponding SUPIs that are part of the PTP instance.

-	If time synchronization error budget is provided by the AF, the TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2].

-	If frequency or phase accuracy compared to UTC phase or frequency respectively, has been provided, the TSCTSF calculates the Uu accuracy value.

4.	If the AF request targets a group of UEs, the TSCTSF uses the Nudm_SDM_Get request to retrieve the subscription information (SUPI list) from the UDM using the Internal Group ID.

5.	The UDM provides the Nudm_SDM_Get response containing a SUPI list that identifies UEs that belong to that group of UEs.

6.	The TSCTSF searches the PCF for the UE using Nbsf_Management_Subscribe with a SUPI as an input parameter, indicating that it is searching for the PCF that handles the AM Policy Association of the UE.

7.	The BSF provides to the TSCTSF the identity of the PCF for the UE for the requested SUPI via an Nbsf_Management_Notify operation. If matching entries already existed in the BSF when step 6 is performed, this shall be immediately reported to the TSCTSF.

8.	The TSCTSF sends to the PCF for the UE its request for the AM policy of the UE (identified by SUPI) using Npcf_AMPolicyAuthorization request, containing the 5G access stratum time distribution indication (enable, disable) and optionally the calculated Uu time synchronization error budget, and the timing resiliency parameters (if provided), including the Uu frequency or phase accuracy.

9.	If the PCF receives multiple time synchronization error budgets or frequency/phase accuracy for a given UE, then the PCF picks the most stringent budget or frequency/phase accuracy. The PCF takes a policy decision and then the PCF may initiate an AM Policy Association Modification procedure for the UE as described in clause 4.16.2.2 in TS 23.502 [3] to provide AMF the 5G access stratum time distribution parameters. As part of this, the AMF shall, if supported, send the 5G access stratum time distribution indication (enable, disable) and the Uu time synchronization error budget and timing resiliency parameters, when they are available, to an NG-RAN node using NGAP procedures (e.g., UE Context Setup/Modification) specified in TS 38.413 [X]. The NG-RAN node shall, if supported, store the information in the UE Context. Based on this information, the NG-RAN node provides the 5GS access stratum timing to the UE according to the Uu time synchronization error budget and timing resiliency parameters as provided by the TSCTSF (if supported by UE and NG-RAN).

NOTE 2:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [Y].

10.	The PCF of the UE replies to the TSCTSF with the result of Npcf_AMPolicyAuthorization operation.

11.	The TSCTSF responds the AF with the Ntsctsf_ASTI_Create/Update/Delete/Get service operation response.

12.	The NEF informs the AF about the result of the Nnef_ASTI_Create/Update/Delete/Get service operation performed in step 2.

### 6.14.4	Impacts on services, entities and interfaces

-	AF: Formulation of the AF request that may include parameters indicating requirements for Timing Resiliency, assisted timing/complement timing and type of timing information. These parameters are delivered to NG-RAN.

NOTE:	There can be implications regarding the provided frequency/phase accuracy influencing the operation of (g)PTP (including correcting the content of (g)PTP messages) and certain parameters (such as clockClass) in the IEEE1588 data sets to be delivered to PMIC/BMIC. (g)PTP messages may not be used (e.g., in case of a frequency synchronization service, or because they may not be needed when the level of accuracy is out of range), however (g)PTP messages may be received and there might be the need to process them at the TTs and update certain fields in the Announce messages. The specific messages/parameters can be finalized during normative phase.

-	NG-RAN: supporting delivery of frequency and phase synchronization. gNB may receive requirement parameters such as Timing Resiliency requirement, assisted/complement timing, the timing information type to be delivered, and frequency/phase accuracy compared to UTC frequency/phase respectively. This information enables NG-RAN to determine if and how to provide 5G reference timing information to the UE efficiently (i.e., RRC/SIB). How NG-RAN is using this information is up to RAN implementation.

-	TSCTSF: calculates the frequency/phase accuracy for the UE (Uu interface) and transfers to NG-RAN via PCF, AMF.

***** End of CHANGES *****