SA WG2 Meeting #151E e-meeting	S2-2203868

Elbonia, May 16th – 20th, 2022

Source:	Ericsson

Title:	KI#3, Solution#10: Update for resolving ENs

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract: This contribution resolves Editor’s notes in solution#10 in TR 23.700-81 v0.2.1.

# Discussion

Solution#10 defines Gateway Exposure Function (GEF) to manage the data/analytics transfer between two PLMNs. In the solution, there are serveral ENs.

The first EN we try to resolve in this contribution is:

Editor's note:	Whether to reuse existing or new service to request data/analytics from the vGEF is FFS

GEF can take the data or analytics request as NWDAF, therefore the current data or analytics subscription defined in  clause 7.2, 7.3 and 7.4 in TS 23.288 can be resued as the data and analytics subscription to GEF.

The second EN we try to resolve in this contribution is:

Editor's note:	Whether vGEF is used in the procedure or vNWDAF can send directly to the hGEF is FFS.

We argue that there are a couple of advantages that vNWDAF contact vGEF for data/analytics collection from HPLMN: One advantage is vGEF is the gateway and can aggregate the data/analytics request from vNF, so that there will not be multiple identical request to the hGEF from the same vPLMN. The second advantage is better service access and authorization control, i.e hNRF only issue access token to vGEF to access hGEF, not to other vNFs; SEPP only allows GEF service communiction from other GEFs, not from other types of NFs. Therefore, we suggest that vNF collect data/analytics via vGEF.

The third EN we try to resolve in this contribution is:

Editor's note:	How to discover hGEF via vNRF and hNRF is FFS.

How vNRF discover a hNF via hNRF is already defined in reference point N27 in TS 23.501.

# Proposed Solution

The following text is proposed to be added to TR 23.700-81 v0.2.1.

********** Start of Changes ***************

## 6.10	Solution #10: Support for Data and Analytics Exchange in Roaming Case

### 6.10.1	Description

When two VPLMN and HPLMN expose data or analytics to each other, both PLMNs need the ability to control the amount of data exposed and to abstract or hide network-internal aspects based on user consent, operator policy, regulatory constraints and/or roaming agreements.

The solution following the following assumptions and principles:

-	The hPLMN may impose constraints on the type or amount of data collected. The constraints may differ for different vPLMNs.

-	The vPLMN may impose constraints on the type or amount of data collected. The constraints may differ for different hPLMNs.

-	The data collection must honor user data consent for user related data collection.

The solution proposes a Gateway Exposure Function (GEF) in a PLMN, which is responsible for data/analytics exposure to other PLMNs. The GEF is provisioned with constraints on the type and amount of data exposed to each PLMN according to roaming agreement. The GEF is also responsible for enforcing obtaining the user consent for data collection where applicable.

When a data/analytics consumer in PLMN A (vPLMN/hPLMN) needs to collect data/analytics from PLMN B (hPLMN/vPLMN), the consumer contacts the GEF in PLMN A, then GEF in PLMN A contacts GEF in PLMN B, which discovers the data/analytics producer in PLMN B and collects the data/analytics. GEF is also responsible for data/analytics manipulation before sending to the other PLMN according to the regulatory constraints and agreement.

Editor's note:	It is FFS if the GEF is a standalone function or collocated with either NEF, DCCF or NWDAF.

### 6.10.2	Procedures

Figure 6.10.2-1: Data and Analytics Exposure via GEF when data/analytcis consumer is in vPLMN

Figure 6.10.2-1 shows the procedure when the data/analytics consumer is in the visiting PLMN (vPLMN), the NWDAF in the visiting PLMN (vNWDAF) needs to collect data/analytics from home PLMN (hPLMN) of the UE.

NOTE:	The UE can be one single UE, group of UE or any UE roaming to the VPLMN from the HPLMN.

It is assumed that both vGEF and hGEF have been provisioned, by their respective operators, with the policies and constraints for exporting data to or from roaming/home networks.

0.	GEF is registered itself in the NRF.

1-2.	The analytics consumer in the vPLMN (e.g. an AMF) wants to subscribe an Analytics for an inbound roaming UE, the vNWDAF determines that it needs to collect data/analytics from hPLMN of the UE. For example, the data to be collected might be related to expected UE behaviour. The analytics consumer and vNWDAF obtain the hPLMN ID from the SUPI of the UE. In the analytics request in step 1, the analytics consumer may provide indication to the vNWDAF, that the analytics needs data from hPLMN of the UE, or the vNWDAF can determine that based on the Analytics ID.

3.	vNWDAF discovers vGEF via vNRF. vNRF provides the ID of vGEF that is responsible for collecting data/analytics from that PLMN.

Editor's note:	It is FFS on GEF discovery and selection.

4.	vNWDAF sends data/analytics subscription request to the vGEF. Data management and analytics exposure service defined in clause 7.2, 7.3 and 7.4 in TS 23.288[5] can be reused as the data and analytics subscription services provided by the GEF.

Editor's note:	It the GEF is collocated with NWDAF, NWDAF services could be reused and otherwise (e.g. for a standalone GEF) new similar services provided by the GEF can be defined.  However, if the GEF is collocated with NWDAF and existing services are reused, it is FFS how the NWDAF determines that special constraints need to be enforced and whether new parameters are required.

5.	vGEF discovers hGEF via vNRF and hNRF. vNRF interacts with hNRF via N27 reference point as defined in TS 23.501[2].

6.	vGEF send data/analytics subscription request to the hGEF which is got in step 5. hGEF checks roaming agreements, hPLMN policies and regulatory constraints between hPLMN and the origin vPLMN to determine if the request can be accepted or must be rejected.

7-8.	If the checks in step 6 is OK, hGEF enforces user consent (for UE-related data collection) and collects data/analytics from the data/analytics sources in the hPLMN.

9.	hGEF manipulates the data/analytics according to the hPLMN policies and regulatory constraints and roaming agreements, e.g. remove some sensitive data, changes the granularity of the data, anonymization, formatting, etc.

10-11.	hGEF sends the data/analytics to vGEF and vGEF forwards to vNWDAF. vGEF may also manipulate the data/analytics to the format that the vNWDAF can understand.

12.	Alternatively, hGEF can send the data/analytics notification directly to the vNWDAF, if vGEF provides notification endpoint information of the vNWDAF in step 6.

When the analytics/data consumer is located in hPLMN, and the UE is roaming, the procedure shown in figure 6.10.2-2 applies instead. The difference from in figure 6.10.2-1 is step 3, which is now split into steps 3a and 3b. In step 3a the hNWDAF enforces the user consent with the hUDM. In step 3b the hNWDAF queries hUDM for retrieving the GUAMI of the AMF that is serving the user. This GUAMI contains the PLMN ID of the AMF, therefore, it can be used for determining whether the UE is roaming or not.

Figure 6.10.2-2: Data and Analytics Exposure via GEF when data/analytics consumer is in hPLMN

### 6.10.3	Impacts on services, entities and interfaces

Editor's note:	The impacts on services, entities and interfaces are FFS.

********** End of Changes ***************