3GPP TSG-SA2 Meeting #151E	S2-2203869r01

# E-Meeting, 16th – 20th  May 2022

Source:	Ericsson, Huawei

Title:	KI#4: New Solution: Requesting collection of buffered data

Spec:	3GPP TR 23.700-81

Agenda item:	9.23

Document for:	Approval

Work Item / Release:	FS_eNA_ph3 / Rel-18

1. Introduction

The Key Issue #4: How to Enhance Data collection and Storage includes the following item to study:

- Whether and what other enhancements are required for storage of data and/or analytics in ADRF, NWDAF and/or data source NF.

This solution is solving the problem with a data producing NF or NWDAF having shortage of memory to store data or that it will soon purge data for any other reason. An NF may have reserved certain amount of memory for data to be stored for each subscription of data that has been muted or is low on memory in general.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-81 v0.2.1.

* * * First change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1 | X |  |  |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |  |  |  |
| 7 | X |  |  |  |  |  |  |  |  |  |
| 8 |  | X |  |  |  |  |  |  |  |  |
| 9 |  | X |  |  |  |  |  |  |  |  |
| 10 |  |  | X |  |  |  |  |  |  |  |
| 11 |  |  | X |  |  |  |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |  |  |
| 13 |  |  |  |  | X |  |  |  |  |  |
| 14 |  |  |  |  | X |  |  |  |  |  |
| 15 |  |  |  |  | X |  |  |  |  |  |
| 16 |  |  |  |  |  | X |  |  |  |  |
| 17 |  |  | X |  |  | X |  |  |  |  |
| 18 |  |  |  |  |  |  | X |  |  |  |
| 19 |  |  |  |  |  |  | X |  |  |  |
| 20 |  |  |  |  |  |  | X |  |  |  |
| 21 |  |  |  |  |  |  |  | X |  |  |
| 22 |  |  |  |  |  |  |  | X |  |  |
| 23 |  |  |  |  |  |  |  | X |  |  |
| 24 |  |  |  |  |  |  |  | X |  |  |
| 25 |  |  |  |  |  |  |  |  | X |  |
| 26 |  |  |  |  |  |  |  |  | X |  |
| 27 |  |  |  |  |  |  |  |  | X |  |
| X |  |  |  | X |  |  |  |  |  |  |

* * * Second change (all new text) * * * *

6.X	Solution #X: Sending data that is about to be purged

6.X.1	Description

This solution is solving the problem with a data producing NF or NWDAF having heavy load or performance (e.g. due to shortage of memory) to store data or that it will soon purge data for any other reason. An NF may have light load or performance (e.g. due to reserved certain amount of memory or low on memory) for data to be stored for each subscription of data that has been muted.

6.X.2	Procedures

This solution re-use existing procedures.

Figure 6.X.2-1: Procedures of the solution

1.	The data producer (any NF storing data - subject to mute or similar) determines to soon purge data subscribed to by data consumer NF (NWDAF or DCCF).

2.	The data producer does the following.

-	In the case of a data producer NF being subscribed to with mute (Deactivate notification flag is set) it performs an Nnf_EventExposure_Notify service operation including the data collected and the cause code why sending the data.

NOTE:	In the case of NWDAF, DCCF being subscribed for Data and formatting and pre-processing instructions is instructed it performs an Nnwdaf_DataManagement_Notify service operation including fetch instruction, according as described in TS 23.288 [6].

3.	The data producer purges data.

6.X.3	Impacts on services, entities and interfaces

Data producer:

- Adding a cause (e.g. heavy NF load or performance) in the exposure explaining why data is sent even when Deactivate notification flag is set

* * * End of changes * * * *