3GPP TSG-WG SA2 Meeting #151-e	S2-2203890r1

# Electronic Meeting, 2022‑05‑16 -- 2022‑05‑20

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3635 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NRF discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI17 |  |  |  |  |  | Date: |  |  | 2022-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Discovery and selection of NRF no specified, alignment with stage 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding discovery and selection and use of NRF boot strapping service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missalignement between Stage 2 and Stage 3, can lead to wrong functionality from stage 2 perspective. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.1, 6.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 23.502 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st changes ********************

### 6.2.6	NRF

#### 6.2.6.1	General

The Network Repository Function (NRF) supports the following functionality:

-	Supports service discovery of NRF services and their endpoint addresses by the NRF bootstrapping service.

-	Supports service discovery function. Receive NF Discovery Request from NF instance or SCP, and provides the information of the discovered NF instances (be discovered) to the NF instance or SCP.

-	Supports P-CSCF discovery (specialized case of AF discovery by SMF).

-	Maintains the NF profile of available NF instances and their supported services.

-	Maintains SCP profile of available SCP instances.

-	Supports SCP discovery by SCP instances.

-	Notifies about newly registered/updated/ deregistered NF and SCP instances along with its potential NF services to the subscribed NF service consumer or SCP.

-	Maintains the health status of NFs and SCP.

In the context of Network Slicing, based on network implementation, multiple NRFs can be deployed at different levels (see clause 5.15.5):

-	PLMN level (the NRF is configured with information for the whole PLMN),

-	shared-slice level (the NRF is configured with information belonging to a set of Network Slices),

-	slice-specific level (the NRF is configured with information belonging to an S-NSSAI).

In the context of roaming, multiple NRFs may be deployed in the different networks (see clause 4.2.4):

-	the NRF(s) in the Visited PLMN (known as the vNRF) configured with information for the visited PLMN.

-	the NRF(s) in the Home PLMN (known as the hNRF) configured with information for the home PLMN, referenced by the vNRF via the N27 interface.

******************** 2nd changes ********************

### 6.3.x	NRF discovery and selection

The following mechanisms may be used for discovery of NRF service instances and their endpoint addresses:

- NF consumers or SCP may have the NRF services instances and their endpoint addresses as configured data.

- NF consumers or SCP may have the endpoint address of a NRF discovery service to discover the NRF(s) and get the NF profile of the NRF(s).

- NF consumers (e.g. v-NRF) or SCP may have enpoint addresses of the NRF bootstrapping service and utilize it to discover the NRF service instances and their endpoing addresses. The NRF bootstrapping service is a version independent API, which may be especially usefull over roaming interfaces.

- The NF consumer, e.g. AMF may use the Nnssf_NSSelection service to discovery the NRF discovery service for a certain slice.

******************** End changes ********************