SA WG2 Meeting #S2-151e(e-meeting)	S2-2203902r04

16 - 20 May 2022, Electronic Meeting	(revision of S2-220xxxx)

Source:	Ericsson, Huawei

Title:	        Updates to the TR based on response SA4 LS

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This contribution provides updates to the TR based on the SA4 response LS in S4-220505.

1. Introduction

Based on the response LS in S4-220505 on LS Reply on QoS support with PDU Set granularity, following updates are proposed:

- Replacement of video slice definition by a reference to SA4 TR 26.926 Adding further clarification concerning handling of lost PDUs.

- Annex A1 – clarification of video frame type concepts.

- Annex A2 – clarification on periodicity&jitter and on frame importance.

2. Proposal

This paper proposes the following updates to TR 23.700-60 clause 6.

* Start of change *

**** First Change ****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[X]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

**** Next Change ****

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926). In some implementations all PDUs within the PDU Set are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the application layer can use the data up to the first lost PDU in the PDU Set and apply error concealment to recover parts of the information unit.

Multi-modal Data: Multi-modal Data is defined to describe the input data from different kinds of devices/sensors or the output data to different kinds of destinations (e.g. one or more UEs) required for the same task or application. Multi-modal Data consists of more than one Single-modal Data, and there is strong dependency among each Single-modal Data. Single-modal Data can be seen as one type of data.

NOTE 1:	This definition was taken from TR 22.847 [6].

Tactile Internet: A network (or network of networks) for remotely accessing, perceiving, manipulating, or controlling real or virtual objects or processes in perceived real time by humans or machines.

NOTE 2:	This definition is based on IEEE SA P1918.1 [7].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

PSDB	PDU-Set Delay Budget

PSER	PDU-Set Error Rate

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Architectural Assumptions and Requirements

4.1	Architectural Assumptions

-	The architecture, framework and the QoS model as specified in TS 23.501 [2], TS 23.502 [3], and TS 23.503 [4] are regarded as the baseline for this study.

NOTE 1:	The study focuses on using NR as access technology. Specific enhancements to other access types are not required to be studied in this TR (although not prohibited).

-	The functional split in 5GS between UE, RAN and CN remains unchanged, i.e. packet classification of DL packets is performed in CN, and the packet classification of UL packets is performed in UE.

-	XRM services are assumed to use the IP PDU session types (however other PDU types are not excluded).

-	XRM services shall be able to coexist within a PLMN or SNPN with existing services simultaneously

-	XRM services can be between client-server (i.e. UE - application server) and/or peer-to-peer (i.e. between two UEs routed via the 5G CN).

-	Architecture enhancements should support XRM applications and its traffic characteristics. However, specific media codec mechanisms are not in the scope of this study. Interested readers may check TR 26.926[x] for related mechanisms.

-	The traffic characteristics (e.g. Burst periodicity, PDU Set importance and traffic correlation) can vary for different media applications and application is aware of this.  Whether the application layer indicates to 5GS or 5GS can detect by itself without interaction with applications is solution-specific.

-	XR and media application data may be encrypted by the client and/or server in some cases and unencrypted in other cases.

NOTE 2:	It is assumed that the some header information necessary for the identification of PDUs is not encrypted.

4.2	Architectural Requirements

The following architectural requirements are applicable to this study:

-	Solutions shall build on the 5G System architectural principles as in TS 23.501 [2], including:

-	flexibility and modularity for newly introduced functionalities.

-	existing methods for communicating the QoS profile (i.e. Standardized 5QI, Operator Pre-configured 5QI, Dynamically Signalled QoS Profile).

-	Any enhancements for this study shall not impact Emergency Services and other Priority Services (MPS, Mission Critical, etc) capabilities.

-	The existing specific 5GS services using existing QoS functions shall not require enhancement to co-exist with any QoS enhancement being specified for XRM services.

**** Next Change ****

**** Last Change ****

* End of changes *