SA WG2 Meeting #151E (e-meeting)	S2-2203919

16 - 20 May, 2022, Electronic meeting	(revision of)

Source:	Ericsson, Deutsche Telekom, NEC

Title:	New WID: Enhancements to usage of Target NSSAI

Document for:	Endorsement

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Enhancements to usage of Target NSSAI

Acronym: TEI18_TANSSAI

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 900032 | Enhancement of Network Slicing Phase 2 | Related functionality that is proposed to be enhanced was introduced by this Work Item. |

# 3	Justification

At the moment when NG-RAN received the Target NSSAI (and the associated RFSP), it may occur that the serving NG-RAN node is not able to identify a redirection target cell/frequency that supports the network slices in the Target NSSAI. In this case, the serving NG-RAN node can continue searching for a redirection target cell/frequency and, when available, redirect the UE accordingly. For the NG-RAN node to be able to do that the Target NSSAI and the (and the associated RFSP) needs to be stored in the UE context in the NG-RAN.

However, the UE may be handed over to a different NG-RAN node before redirection occurs. In this case the target NG-RAN node should receive the Target NSSAI (and the associated RFSP) to ensure that the UE is redirected as soon as possible to the cell/frequency where the Target NSSAI is served.

The above use case is illustrated in the figure below.

Figure 1: example use case showing the need of signalling the Target NSSAI during HO procedures

In Figure 1 a typical case of overlapping frequency deployment is shown. Over F1 the operator has deployed cells serving a smart factory plant. The smart factory plant is served by S-NSSAI1.

Over F2, the operator has deployed its macro coverage, served by S-NSSAI2.

Once the UE is in proximity of the smart plant deployment, the UE may issue a NAS request to access the smart factory services over S-NSSAI1. However, S-NSSAI 1 is not allowed/supported over F2, hence the CN will issue a NAS rejection. With that S-NSSAI 1 will be included in the Rejected NSSAI for the RA and signalled to the UE over NAS. The UE will not be able to request services for S-NSSAI1 until it moves to a different RA.

As a consequence of NAS rejection, the AMF may signal to the NG-RAN a Target NSSAI, including S-NSSAI1. The NG-RAN should use the Target NSSAI to redirect the UE to F1, namely to the frequency where the Target NSSAI is supported. However, F1 is not necessarily available at the time when the Target NSSAI is received by the NG-RAN.

Shortly after the UE may be handed over to gNB2b. Cells served by gNB2b having coverage overlap with F1, hence the UE could be successfully redirected to F1 while served by gNB2b.

However, lack of forwarding of the Target NSSAI to gNB2b during handover procedures implies that gNB2b has no knowledge of the need to redirect the UE to F1.

As a consequence, the UE is ”stuck” in F2. This constitute a major problem because the UE is not always able to redirect itself to the frequency where the wanted network slices are available.

# 4	Objective

The objective is to enhance the usage of the Target NSSAI by:

-	Assume that the Target NSSAI is stored in UE context of NG-RAN such that NG-RAN node tries to redirect the UE while the UE resides in the NG-RAN node and allow the AMF to re-send the same Target NSSAI when the NG-RAN node is changed

-	?

Note: This work will require 0.25 TU.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Add description that NG-RAN can store the Target NSSAI and associated RFSP in UE context and retry when target cell becomes available, AMF can re-send the Target NSSAI and associated RFSP when NG-RAN node is changed. | SA#98 (Dec 2022)? | Target completion to be determined at SA2#152 |
| 23.502 | Add description that Target NSSAI can be also in xyz NGAP message. | SA#97 (Sep 2022)? |  |

# 6	Work item Rapporteur(s)

xxxx, Ericsson (@ericsson.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Impacts to NGAP is to be handled by RAN3.

# 9	Supporting Individual Members

Supporting IM name

Deutsche Telekom

Ericsson

NEC