3GPP SA WG2 Meeting #151E				     S2-2203925

Elbonia, May 16-20, 2022	(was S2-22xxxx)

Source:	Ericsson

Title:	KI#3: new solution: enabling graceful slice termination with support of UE policies

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This contribution provides a new solution for KI#3.

# 1.	Discussion

Currently architecture and procedures already support the ability to add and remove network slices. What may be missing is to enable a graceful replacement of network slices.

# 2 Proposal

It is proposed to include a conclusion that this solution is not be considered.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| Solution #1: Additional S-NSSAI associated with the PDU session | X |  |  |  |  |  |
| Solution #2: Slice Re-mapping Capabilities for Network Slice Service Continuity | X |  |  |  |  |  |
| Solution #3: Support of Network Slice Service continuity using SSC mode 3 | X |  |  |  |  |  |
| Solution #4: PDU Session on compatible network slice | X |  |  |  |  |  |
| Solution #5: PDU session handover to a target CN with an alternative S-NSSAI support | X |  |  |  |  |  |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs |  | X |  |  |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs |  | X |  |  |  |  |
| Solution #8: Gracefully network slice termination |  |  | X |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  |  | X |  |  |  |
| Solution #10: Associating a validity timer with a temporary slice |  |  | X |  |  |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available |  |  | X |  | X |  |
| Solution #12: Solution for Centralized Counting for Multiple Service Areas and 5GS-EPS Interworking |  |  |  | X |  |  |
| Solution #13: Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |  |  |
| Solution #14: Maximum Number Distribution in multiple NSACFs |  |  |  | X |  |  |
| Solution #x: Enabling graceful slice termination with support of UE policies |  |  | X |  |  |  |

NEXT CHANGE

## 6.x	Solution #x: Enabling graceful slice termination with support of UE policies

### 6.11.1	Introduction

This solution applies to KI#3.

This solution enables a graceful replacement and removal of network slices.

NOTE:	As temporary network slice instances are not part of the KI#3 the removal of a network slice means the removal of one or more S-NSSAIs.

### 6.11.2	Functional Description

The basic principles of this solution are the following:

1.	The "time" or area where a network slice is to be terminated is known in advance

2.	If a network slice is to be replaced by a new network slice then:

-	First a new network slice is created i.e. NFs and resources etc and then add the S-NSSAI to subscriptions, and then use PCF to update the UE policies (URSP)

-	Update UEs URSP rules such that the UEs use the new S-NSSAI for new access attempts;

-	Update UEs URSP rules Route Selection Validation Criteria with appropriate Time Window and Location Criteria;

-	The UE is assumed to try and use the Time Window and use higher priority rules when the Time Window is no longer valid.

-	Update the UE URSP rules such that PCF can enforce/command the UE to re-apply the URSP rules for also existing usage which, if possible, triggers the UE to move the user data from S-NSSAI to be removed to the new S-NSSAI e.g. using SSC 3 etc.

3.	OAM knows when and which network slice is to be terminated and configures NFs

1.-	UDR/UDM are updated such that Network Slices are added and UDM notifies AMFs about the added Subscribed S-NSSAIs.

2.	NFs of network slice to be replaced updates NF profile such that load is reduced e.g. no new entrants

3.-	PCF updated as per bullet 2

4.-	At a suitable time, UDR/UDM are updated such that the to be removed Subscribed S-NSSAIs  are removed, and UDM notifies AMFs.

5.	After all subscriptions been updated, OAM removes any remaining usage of the S-NSSAI in the 5GC and 5G-AN.

4.	AMFs follows current logic i.e.

-	AMF updates NSSAI information towards the UE

-	AMF enforces removal of S-NSSAIs e.g. initiates release of PDU Sessions within the network.

### 6.11.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The existing procedures are used as is with minimal impacts to the content of the URSP rules.

### 6.11.4	Impacts on services, entities and interfaces

The following impacts have been identified:

-	UE:

-	UE supports the new conditions in the URSP rules such that PCF can enforce/command the UE to re-apply the URSP rules for also existing usage.

NOTE:	The functionality works also without UE support, but there can then be a service interruption when changing network slice e.g. when UDM notifies AMF that a subscribed S-NSSAI been removed the AMF updates the UE and the AMF also de-registers the S-NSSAI

-	PCF:

-	supports the new conditions in the URSP rules such that PCF can enforce/command the UE to re-apply the URSP rules for also existing usage.

END OF CHANGES