3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2203938

Elbonia, May 16 – 20, 2022

Source:	Ericsson

Title:	KI#6, Sol#17: Update to resolve EN

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: This paper update Solution#17 on Performance Improvements for Public Safety

# 1. Discussion

In TR 23.700-47 v0.2.0, Solution#17 proposes a solution which speed up the paging (include group paging) towards CM-IDLE UEs and CM-CONNECTED UEs with RRC_INACTIVE state, as well as allowing CM-CONNECTED UEs with RRC_INACTIVE state to receive multicast MBS session data without being RRC_CONNECTED. It can speed the group call setup time for public safety services.

There was an EN in clause 6.17.3.2:

Editor's note:	Different AMFs may interact with the same RAN nodes for different PDU sessions.  AMF also store information when the shared delivery is established. Thus information about the same multicast session may reside on multiple AMFs. Whether and how they synchronize their information about MBS sessions is FFS.

According to the proposal in clause 6.17.3.2, the additional information stored in the AMF is the MBS Session ID and associated PDU session ID included by the SMF. The AMF only needs to understand the UE join information to speed up the paging and group paging when multicast MBS session is activated. There are no multicast MBS session contexts (information included in the N2 container from MB-SMF) stored in the AMF. And thus, there are no further needs to synchronize such information about MBS sessions.

[Proposal-1] Resolve the EN based on the clarification above.

There was another EN in clause 6.17.3.4:

Editor's note:	Whether a joined UE list in the AMF has any significant advantage compared to sending the UE list in step 3 (Rel-17 procedure) requires further study. The required signalling interactions may remain the same. It is FFS whether the service requests in step 6, which are separate per UE, constitute the most significant bottleneck in this procedure.

The two figures below show examples about some interactions among SMF, AMF and NG-RAN. Assume there are many UEs located in NG-RAN1, NG-RAN2, NG-RAN3 joined a multicast MBS session, and they are served by SMF1, SMF2, SMF3 via AMF1. Figure 1 shows the example based on the current architecture (Rel-17), while Figure 2 shows the example based on the solution with AMF involvement.

In the current solution (Figure 1), when the multicast MBS Session is activated, the MB-SMF notifies SMF1, SMF2 and SMF3 about the session activation. The three SMFs will send EnableGroupReachability to AMF1 with their own UE list containing the UEs without PDU session user plane and AMF1 will identify the CM-IDLE UEs in each UE list and send group paging request towards each NG-RAN node 3 times.

In the solution with AMF involvement (Figure 2), when the multicast MBS Session is activated, the MB-SMF notifies SMF1, SMF2 and SMF3 about the session activation. The three SMFs will send EnableGroupReachability towards AMF1 with their own UE list containing the UEs without PDU session user plane). However, the AMF1 check the CM-IDLE UEs based on the joined UE list in the AMF and sends the group paging request towards each NG-RAN once, when it receives the first EnableGroupReachability Request or multicast activation request from the MB-SMF (depends on which one reaches the AMF firstly). For the later arrived EnableGroupReachability request (or multicast activation request), they will be handled by the AMF1, but no longer triggers the group paging towards the NG-RAN.

Based on the two examples, after involving the AMF, the interactions between SMF and AMF are the same. However, number of group paging request towards NG-RANs can be minimized, so that it can improve the efficiency of group paging.

[Proposal-2] Update the description and resolve part of the EN (i.e., “Whether a joined UE list in the AMF has any significant advantage compared to sending the UE list in step 3 (Rel-17 procedure) requires further study. The required signalling interactions may remain the same.”) based on the clarification above.

# 2. Proposal

It is proposed to capture the following changes in TR 23.700-47.

# * * * * First change * * * *

#### 6.17.3.2	UE join multicast MBS session

Figure 6.17.3.2-1: UE join multicast MBS session

The following additions apply compared to clause 7.2.1.3 of TS 23.247 [4]:

5.	In Nsmf_PDUSession_UpdateSMContext response, the SMF includes UE join information including the associated PDU Session ID and MBS Session ID outside the N2 SM information. The AMF stores the information and maintains the joined UE list of the MBS session with its associated PDU Session ID.

NOTE:	Different AMFs may interact with the same RAN nodes for different PDU sessions. It is assumed that each such AMF only stores MBS session IDs and UE join information for the UEs it serves. Different AMFs also store information about the MBS session (MBS session ID and involver RAN nodes) when the shared delivery is established. Thus different information about the same multicast session may reside on multiple AMFs. It is assumed that no synchronisation is performed.

Editor's note:	It is FFS on how Target AMF collect UE join information in N2 based handover. It is also FFS if the Rel-17 AMF is involved.

# * * * * Second change * * * *

#### 6.17.3.5	MBS Session Activation

Figure 6.17.3.4-1: MBS Session Activation

The following additions apply compared to clause 7.2.5.2 of TS 23.247 [4]:

Paging:

-	At step 5 (enable group reachability request is received) or after step 11 (MBS session activation request is received), AMF performs group paging and individual paging based on joined UE list in the AMF, when receiving an enable group reachability request from an SMF or MBS session activation request from the MB-SMF (if the AMF is used for shared delivery establishment), if the paging for the multicast MBS session hasn't been performed. The paging will be requested from each involved AMF once per MBS session activation procedure. A RAN node can receive paging request from multiple AMFs. A possible enable group reachability request or MBS session activation request received afterwards at the AMF from a different SMF will not trigger the paging again.

In step 11, to utilize MBS session activation request to trigger paging in the AMF, the MB-SMF needs to provide the MBS activation information to the AMF outside the N2 container.

Editor's note:	It is up to RAN WG to determine whether RAN paging can be performed upon receiving group paging request in step 5, if the MBS session activation request has not been received. Steps 11 to 14 are required in any case to reach all RAN nodes handling inactive UEs

Editor's note:	Whether a joined UE list in the AMF enables a significant overall signalling reduction and speed up compared to sending the UE list in step 3 (Rel-17 procedure) requires further study. It is FFS whether the service requests in step 6, which are separate per UE, constitute the most significant bottleneck in this procedure.

NG-RAN provide a method to allow UEs to receive multicast MBS session data without being RRC-CONNECTED:

-	After step 12, NG-RAN needs to provide a method for those CM-CONNECTED UEs with RRC_INACTIVE state to enable them to receive multicast MBS session data without being RRC-CONNECTED.

Editor's note:	It is up to RAN WG to determine whether and how the method can provided (e.g. NG-RAN updates SIBx/MCCH) when the multicast MBS session is activated.

UE receives multicast MBS Session data without being RRC-CONNECTED

-	The CM-CONNECTED UE with RRC_INACTIVE state is notified by CN-Paging in step 5 (if share the same Paging Occasion) or RAN-Paging in step 12. It utilizes the method provided by NG-RAN or utilizes the locally stored RRC configuration if it has such information and receives multicast MBS session data, in parallel with sending RRC RESUME to NG-RAN.

# * * * * Third change * * * *

### 6.17.4	Impacts on services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] are reused exception for the following additions:

SMF:

-	Provide UE join information to the AMF when UE join the multicast MBS session.

-	Provide UE leave information to the AMF when UE leave the multicast MBS session or when multicast session leave requested by the network or MBS session release.

AMF:

-	Receive UE join/leave information from the SMF and maintain the joined UE list and the group paging information of the multicast MBS session with the associated PDU session ID.

-	Trigger group paging and individual paging, when multicast MBS session activation request or enable group reachability request is received, if the paging for the MBS session activation hasn't been performed.

MB-SMF:

-	Provide MBS activation information to the AMF when sending multicast MBS activation request.

NG-RAN:

-	NG-RAN provides a method to configure those CM-CONNECTED UEs with RRC_INACTIVE state to enable UEs to receive multicast MBS session data without being RRC-CONNECTED.

Editor's note:	It is up to RAN WG to determine how the method can provided (e.g. NG-RAN updates SIBx/MCCH) when the multicast MBS session is activated.

Editor's note:	It is up to RAN WG to determine whether RAN paging should be performed upon receiving group paging request, if the MBS session activation request has not been received.

UE:

-	For CM-CONNECTED UEs with RRC_INACTIVE state, when they are notified upon MBS session activation, they should utilize the method provided by NG-RAN or utilize the locally stored RRC configuration to receive multicast MBS session data, in parallel with sending RRC RESUME to NG-RAN.

Editor's note:	Other additional impacts are FFS.

# * * * * End change * * * *