SA WG2 Meeting #151E	S2-2203972

16 - 20 May, 2022, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	solution proposal for key issue 1 (inactive reception)

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: this paper proposes a solution for key issue 1 (inactive reception).

# 1		Discussion

Key issue 1 is defined as follows:

## 5.1	Key Issue #1: Multicast MBS data reception in RRC Inactive state

### 5.1.1	Description

In order to provide MBS service to more UEs in a cell, NG RAN could enable UEs within an MBS multicast session to receive MBS session data while in CM-CONNECTED with RRC Inactive state.

The following aspect will be studied for multicast:

-	Whether, how and what MBS assistance information to provide from 5GC to RAN for an MBS session allowing UEs in CM-CONNECTED with RRC Inactive state to receive MBS content, including the aspect which 5GC NF(s) determine the MBS assistance information and how they do so;

-	Whether and how to enhance the current procedures (including mobility related procedures) for MBS session with member UEs in RRC Inactive state.

NOTE 1:	During the study of this key issue, coordination with RAN WGs is needed before final conclusion.

NOTE 2:	RAN WG will determine how the switching for the UEs belonging to MBS session from CM-CONNECTED state to CM-CONNECTED with RRC Inactive state (and vice versa) is performed by the RAN node.

This contribution is based on S2-2203972 which has been objected by one company at SA2#150 as too RAN centric. However, this contribution has system wide impacts and the related possibilities will need to be considered in the evaluation phase:

- This contribution contains proposal for the activation of MBS sessions with inactive UEs. This topic is also touched by solutions 4 and 17

- As inactive UEs can move freely between RAN nodes within the RAN paging area, additional RAN nodes may need to be made aware of the MBS session and may need to be distribute MBS data. This can be accomplished with more and less core network impact, and different possibilities need to be compared.

# 2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change ***

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 MBS session reception in RRC Inactive | 2 MOCN network sharing | 3 On demand multicast MBS session | 4 Group Message Delivery | 5 Coexistence with existing power saving mechanisms for capability-limited devices | X Improvement on performance issues for public safety UEs |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 | X |  |  |  |  | X |
| 4 | X |  |  |  |  |  |
| 5 | X |  |  |  |  |  |
| 6 | X |  |  |  |  |  |
| 7 |  | X |  |  |  |  |
| 8 |  | X |  |  |  |  |
| 9 |  | X |  |  |  |  |
| 10 |  |  | X |  |  |  |
| 11 |  |  | X |  |  |  |
| 12 |  |  |  | X |  |  |
| 13 |  |  |  | X |  |  |
| 14 |  |  |  |  | X |  |
| 15 |  |  |  |  | X |  |
| 16 |  |  |  |  |  | X |
| 17 |  |  |  |  |  | X |
| x | x |  |  |  |  |  |

*** 2nd Change (all new)***

## 6.X	Solution #X: Procedures for Transmission mode tor inactive data reception

### 6.X.1	Introduction

This solution addresses Key Issue #1.

### 6.X.2	Functional description

This solution builds on top of solution 1.

RAN nodes not serving any connected UEs in the MBS sessions thus need to be aware of the MBS session. This solution describes one option to accomplish that with CN impacts:

The MB-SMF informs all RAN nodes in a service area of an MBS session via the AMF about the MBS session

Editor's Note: Other options may exist to inform RAN nodes that are not serving any connected UEs within the RAN notification area, but are in scope of RAN WGs.

As service areas are optional for MBS sessions, all RAN nodes in a PLMN are possible affected by this option.

RRC_inactive UEs can move to cells where the transmission mode for RRC connected state is applied and then need to transition to the RRC connected state to receive MBS data.

The current MBS session activation procedures use paging for the MBS sessionfor UEs in CM-IDLE as defined in TS 23.247.

Editor's note:	Related procedures need to be defined by RAN WGs.

It was discussed in Rel-17 that it would be desirable for location dependent services to enable that the MBS session is active in some areas and inactive in other areas. However, UEs moving in RRC-Inactive state between such areas will not know whether the MBS session is active in their location.

### 6.X.3	Procedures

#### 6.X.3.1	MBS service activation

Figure 6.x.3.1-1: MBS service activation

- RAN 1 MBS session activation request.

- Related RAN procedures.

Editor's note:	The procedures in this clause need be determined by RAN WGs. However, transfer of assistance information and shared delivery impacts SA2

#### 6.X.3.2	Triggering MBS service announcement by MB-SMF (Option B)

Figure 6.x.3.4-1: Triggering MBS service announcement by MB-SMF (Option B)

- MBS multicast session suitable for transmission mode for inactive reception is created is created at MB-SMF.

2.-5.	MB-SMF informs RAN nodes in service area of multicast session via AMFs of multicast session and asks them to announce multicast session

6. RAN decides transmission mode for MBS session

- RAN related procedures

Editor's note:	The steps 6 to 9 need to be decided by RAN WGs.

### 6.1.4	Impacts on services, entities and interfaces.

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] are reused.

In addition to impacts for solution 1:

NG-RAN:

UE:

-	Receive MBS data in indicated transmission mode while MBS data is active.

MB-SMF:

-	inform RAN nodes in service area of multicast session