3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2203985

Elbonia, May 16 – 20, 2022

Source:	Rakuten Mobile, NTT DOCOMO, AT&T, Deutsche Telekom, T-Mobile USA, Telecom Italia, Verizon, Vodafone, Orange, China Mobile

Title:	TR 23.700-80: KI#2 update

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_5G_AIMLsys / Rel-18

Abstract: Update of KI#2 on requirements and security aspects.

# 1. Introduction

Based on discussion in S2-2203768, we propose a NOTE to the KI.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-28.

# * * * * First change * * * *

## 5.2	Key Issue #2: 5GC information exposure to UE

As documented in SA WG1 TS 22.261 [2], the application layer AI/ML model can be downloaded from AF/AS to UE to facilitate local intelligence, i.e. the UE can do inference and make decisions locally, for the use cases such as image recognition, speech recognition, media quality enhancement, etc. The UE may need information from the 5GC to assist its local decision.

This Key Issue is related to WT#1.1b, and aims at studying whether and how to enhance the 5GC to expose information to the UE to facilitate its Application AI/ML operation (e.g. Model Training, Splitting and inference feedback etc.):

-	Whether and for which purpose there is a need for 5GC to expose information or data analytics to the UE, and what are the information or data analytics that can be provided by 5GC to the UE to assist the Application AI/ML operation at the UE side.

-	How the 5GC exposes such information or data analytics to the UE.

NOTE:	SA1 feedbackon exposure of network information to the UE and security analysis by SA3 are expected for this Key Issue.

# * * * * End of changes * * * *