SA WG2 Meeting #151E (e-meeting)	S2- S2-2204002r03

May 16 – 20, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	KI #4, #5, New Sol: Access to localized service by using LADN and N3IWF overlay architecture

Document for:	Approval

Agenda Item:	9.4 Study on enhanced support of Non-Public Networks phase 2

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes a new solution to Key Issue #4 and #5.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-08 on FS_eNPN_Ph2.

* * * * Start of 1st Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| 4 |  | X |  |  |  |  |
| 5 |  | X |  |  |  |  |
| 6 |  | X |  |  |  |  |
| 7 |  |  | X | X | X | X |
| 8 |  |  |  |  |  | X |
| 9 |  |  |  |  |  | X |
| 10 |  |  |  | X |  | X |
| 11 |  |  |  |  | X |  |
| 12 |  |  |  | X |  |  |
| 13 |  |  |  | X | X |  |
| 14 |  |  |  | X |  |  |
| 15 |  |  |  | X | X |  |
| 16 |  | X |  |  |  |  |
| 17 |  |  |  |  |  | X |
| 18 |  |  |  |  | X |  |
| X |  |  |  | X | X |  |

* * * * Start of 2nd Change (All new text) * * * *

6.x	Solution #X: Access to localized service by using LADN and N3IWF overlay architecture

6.X.1	Introduction

This solution addresses Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services and Key Issue #5: Enabling access to localized services via a specific hosting network. The basic principle of this solution is reusing N3IWF overlay architecture and LADN mechanism.

This solution can be applies in case of:

-	home network is a PLMN and hosting network is a SNPN.

-	home network is a SNPN and hosting network is a SNPN.

-	home network is a SNPN and hosting network is a PNI-NPN.

NOTE:	In case of home network is a PLMN and hosting network is a PNI-NPN is covered by Solution #11.

6.X.2	Functional Description

6.X.2.1	Architecture

The following figure 6.X.2-1 and figure 6.X.2-2 depict the proposed architecture, which are reusing 5G System architecture with access to SNPN using credentials from Credentials Holder and 5G System architecture accessing overlay network (home network) via underlay network (hosting network). When a UE registers to the selected hosting network and receives localized services provided by the hosting network, architecture in figure 6.X.2-1 is used. If the UE also need to get services from the home network, the UE may establish PDU session as shown in figure 6.X.2-2. Whether the UE receives a specific service from the hosting network or home network is determined based on URSP rule of the UE. The UE just follows the existing PDU Session Establishment procedure based on URSP rule provided by the home network.

Figure 6.X.2-1: Architecture when a UE uses localized services provided by the hosting network

Figure 6.X.2-2: Architecture when a UE uses services provided by the home network

6.11.2.2	Hosting network discovery and selection

It is assumed that there is service level agreement between the home network and hosting network and based on the agreement the AMF in the home network is configured with Localized Service Information. The Localized Service Information contains following information:

-	Localized Service Name

-	Validity condition

-	Time and Spatial validity

-	Hosting network information

-	Precedence

-	Hosting network ID (SNPN or PLMN ID) and RAT type (e.g. NR, E-UTRA)

-	LADN DNN

-	Allowed CAG information

When a UE is registered in home network, the UE may request Localized Service Information to the AMF by sending NAS message (e.g. Registration). If subscription of the UE allows to use the Localized service, the AMF provides localized service information to the UE. The UE stores the received Localized Service Information until time validity condition is fulfilled. The spatial validity condition in the Localized Service Information can be represented by geographical area. The exact service area information of hosting network is provided to the UE as a part of LADN information during the registration to the hosting network.

NOTE 1:	In case of roaming scenario, the AMF in the serving PLMN provides Localized Service Information to the UE.

NOTE 2:	The AMF can provide Localized Service Information to the UE without UE request if subscription of the UE allows to use the Localized service. For example, the AMF may provide Localized Service Information when a UE enters an area where the Localized service is available.

If the hosting network uses CAG for Localized service (i.e. in case of hosting network is PNI-NPN), the Allowed CAG information can be provided to the UE. If the UE receives Allowed CAG information via Localized Service Information, the UE shall use the Allowed CAG information only when the UE access the hosting network for the Localized service.

6.11.2.3	Registration to hosting network and access to Localized services

Based on received Localized Service Information, the UE selects hosting network and registers to the network. When a UE registers to the hosting network and has the subscription to a specific LADN DNN for local service or a wild card DNN, the AMF provides LADN information (i.e. LADN DNN and LADN service area) to the UE according to the existing LADN mechanism. The UE uses the LADN information to access localized services provided by the hosting network.

Editor's note:	It is FFS whether LADN information needs to be extended so that service area of LADN DNN can be different depending on associated S-NSSAI. This enables the hosting network can provide multiple services over the different service area using a single LADN DNN.

NOTE:	When hosting network operator configures LADN information in the AMF, the operator can provide associated validity time of LADN DNN so that the AMF provides LADN information when the validity time is satisfied.

6.X.3	Procedures

6.X.3.1	Requesting Localized service information

Figure 6.X.3.1-1: Requesting Localized service information

The UE may request Localized Service Information to the AMF by sending NAS message (e.g. Registration request). The UE may indicates specific service the UE want to receive by including Localized service name. The AMF provides Localized Service Information to the UE as described in clause 6.X.2.1.

NOTE:	How the UE gets Localized service name is out of 3GPP scope. The UE can get Localized service name via webpage, by installing application, etc.

6.X.3.2	Connection to hosting network and access to Localized services

Figure 6.X.3.2-1: Registration in the hosting network and access to Localized services

If the UE wants to receive Localized service, the UE requests Localized service information as specified in clause 6.X.3.1 and selects hosting network based on received Localized Service Information. If selected hosting network is available, the UE performs registration with the selected hosting network. The Registration procedure in TS 23.502 [4] is used with following modifications:

-	When the UE performs Registration procedure, Registration type indicates that the UE is accessing the network for localized services. Based on this information the AMF determine to provide LADN information to the UE.

-	The AMF indicates to the UDM that the UE is registering for localized services. The UDM use the indication whether to accept UE registration.

After Registration is completed, the UE establishes PDU Session to the LADN DNN.

If the UE needs to get home network services, the UE may establishes PDU Session and registers to the home network via N3IWF.

6.X.4	Impacts on services, entities, and interfaces

UE:

-	The UE indicates that the UE is accessing the network for localized services during the Registration procedure.

-	The UE requests and receives Localized Service Information from the AMF and selects hosting network based on the received Localized Service Information.

AMF:

-	The AMF indicates to the UDM that the UE is registering for localized services.

-	The AMF provides Localized Service Information to the UE.

-	The PLMN AMF allows a SNPN subscribed UE to access PNI-NPN.

-	The SNPN AMF provides Allowed CAG information used for hosting network to the UE.

UDM:

-	The UDM determines whether to accept registration taking into account the indication that UE is registering for localized services.

* * * * End of Changes * * * *