3GPP TSG-SA WG2#151E	S2-2204054r01

Elbonia, 16 - 20 May 2022	(was S2-22xxxxx)

Source:	CATT

Title:	Scope alignment with NR_SL_relay_enh WI

Document for:	Discussion/Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This contribution proposes some updates to architecture requirements to align with NR_SL_relay_enh WI.

- Introduction

In the approved NR_SL_relay_enh WI (RP-213585), the following objectives are listed:

- Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

  - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

  - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

  - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

  - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

- Study the benefit and potential solutions for multi-path support to enhance reliability and throughput (e.g., by switching among or utilizing the multiple paths simultaneously) in the following scenarios [RAN2, RAN3]:

  - A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2).

For multi-path transmission over Layer-2 UE-to-Network Relay, it is clearly stated that the scenario is limited to the same gNB only. This should be made clear in architecture requirements of TR 23.700-33.

For path switching between two indirect network communication paths, it does not mention path switching between Layer-2 UE-to-Network Relay and Layer-3 UE-to-Network Relay w/ or w/o N3IWF with the understanding that no RAN impact during these procedures. This should also be made clear in architecture requirements of TR 23.700-33.

- Proposal

It is proposed to capture the following changes into TR 23.700-33.

>>>>Start Changes<<<<

4.1	Architecture Requirements

Solutions shall build on the 5G ProSe architecture principles as defined in TS 23.304 [3] and 5G System architectural principles as defined in TS 23.501 [7], including flexibility and modularity for newly introduced functionalities.

In order to satisfy the normative stage-1 general requirements in TS 22.278 [5], TS 22.261 [4] and TS 22.115 [6], the system shall:

-	support single NR PC5 hop UE-to-UE Relay for unicast.

-	enhance UE-to-Network Relay functionality to support:

-	service continuity when switching between two indirect network communication paths for UE-to-Network Relay;

-	service continuity when switching between direct network communication path and indirect network communication path for 5G ProSe Layer-2 UE-to-Network Relay, including inter-gNB indirect-to-direct and inter-gNB direct-to-indirect path switching;

-	multi-path transmission using only one direct network communication path and only one indirect network communication path e.g. for improved reliability or data rates;

-	Emergency Services for Remote UE over UE-to-Network Relay.

-	support path switching between direct NR Uu communication path and direct NR PC5 communication path;

NOTE 1:	The UE-to-Network Relay and UE-to-UE Relay in this study include both Layer-3 and Layer-2 Relays unless explicitly stated.

NOTE 2:	It is not targeted to support session continuity (e.g. IP address preservation) during path switching between direct NR Uu communication path and direct NR PC5 communication path.

NOTE 3:	For multi-path transmission over Layer-2 UE-to-Network Relay, a UE is connected to the same gNB using one direct path and one indirect path via Layer-2 UE-to-Network relay.

NOTE 4:	Path switching between Layer-2 UE-to-Network Relay and Layer-3 UE-to-Network Relay with/without N3IWF do not have RAN impacts.

>>>>End Changes<<<<