SA WG2 Meeting #151e	S2-2204096

# May 16th – 20th, 2022; Elbonia               	    		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI+1 KI2: New Solution: 5GS DetNet Node IP Operation and Management

Document for: 	Approval

Agenda Item: 	9.6

Work Item / Release:	FS_DETNET / Rel-18

Abstract of the contribution: This paper proposes a solution to Key Issues #1 and #2 in TR 23.700-46 related to how 5GS DetNet Node operates wrt. IP routing and forwarding, how its IP addressing and routing information is managed so that it can be exposed to or influenced by the DetNet controller.

# 1	Discussion

This paper proposes a Solution for 5GS DetNet Node operation as an IP Router. It explains how IP address management for the 5GS DetNet Node’s interfaces works and how IP routing/forwarding configuration is managed within the Node. It also explains how TSCTSF, as the controller plane entity of the 5GS DetNet Node, becomes aware of the 5GS Node interface, IP addressing and IP routing information and configuration status, so that it is able to make it available to the DetNet controller, and even influence the IP forwarding/routing configuration based on explicit routes provided by the DetNet controller. This is essential to solve Key Issues #1 and #2 which are about reporting information to and receiving configuration from the DetNet controller, respectively.

The approach taken is to base the solution on Release 16 5GS Bridge and Release 17 generalized TSC procedures. DS-TT and NW-TT are utilized for information learning and configuration by TSCTSF. 5GS DetNet Node and DetNet specific features are isolated as much as possible to the TSCTSF, DS-TT and NW-TT, eliminating the impact to other Core NFs and the UPF and the UE, except for SMF for which a small enhancement is proposed. For this reason, configuring DetNet related information, e.g., in UDM as part of the current Framed Routing feature, or using, e.g., NAS procedures or IPv6 prefix delegation for UE/DS-TT’s 5GS DetNet Node IP address or IP routing information exchange are not proposed.

# 2 Proposal

It is proposed to update TR 23700-46 as follows

*** Start of changes (all new text) ***

## 6.x	Solution #x: 5GS DetNet Node IP Operation, Management and Exposure

### 6.x.1	Introduction

The 5G System supports IETF DetNet deterministic networking by abstracting the whole 5GS as a DetNet Node. The 5GS DetNet Node is comprised of a single UPF and a number of UEs connected to it with IP PDU Sessions. It supports DetNet IP data plane and forwarding sublayer operations, essentially acting as an IP router with specific QoS and management capabilities that are exposed to the DetNet controller.

No UE impact is required even though the UE may support a DS TT for the sake of taking benefit from R17 TSC related Time synch work.

The architecture of the 5GS DetNet Node is shown in Figure -1.

Figure 6.X.1-1. 5GS DetNet Node

NOTE:	Aspects related with NEF as shown in Figure 6.X.1-1 are not addressed in this solution but in another solution Y and do not relate with the current solution.

The externally visible interfaces of the 5GS DetNet Node are located either at the UPF and supported by a NW-TT (2)  or at the UEs and for UEs they may be supported by a DS-TT (1) when R17 TSC features are required. They may be connected to other DetNet Nodes, which may be IP routers or hosts. Each interface has a distinct identity and is configured with IPv4 and/or IPv6 address information.

Editor’s NOTE: It is FFS which functionality of the NW-TT is required for DetNet,

The 5GS DetNet Node is able to forward IP packets between any of its interfaces according to DetNet IP data plane requirements. The TSCTSF maps the configuration between the DetNet controller plane entity and the 5GS DetNet Node, maintaining the overall configuration, capability and state information of the Node. It exposes the 5GS DetNet Node’s interfaces and associated IP address information to the DetNet controller. It receives DetNet flow information including flow identification and QoS requirements from the DetNet controller and sets up the 5G user plane accordingly..

The 5GS DetNet Node may support PTP time synchronization if the 5GS is configured to act as a PTP instance (operating as a boundary clock, peer-to-peer transparent clock or end-to-end transparent clock according to 3GPP Release 17 procedures applicable to IP PDU Sessions). The PTP time synchronization processes are orthogonal to DetNet IP QoS or IP routing (forwarding) procedure but both can be used together with the same TSCTSF, and NW-TT instances. Support of PTP time synchronization is not further detailed in the solution as it relies on R17 features.

The architecture figure above shows a NEF as an optional NF. How the NEF may be used as the API exposure of the 5GS acting as a DetNet router is defined in another solution.  The current solution “5GS DetNet Node IP Operation, Management and Exposure” is independent of whether the NEF may be used as the API exposure of the 5GS acting as a DetNet router.

The granularity of the 5GS DetNet Router is per UPF for each network instance or DNN/S-NSSAI

6.x.2	Functional Description

5GS acts as a DetNet Node supporting IP Data Plane and forwarding sub-layer functionality. To support IP Data Plane, it acts as a Layer 3 IP Router. When integrated with IETF DetNet network, 5GS acts as one or more DetNet Nodes of the DetNet network (more than one DetNet node when there are more than one UPF). The 5GS DetNet Node is composed of the interfaces on a single UPF (i.e., PSA) side, the user plane tunnel between the UE and UPF (including 5G RAN and possibly intermediate UPF(s)), the interfaces on the NW-TT side and possibly the DS-TT (s) when R17 TSC time synchronization is needed. For each 5GS DetNet Node of a DetNet network, the interfaces on NW-TT support the connectivity to the DetNet network, and the ports on UE side are associated to the PDU Session providing connectivity to the DetNet network.

The 5GS DetNet Node operation and management follow the procedures of 5GS Bridge and Support of integration with TSN as described in Clause 5.28 of 23.501 with the following differences and clarifications:

- The logical networking entity is called 5GS DetNet Node and it has a unique Node ID.

- 5GS DetNet Node supports IP packet forwarding as an IP router and uses IP PDU Sessions.

- TSCTSF takes the role of the 5GS DetNet Node control and management entity and implements the control and management interface to the DetNet controller.

- Port numbers are used to identify IP interfaces on UE and NW-TT.

- The association between the UE/ IP address, 5GS DetNet Node ID and port number on DS-TT is maintained at TSCTSF and further used to assist to bind the DetNet flow IP traffic with the UE's IP PDU session.

NOTE: 	DS-TT may be needed in future releases to support specific QoS mechanisms such as Time Aware Shaper at the 5GS DetNet Node egress interfaces. IETF DetNet framework supports different types of QoS mechanisms but how the Controller interacts with them has not yet been specified. DS-TT may also be needed in future releases if specific IP interface, neighbour node or routing information similar from the NW-TT interfaces should be provided from the UE interfaces. This would allow also the UE side interfaces to be connected to DetNet Nodes acting as routers to reach IP prefixes or subnets behind them.

5GS DetNet Node Interface IP addressing and IP Routing information management

The User Plane Node and Port Management Information Container data models are extended with IP address and IP routing information.

The IP address information includes at least the following information for both IPv4 and IPv6:

- IP address of the interface

- IP address/prefix length

This allows the TSCTSF to learn the IP address information of each DetNet Node interface.

NOTE: Additionally, other IP configuration information about the IP interface can be included, such as the underlying Layer 2 interface type, properties and identifiers (e.g., MAC address) or the interface MTU size. The TSCTSF can only read the underlying Layer 2 interface type, properties and identifiers (e.g., MAC address).

The Detnet controller needs to manage or to be aware of the data path taken by IP flows exchanged between Detnet hosts in order to be able to manage the QoS/resource reservation across the whole end-to-end path. For instance, the DetNet controller needs to know that if the 5GS DetNet Node receives an IP packet with a particular destination IP address from one of its UE interfaces, from which egress interface to which next hop DetNet Node it will forward it to. Knowing the next hop address at the NW TT is not enough information for the Detnet controller  to know which IP path a given IP flow will use once it has been forwarded by the 5GS Detnet node .

This version of the solution focuses on notifying the Detnet controller via the TSCTSF.

As there is no DS-TT, the IP knowledge that the SMF can share to the Detnet controller via the TSCTSF is limited to the UE IP address and to Frame Route information read in UDM subscription data; this means that the Detnet controller cannot manage traffic paths involving routers reached behind the UE beyond the knowledge of the frame routes that these routers allow to reach.

The rest of this § focuses on information retrieved at NW-TT side.

For the NW-TT side, The IP routing information includes at least an array of routing table entries with at least the following information for both IPv4 and IPv6:

- IP address/prefix

- address/prefix length

- Next hop IP address

NOTE: Additionally, other information about the routes can be included such as cost or delay, or the source of the routing information (static vs. dynamic).

NOTE: All the above information can be included in Port Management Information Containers for NW-TT it might be more efficient to use User Plane Node Management Information Containers to exchange the information for all NW-TT interfaces in a single container.

This allows the TSCTSF either to learn interface specific IP routing information of each 5GS DetNet Node NW-TT side interface for address/prefixes not directly connected to the interface.

TSCTSF may learn the IP address and IP routing information from NW-TT interfaces, where this information may have been configured during the UPF / NW-TT setup or is even updated by IP routing protocols terminated at the NW-TT.

If IP addressing or IP routing information changes in the NW-TT side (e.g., based on IP routing protocol update in UPF/NW-TT or based on receiving new explicit route from the DetNet controller) the update can be done by existing PMIC notification and updating procedures.

IETF RFC 8344 [X] and RFC 8349 [Y] define YANG data models for IP (address) and IP routing management, respectively. They form a good basis for the PMIC data model extensions.

TSCTSF exposing information to the DetNet controller

After it has learned or configured the IP interface, IP address and IP routing information related to UE and NW-TT interfaces, the TSCTSF combines all the information into a single representation and makes it available for the DetNet controller. Based on this information the DetNet controller learns 5GS DetNet Node interfaces, their IP addresses and IP prefixes, and the 5GS DetNet Node IP routing base. Using this information and similar information collected from the other DetNet IP data plane nodes, the DetNet controller is able to calculate routes for DetNet traffic flows through the network.

The way to exchange PMIC and UMIC between the TSCTSF and the NW-TT (via PCF, SMF, etc…) is as defined in R17 for TSC. The difference is that the content of the PMIC/UMIC may differ as explained above.

### 6.x.3	Procedures

4.

3.

- PDU Session Establishment as defined clause 4.3.2.2.1-1 of 23.502 [X2] is used to establish a PDU Session serving for TSC.

The SMF may determine the need to use dedicated resources (e.g. dedicated UPF(s)) for the 5GS DetNet Router based on local policies associated with the DNN and/or S-NSSAI

During this procedure, the SMF selects a UPF, which supports functions as defined in clause 5.28.1 of TS 23.501 [X1], for the PDU Session.

During this procedure, the SMF receives the allocated port number for UE IP interface and user-plane Node ID from the UPF: The UPF allocates the port number for UE, after receiving N4 Session Establishment Request message. The UPF retrieves also the user-plane Node ID (from the NW-TT) and provides its value to the SMF via N4 Session Level Reporting.

During this procedure the SMF may also receive the UE-DS-TT residence time (if a DS TT is supported)

2.	The SMF sends the information received in step 1 to the TSCTSF via PCF (via Npcf_SMPolicyControl service as defined in 23.502 [X2] clause 4.16.).  The TSCTSF stores the binding relationship between 5GS user-plane Node ID, port number and UE IP address for future configuration. The TSCTSF requests creation (using the Npcf_PolicyAuthorization service as defined in 23.502 clause 4.16.5.1) of a new AF session associated with the UE IP address and may subscribe for TSC events over the newly created AF session.

Using the 5GS user-plane Node ID the TSCTSF subscribes with the NW-TT for receiving user plane node management information changes for the 5GS DetNet Node indicated by the 5GS user-plane Node ID as described in clause 5.28.3.1 of TS 23.501 [X1].

After receiving a User plane node Management Information Container (UMIC) containing the NW-TT port numbers, the TSCTSF subscribes with the NW-TT for receiving NW-TT port management information changes for the NW-TT port indicated by each of the NW-TT port numbers as described in clause 5.28.3.1 of TS 23.501 [X1].

The TSCTSF can use any Npcf session (any PDU Session) to subscribe with the NW-TT for node or port management information notifications. Similarly, the UPF can use any N4 session (any PDU Session) to send node or port management information notifications.

- The TSCTSF learns the port management capabilities and interface information of each NW-TT interface. If NW-TT interfaces have locally available IP address or IP routing information, the TSCTSF learns it from the NW-TT..

4.	The TSCTSF now has the up-to-date IP address and IP routing information of each UE and NW-TT interface. It combines it and makes it available for the DetNet controller.

### 6.x.4	Impacts on services, entities and interfaces

-	TSCTSF:

- Learns IP addressing and IP routing information on NW-TT using extended Port Management Information Container data model.

- Maintains the 5GS DetNet Node interface, IP addressing and IP routing information base.

- Exposes the 5GS DetNet Node interface, IP addressing and IP routing information to the DetNet controller.

-	SMF:

-

-	NW-TT:

- Maintains IP address management and IP routing (forwarding) configuration for the NW-TT related IP interface(s). Provides information to TSCTSF using extended User Plane Node and Port Management Information Container data models.

*** next change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	IETF RFC 8655: "Deterministic Networking Architecture".

[3]	IETF RFC 8939: "Deterministic Networking (DetNet) Data Plane: IP".

[4]	IETF RFC 9016: "Flow and Service Information Model for Deterministic Networking (DetNet)".

[5]	draft-ietf-detnet-yang: "Deterministic Networking (DetNet) YANG Model".

[X]	IETF RFC :8344 "A YANG Data Model for IP Management”

[Y]	IETF RFC 8349: "A YANG Data Model for Routing Management (NMDA Version)”

[X1]	3GPP TS 23.501

[X2]	3GPP TS 23.502

*** next change ***