SA WG2 Meeting #151e	S2-2204097

# May 16th – 20th, 2022; Elbonia               	    		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI+1 KI2: New Solution: Architecture to Support of 5GS DetNet Node control via NEF

Document for: 	Approval

Agenda Item: 	9.6

Work Item / Release:	FS_DETNET / Rel-18

Abstract of the contribution: Support of 5GS DetNet Node control via NEF.

# 1	Discussion

Void

# 2 Proposal

It is proposed to update TR 23700-46 as follows

*** Start of changes (all new text) ***

## 6.x	Solution #x: Architecture to Support of 5GS DetNet Node control via NEF

### 6.x.1	Introduction

The 5G System supports IETF DetNet deterministic networking by abstracting the whole 5GS as a DetNet Node. The 5GS DetNet Node is comprised of a single UPF and a number of UEs connected to it with IP PDU Sessions. It supports DetNet IP data plane and forwarding sublayer operations, essentially acting as an IP router with specific QoS and management capabilities that are exposed to the DetNet controller.

The architecture of the 5GS DetNet Node is shown in Figure -1.

Figure -1. 5GS DetNet Node.

The DS TT is an optional component only needed when R17 TSC features are needed. The presence/ absence of the DS TT is transparent to this solution

The externally visible interfaces of the 5GS DetNet Node are located either at the UEs or at the UPF supported by a NW-TT (2). They may be connected to other DetNet Nodes, which may be IP routers or hosts. Each interface has a distinct identity and is configured with IPv4 and/or IPv6 address information.

The 5GS DetNet Node is able to forward IP packets between any of its interfaces according to DetNet IP data plane requirements. TSCTSF acts as the DetNet controller plane entity of the 5GS DetNet Node, maintaining the overall configuration, capability and state information of the Node. It exposes the 5GS DetNet Node’s capabilities, interfaces and associated IP address information and IP routing information to the DetNet controller. It receives DetNet flow information including flow identification and QoS requirements from the DetNet controller and sets up the 5G user plane accordingly

The solution focuses on the Northbound interface of the 5GS DetNet Node.

In case the interface between the Detnet Controler and the TSCTSF is based on plain NETCONF and there is no NEF in-between between the Detnet Controler and the TSCTSF, no tools defined by 5GC and deployed by operators (SCP, HTTP REST based tracing tools….) can be used. Further more, the absence of a NEF prevents from running 3GPP defined API authentication, security and charging collection (logging) defined at NEF.

6.x.2	Functional Description

The solution aims at allowing deployments supporting following set of functionalities on the Northbound interface of the 5GS DetNet Node:

- Support of CAPIF features and API as specified in TS 23.222 like

-	Exposure of 5GC API capabilities

-	Secure Interface between the DetNet controller and the 3GPP network with security profile as defined by SA3

-	charging (possibly for statistics) of events at Northbound (API) interface of the 5GC as defined by SA5

-	Translation of internal-external information

-	usage of an HTTP/REST interface as inside 5GC and as defined on the Northbound of the NEF,

- 	support of 5GC defined SBA interface principles and coding (following 3GPP 29.500 specification) between the NEF and the TSCTSF, allowing the 5GC operator to use SCP or tracing tools on this interface

For this purpose the solution assumes the Optional possibility to deploy a NEF between the DetNet controller and the TSCTSF.

The interface between the DetNet controller and the TSCTSF relies on the Yang model that will be defined by IETF between a DetNet controller and a DetNet Router. Other solutions detail which subset of this model is endorsed by 3GPP. When a NEF is deployed, the same YANG model is carried at the TSCTSF and at the Detnet Controller side of the NEF

It is to be determined whether to use RESTCONF or NETCONF for the YANG model applicable to a DetNet Router on the interface between the DetNet controller and the TSCTSF.

If a RESTCONF interface is used it can naturally be extended support a NEF and/or 3GPP based mechanisms such as SCP

Editor’s note: It is FFS how a Restconf interface is extended to support a NEF and/or 3GPP based mechanisms such as SCP

Editor’s Note: If a NETCONF mapping would be used, then usage of the NEF and/or of SCP between the Detnet controller and the TSCTSF seems not possible FFS.

### 6.x.3	Procedures

There is no impact on the procedures

### 6.x.4	Impacts on services, entities and interfaces

### In case a RESTCONF interface is selected

-	TSCTSF: support Detnet YANG / RESTCONF API

-	NEF: support Detnet YANG / RESTCONF API relaying back and forth between the Detnet controller and the TSCTSF

- 	TSCTSF, NEF: Support extensions to RESTCONF to allow the NEF into the signalling path.

-	Detnet controller: support Detnet YANG / RESTCONF API:

In case a NETCONF interface is selected

-	TSCTSF: support Detnet YANG / NETCONF API