3GPP WG SA WG2 Meeting #151E e-meeting	S2-2204101r03

Elbonia, May 16th – 21st, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI #2, Sol #4: Update to clarify 5GC Analytics exposure to UE

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes update of procedures in Solution #4 for KI#2.

1. Introduction

In the last SA2 meeting, there was a discussion on two issues:

- How the NWDAF can send analytics data to the UE via the serving AMF.

- Impacts to UE.

This pCR is to update the related procedure to clarify the operation of the proposed solution and impacts.

2.	Proposal

It is proposed to adopt the following change in the TR 23.700-80.

* * * * First Change * * * *

6.4	Solution #4: 5GC Analytics exposure to UE

6.4.1	Description

This solution is proposed to address Key Issue #2: 5GC information exposure to UE.

To facilitate its Application AI/ML operation between the UE and the AF, it will be helpful for the UE to perform the AI/ML operation if the NWDAF can expose the related analytics data. For example, the UE and the AF may exchange the intermediate result to complete the E2E AI/ML operation over application layer, which may require large bandwidth or low latency for the AI/ML operation traffic. For such situation, the NWDAF can expose "Slice load level related network data analytics", which can be used by the UE to decide the splitting of AI/ML operation appropriately.

Editor's note:	Whether and how the UE can use 5GC information for AI/ML operations is FFS and needs to be described before solution can be adopted, considering also that the same information will be used by the AI/ML application server as well.

This solution aims at studying how to enhance the 5GC, i.e. NWDAF, to expose 5GC information to the UE to facilitate its Application AI/ML operation (e.g. Model Training, Splitting and inference feedback etc.).

The procedure for the NWDAF to expose analytics data (e.g. UE Mobility) to the UE is as illustrated in Figure 6.4.2-1.

6.4.2	Procedures

Figure 6.4.2-1: Procedure for analytics exposure to UE

1.	The UE negotiates with the AF though application layer on analytics subscription and generates a UE’s subscription correlation ID.

2.	The AF selects the NWDAF (possibly via NEF if AF is untrusted for authentication) either by local configuration or querying the NRF according to Analytics ID, Area of Interest etc.

3.	The AF subscribes to the NWDAF via Nnwdaf_AnalyticsSubscription_Subscribe(UE identifier, Analytics ID, Area of Interest, Notification Target Address (+ callback URI), Target UE IP Address, UE’s subscription correlation ID) for analytics exposure to the UE.

Note: In the case that the AF subscribes to the NWDAF on behalf of UE via SMF, the AF includes the Notification Target Address (+callback URI) of SMF which is to be notified, and also includes Target UE IP address and UE’s subscription correlation ID, then performs the following step 4a-5a.

Editor's note:	There might be potential security issue when the AF subscribes to the NWDAF on behalf of the SMF, which is FFS.

4a.	The NWDAF may return analytics data with subscription correlation ID to the serving SMF according to the Notification Target address from AF, and instructs the notification message is to be exposed to which UE by Target UE IP address.

5a. The SMF forwards the Analytics data together with an indication of subscription correlation ID for exposure to UE via NAS by invoking Namf_Communication_N1N2MessageTransfer to AMF as defined in clause 4.2.3.3 of TS 23.502 [4].

4b.	Alternatively, the NWDAF may return analytics data to the AF by invoking Nnwdaf_AnalyticsSubscription_Notify.

5b.AF sends the analytics data to the UE over application layer.

NOTE:	It will be considered in the evaluation phase that whether 5GC Analytics exposure to UE via the network could be efficient compared to the GBR flow.

6.4.3	Impacts on services, entities and interfaces

AMF/SMF:

-	Supporting analytics exposure from NWDAF to UE.

NWDAF:

-	Supporting analytics exposure to UE via SMF.

UE:

-	Supporting analytics exposure from NWDAF via NAS.