3GPP TSG-WG SA2 Meeting #151E e-meeting 	S2-2204111r04

Elbonia, May 16 – 20, 2022	(revision of S2-220xxxx)

Source:	KDDI, Huawei, Hisilicon

Title:	KI #4, Sol #12: Update to apply identification of PDU Set as a H.265/H.266 video slice

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: This contribution proposes update of solution#12 for KI#4 to apply identification of PDU Set as a H.265/H.266 video slice.

# 1. Introduction

Solution #12 “PDU Set integrated packet handling” was agreed to be added to TR 23.700-60. This solution contains identification of PDU Set as a H.264 video slice.

H.265/HEVC and H.266/VVC are extensions of the concepts for H.264 and offer better data compression at the same level of video quality, or substantially improved video quality at the same bit rate in comparison to H.264. These codecs are also used in wide range of contents, such as XR, 4K/8K video stream and Cloud gaming.

This paper proposes how to identify PDU Set as a H.265/H.266 video slice for the Solution #12.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# *** Start of Changes ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[7]	IEEE SA P1918.1: "Tactile Internet: Application Scenarios, Definitions and Terminology, Architecture, Functions, and Technical Assumptions", https://standards.ieee.org/project/1918_1.html

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[9]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", STD 64, July 2003.

[10]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[11]	draft-ietf-avtext-framemarking: "IETF experimental draft-ietf-avtext-framemarking-13 - Frame Marking RTP Header Extension".

[12]	IETF RFC 6184: "RTP Payload Format for H.264 Video", May 2011.

[13]	IETF RFC 6190: "RTP Payload Format for Scalable Video Coding", May 2011.

[14]	IETF RFC 6437: "IPv6 Flow Label Specification", November 2011.

[15]	IETF RFC 8285: A General Mechanism for RTP Header Extensions.

[16]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[17]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[18]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[19]	ITU-T Recommendation H.264: "Advanced video coding for generic audiovisual services".

[20]	IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control", July 2003.

[21]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC)", March 2016.

[22]	ISO/IEC 14496-12 | 15444-12: "Information technology - Coding of audio-visual objects - Part 12: ISO base media file format".

[23]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[24]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[25]	IETF RFC 1889: "RTP: A Transport Protocol for Real-Time Applications", January 1996.

[26]	3GPP TS 29.998: "Open Services Architecture (OSA) Application Programming Interface (API) part 2".

[W]	ITU-T Recommendation H.265: "High efficiency video coding".

[X]	ITU-T Recommendation H.266: "Versatile video coding.

[Y]	draft-ietf-avtcore-rtp-vvc-14: "RTP Payload Format for Versatile Video Coding (VVC)".

# *** Next Change ***

#### 6.12.3.1	PDU Set integrated packet handling

High level procedure of PDU Set integrated packet handling can be shown as following.

Figure 6.12.3.1-1: High-level Procedure of PDU Set integrated packet handling

As shown in Figure 6.12.3.1-1, a schematic diagram of the modification process of the PDU session for the XR service is given. The process includes the following steps:

0.	The UE establishes a PDU Session as defined in clause 4.3.2.2.1 of TS 23.502 [3]. A network slice type for XR service can be used for such a PDU Session.

1.	Optionally, as defined in clause 4.15.6.6 of TS 23.502 [3], the AF may invoke the Nnef_AFsessionWithQoS_Create request to set up an AF session with required QoS. In the step 1, the AF may send the following information to PCF:

Flow description of the target media service data flows for PDU Set handling.

Burst periodicity.

The PDU Set level packet handling/treatment requirements. The PDU Set level packet handling/treatment requirements are optional and may include e.g. PDU Set Error Rate, PDU Set Delay Budget, Maximum PDU Set Loss Rate/Number, Media protocol info (e.g. which RFC is used in user data) etc.

2.	PCF may initiate the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3]. The PCF generates appropriate PCC rules based on the information from AF as mentioned in step 1, e.g. 5QI, PER and PDB. The PDU Set level packet handling/treatment requirements may be considered during the PCC rule generation. The PCC rules may also include the detection rules of service data flow, PDU Set level packet handling/treatment policy, PDU Set identification rules. The PCF sends the PCC rules to SMF.

3.	SMF generates the QoS profiles and N4 rules based on the PCC rules from PCF, which may include the packet handling/treatment policy. SMF sends the N4 rules to UPF via the N4 rule, which may include the identification and marking rule for PDU Set. Besides, SMF also sends the QoS profiles to the RAN node via AMF, and instructs RAN to perform PDU Set integrated QoS handling.

NOTE: Based on Media protocol info included in the PDU Set level packet handling/treatment requirements or based on pre-configuration, SMF generates N4 rules accordingly to instruct UPF to identify the PDU Sets of corresponding protocols as shown in subclause 6.12.3.2.

4.	Based on received N4 rules or locally configuration on the UPF, the UPF identifies the PDU Set and marks PDU Set info in the GTP-U layer in the DL packets, including start/end indication of the PDU Set and the PDU Set ID. Details of how to identify the PDU Set are in clause 6.12.3.2.

5.	The RAN identifies the PDU Set based on the PDU Set info in GTP-U header and transmits PDUs within the PDU Set in an integrated manner, e.g. The RAN may drop the PDU Set as a whole in case of poor network condition, and execute packet handling/treatment policy.

NOTE: The details of RAN behaviour is up to RAN WG.

# *** Next Change ***

##### 6.12.3.2.2	Identification of PDU Set as a H.264 video slice.

If the PDU Set represents a H.264 video slice, the identification of PDU Set can be realized by following approach.

According to RFC 6184 [12], within the RTP packet, the payload starts from a one-byte header structured for a Network Abstraction Layer (NAL) Unit encapsulating the slice output of encoded stream and the following is the encoded video content for one, part of or multiple video slices.

Figure 6.12.3.2-3: NAL unit header format for H.264

The 5-bit NAL unit type in the NAL unit header can indicate the content of NAL unit, e.g. coded slice of an I frame, coded slice of a P frame. Besides, it can also indicate the possible structures of the RTP payload, e.g. single NAL unit packet, aggregation packet and fragmentation unit (FU). The NAL unit type for different RTP packet structure is shown below.

Figure 6.12.3.2-4: H.264 NAL unit types and corresponding packet types

For single NAL unit packet, that means one RTP packet can carry one NAL unit.

For aggregation packet, it means one RTP packets can carry multiple NAL unit, especially when the sizes of NAL units are quite small.

For fragmentation unit, that means multiple RTP packets are used to carry one NAL unit.

For single NAL unit packet and aggregation packet, it can be easily detected that each RTP packet carries one NAL unit when the NAL unit type is less than 28.

When NAL unit type is 28 or 29, one NAL unit is carried over multiple RTP packets. In this case, the first byte of RTP payload is also named the fragmented unit (FU) indicator and the following byte is the FU header as shown in Figure 6.12.3.2-3. In the FU header, the "S" bit and "E" bit separately represents the start and end of the NAL unit. Therefore, based on the NAL unit type (also known as FU indicator for fragmented unit) and the FU header, the start/end of the same NAL unit can be identified.

Therefore, with the RTP payload (i.e. NAL unit header and optionally FU header), the UPF can identify the PDU Set for one video slice.

NOTE 1:	The mechanism described in this subclause does not work with SRTP-based payload encryption.

NOTE 2:	Option 1 and 2 in clause 6.12.3.2.1 and methods in clause 6.12.3.2.2 rely on the usage of corresponding RTP protocols.

# *** Next Change (all new texts)***

##### 6.12.3.2.x	Identification of PDU Set as a H.265 video slice.

If the PDU Set represents a H.265 video slice, the identification of PDU Set can be realized by following approach.

According to RFC 7798 [21], within the RTP packet, the payload starts from 2 byte header structured for a NAL Unit encapsulating the slice output of encoded stream and the following is the encoded video content for one, part of or multiple video slices.

Figure 6.12.3.2-x: NAL unit header format for H.265

The 6-bit Type in the NAL unit header can indicate the content of NAL unit. Besides, it can also indicate the possible structures of the RTP payload, e.g. single NAL unit packet, aggregation packet (APs), fragmentation unit (FUs) and Payload Content Information (PACI) carrying RTP packet. The NAL unit type for different RTP packet structure is shown below.

Table 6.12.3.2-x: H.265 NAL unit types and corresponding packet types [W]

| NAL unit type | Packet Type Name |
| --- | --- |
| 0-40 | Single NAL unit packet |
| 41-47 | Reserved |
| 48 | Aggregation Packets |
| 49 | Fragmentation Units |
| 50 | PACI packet |
| 51-63 | Unspecified |

- For single NAL unit packet, that means one RTP packet can carry one NAL unit.

- For aggregation packet, it means one RTP packets can carry multiple NAL unit, especially when the sizes of NAL units are quite small.

- For fragmentation unit, that means multiple RTP packets are used to carry one NAL unit.

- For PACI packet, that contains a payload header (that differs from other payload headers for efficiency), a Payload Header Extension Structure (PHES), and a PACI payload.

For single NAL unit packet and aggregation packet, it can be easily detected that each RTP packet carries one NAL unit when the NAL unit type is less than 49.

When NAL unit type is 49, one NAL unit is carried over multiple RTP packets. In this case, the first two-byte of RTP payload is also named the payload header (denoted as NAL U header) and the following byte is the FU header as shown in Figure 6.12.3.2-x. In the FU header, the “S” bit and “E” bit separately represents the start and end of the NAL unit. Therefore, based on the NAL unit type (also known as FU indicator for fragmented unit) and the FU header, the start/end of the same NAL unit can be identified.

When NAL unit type is 50, this is a PACI packet which may carry a single NAL unit packet or FU. In this case, the first two-byte of RTP payload is also named as the PACI header (denoted as NAL U header). During the following two bytes, the “A” bit is the copy of “F” bit and cType field is the copy of Type field in the PACI payload NAL unit. Then the following is the PHES field, whose length is determined by the PHSize. Finally, the following is the PACI payload NAL unit, during which the first byte is FU header when cType is 49. Therefore, based on the PACI header and PACI payload NAL unit, the start/end of the same NAL unit can be identified.

NOTE 1: The mechanism described in this subclause does not work with SRTP-based payload encryption.

NOTE 2: Option 1 and 2 in clause 6.12.3.2.1 and methods in clause 6.12.3.2.x rely on the usage of corresponding RTP protocols.

# *** Next Change (all new texts)***

##### 6.12.3.2.y	Identification of PDU Set as a H.266 video slice.

If the PDU Set represents a H.266 video slice, the identification of PDU Set can be realized by following approach.

According to draft-ietf-avtcore-rtp-vvc-14 [Y], within the RTP packet, the payload starts from 2 byte header structured for a NAL Unit encapsulating the slice output of encoded stream and the following is the encoded video content for one, part of or multiple video slices.

Figure 6.12.3.2-y: NAL unit header format for H.266

The 5-bit NAL unit type in the NAL unit header can indicate the content of NAL unit. Besides, it can also indicate the possible structures of the RTP payload, e.g. single NAL unit packet, aggregation packet (APs) and fragmentation unit (FUs). The NAL unit type for different RTP packet structure is shown below.

Table 6.12.3.2-y: H.266 NAL unit types and corresponding packet types [X]

| NAL unit type | Packet Type Name |
| --- | --- |
| 0-25 | Single NAL unit packet |
| 26-27 | Reserved |
| 28 | Aggregation Packets |
| 29 | Fragmentation Units |
| 30-31 | Unspecified |

- For single NAL unit packet, that means one RTP packet can carry one NAL unit.

- For aggregation packet, it means one RTP packets can carry multiple NAL unit, especially when the sizes of NAL units are quite small.

- For fragmentation unit, that means multiple RTP packets are used to carry one NAL unit.

For single NAL unit packet and aggregation packet, it can be easily detected that each RTP packet carries one NAL unit when the NAL unit type is less than 29.

When NAL unit type is 29, one NAL unit is carried over multiple RTP packets. In this case, the first two-byte of RTP payload is also named the payload header (denoted as NAL U header) and the following byte is the FU header as shown in Figure 6.12.3.2-y. In the FU header, the “S” bit and “E” bit separately represents the start and end of the NAL unit. Therefore, based on the NAL unit type (also known as FU indicator for fragmented unit) and the FU header, the start/end of the same NAL unit can be identified.

NOTE 1: The mechanism described in this subclause does not work with SRTP-based payload encryption.

NOTE 2: Option 1 and 2 in clause 6.12.3.2.1 and methods in clause 6.12.3.2.y rely on the usage of corresponding RTP protocols.

# *** End of Changes ***