SA WG2 Meeting #151e	S2-2204167r01

# Elbonia, May 16-20, 2022               			            (revision of S2-220xxxx)

Source: 	Apple

Title: 	KI#6, New Sol: On-demand slices

Document for: 	Approval

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes a solution for Key Issue #6 – Improved Network control of the UE behaviour control in TR 23.700-41.

# 1 Discussion

Key Issue#6 was agreed in SA2#150e: Improved Network control of the UE behaviour control:

"This Key Issue will study how to enable network-controlled behaviour and ensure the proper utilization of Slices in the system (e.g. what the network can request to the UE, and how, and what additional policies the network can provide to the UE) taking into account the above aspects (e.g. actual slice usage, UE activity, etc.)."

This solution proposes for the Serving PLMN to be able to configure the UE with an additional indication for a Configured NSSAI as an on-demand S-NSSAI.

# 2 Proposal

It is proposed to update TR 23.700-41 as follows:

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| Solution #1: Additional S-NSSAI associated with the PDU session | X |  |  |  |  |  |
| Solution #2: Slice Re-mapping Capabilities for Network Slice Service Continuity | X |  |  |  |  |  |
| Solution #3: Support of Network Slice Service continuity using SSC mode 3 | X |  |  |  |  |  |
| Solution #4: PDU Session on compatible network slice | X |  |  |  |  |  |
| Solution #5: PDU session handover to a target CN with an alternative S-NSSAI support | X |  |  |  |  |  |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs |  | X |  |  |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs |  | X |  |  |  |  |
| Solution #8: Gracefully network slice termination |  |  | X |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  |  | X |  |  |  |
| Solution #10: Associating a validity timer with a temporary slice |  |  | X |  |  |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available |  |  | X |  | X |  |
| Solution #12: Solution for Centralized Counting for Multiple Service Areas and 5GS-EPS Interworking |  |  |  | X |  |  |
| Solution #13: Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |  |  |
| Solution #14: Maximum Number Distribution in multiple NSACFs |  |  |  | X |  |  |
| Solution #X: On-demand slices |  |  |  |  |  | X |

NEXT CHANGE

## 6.X	Solution #X: On-demand slices

### 6.X.1	Functional Description

When the Serving PLMN configures the UE with Configured NSSAIs, the Serving PLMN may include an additional indication whether each S-NSSAI in the list of Configured NSSAI is an on-demand S-NSSAI. The Serving PLMN is aware whether a slice is resource-constrained and in that case, it configures the corresponding S-NSSAI in the UE as on-demand.

The UE, upon receiving the list of Configured NSSAI for the Serving PLMN, will not attempt to register for any of the Configured NSSAI that are indicated as on-demand S-NSSAI.  Rather, when the UE evaluates URSP rules and an on-demand S-NSSAI appears in the Route Selection Descriptor of a URSP rule, the UE will attempt to register the on-demand S-NSSAI with the AMF. If the S-NSSAI is returned among the Allowed NSSAI in the Registration Accept message, the UE will then further evaluate the Route Selection Descriptor of the URSP rule and establish a PDU session for the on-demand slice accordingly. This results in the UE only registering with a S-NSSAI when it needs to have connectivity to the related network slice.

### 6.X.2	Procedures

The solution relies on existing procedures defined in TS 23.502 [x].

### 6.X.3	Impacts on Existing Nodes and Functionality

UE:

- Registers with Configured NSSAI marked as on-demand NSSAI only when triggered by a corresponding URSP rule.

AMF/NSSF:

- Able to mark Configured NSSAI as on-demand NSSAI to ensure the proper utilization of slices in the system.

END OF CHANGES